# Customizable Paper-Based Virtual Keyboard System Using Computer Vision and Deep Learning

Lahiru Kasun Dilhara

2026-09-04

Table of Contents

# List of Abbreviations

The following alphabetical reference list defines the primary acronyms, domain terminology, neural network architecture notations, and mathematical abbreviations used throughout this thesis:

@p3.0cm p12.0cm@ Abbreviation & Full Definition / Description

1D CNN & One-Dimensional Convolutional Neural Network
1D ResNet & One-Dimensional Residual Neural Network
2D / 3D & Two-Dimensional / Three-Dimensional
API & Application Programming Interface
Attention & Self-Attention / Transformer Attention Mechanism
BCE & Binary Cross-Entropy Loss Function
BiLSTM & Bidirectional Long Short-Term Memory Recurrent Network
CAD & Computer-Aided Design
CLI & Command Line Interface
CNN & Convolutional Neural Network
CPU & Central Processing Unit
CRC & Class Responsibility Collaboration
CSV & Comma-Separated Values
DAW & Digital Audio Workstation
DL & Deep Learning
DLT & Direct Linear Transformation
DPI & Dots Per Inch
DSRM & Design Science Research Methodology
FN & False Negative
FP & False Positive
FPS & Frames Per Second
FR & Functional Requirement
GIL & Global Interpreter Lock (Python Runtime Concurrency)
GPU & Graphics Processing Unit
GUI & Graphical User Interface
HCI & Human-Computer Interaction
HRNet & High-Resolution Network
HW & Hardware
IDE & Integrated Development Environment
IEEE & Institute of Electrical and Electronics Engineers
IR & Infrared Sensing Spectrum
JSON & JavaScript Object Notation
LLM & Large Language Model
LSTM & Long Short-Term Memory Recurrent Neural Network
MAE & Mean Absolute Error
MCP & Metacarpophalangeal Joint (Hand Skeleton Landmark 9)
ML & Machine Learning
MLP & Multi-Layer Perceptron
MSE & Mean Squared Error
MVC & Model-View-Controller Architectural Pattern
NFR & Non-Functional Requirement
NIST & National Institute of Standards and Technology
ONNX & Open Neural Network Exchange
OpenCV & Open Source Computer Vision Library
OS & Operating System
PC & Personal Computer
PDF & Portable Document Format
PIP & Proximal Interphalangeal Joint (Hand Anatomy)
POSIX & Portable Operating System Interface
QoS & Quality of Service
RAM & Random Access Memory
RGB & Red Green Blue (Additive Color Model)
RGB-D & Red Green Blue + Depth Sensor Modality
RMS / RMSE & Root Mean Square / Root Mean Square Error
ROI & Region of Interest
RQ & Research Question
SDK & Software Development Kit
SGD & Stochastic Gradient Descent
SSD & Solid-State Drive
SUS & System Usability Scale
SVD & Singular Value Decomposition
SVM & Support Vector Machine
TAM & Technology Acceptance Model
TN & True Negative
ToF & Time-of-Flight (Active Optical Depth Sensing)
TP & True Positive
UC & Use Case
UGC & University Grants Commission
UI & User Interface
USB & Universal Serial Bus
VR & Virtual Reality
WPM & Words Per Minute (Typing Velocity Performance Metric)
XML & Extensible Markup Language

# 1	Introduction

## 1.1	Chapter Overview

Human-Computer Interaction (HCI) has developed rapidly over the past few decades. Researchers and developers continue to look for natural, simple, and low-cost ways for people to interact with computers. Traditional physical input devices, such as mechanical desktop keyboards, membrane keypads, and laptop touchpads, are still the most common tools for typing text and entering commands. Even though physical keyboards are widely used, their fixed plastic shapes, physical buttons, high manufacturing costs, and inability to change their key arrangements create practical challenges. For example, standard physical keyboards cannot easily adapt to custom user needs, assistive typing for disabled users, or sterile hospital environments [1], [2].

To address these limitations, this research project presents a customizable, paper-based virtual keyboard system. The system works using a single ordinary monocular RGB webcam, AprilTag visual marker tracking, and a lightweight PyTorch deep learning model. The user can design any custom key layout using a desktop software application and print it on a normal sheet of plain paper with standard desktop printers. When the paper is placed on a flat table, a normal webcam observes the surface. Google MediaPipe Hand Landmarker tracks hand joint movements in 2D and 3D space. The joint coordinates are scale-normalized relative to hand size so the system works reliably across different cameras and distances. A lightweight Long Short-Term Memory (LSTM) neural network runs on a standard commodity computer processor (CPU) in real time to detect when a finger presses the paper surface. When a touch is detected, planar homography transforms the camera pixel coordinates into the printed layout coordinate system to trigger the corresponding key press or system command. Crucially, the same physical printed sheet can be reused for multiple different software profiles (such as regular typing, developer shortcuts, sound editing, or terminal commands) without needing to reprint the paper.

This introductory chapter explains the background, motivations, and technical foundations of the research. Section 1.2 presents the problem background and historical development of physical and virtual keyboards. Section 1.3 describes the general and specific problem statements, concluding with the identified research gap. Section 1.4 presents the main research question and sub-questions. Section 1.5 explains the motivation behind this project. Section 1.6 defines the research aim, and Section 1.7 sets out four formal research objectives. Section 1.8 presents a rich picture diagram and step-by-step pipeline description. Section 1.9 lists the hardware and software resources required for the project. Section 1.10 outlines the project scope and boundaries, and Section 1.11 summarizes the chapter.

## 1.2	Problem Background

Physical computer keyboards have been the primary method for entering text and commands for more than fifty years. While physical keyboards provide clear tactile feedback that helps trained typists type quickly, their fixed physical structure brings several practical drawbacks [1]. A standard physical keyboard is rigid. Its key sizes, shapes, and button positions cannot be modified after manufacturing. As a result, users cannot easily adapt a physical keyboard for non-standard language character sets, single-handed typing, specialized application shortcuts, or assistive setups for individuals with hand tremors or motor disabilities.

To overcome the rigid nature of physical hardware, researchers in computer vision and human-computer interaction have studied projection-based and camera-based virtual keyboards [3], [4], [5], [6]. Early virtual keyboard systems often used optical laser projectors paired with dedicated infrared receivers or Time-of-Flight (ToF) depth cameras [7], [8]. Depth sensors measure the physical distance between the camera and the user’s hand along the Z-axis, which makes it easier to tell when a finger touches a surface. However, depth cameras and laser projection units are expensive, consume extra power, and are not built into ordinary laptops or low-cost computers. Other early systems attempted to detect surface touches using a single standard RGB camera by observing fingertip shadow movements [9], [10]. In practice, shadow-based touch detection is very unstable. Changes in room lighting, shadows from moving hands, dynamic lighting angles, and different table colors cause frequent false key presses, making shadow-based typing unreliable for daily use [11].

In recent years, open-source deep learning frameworks such as Google MediaPipe Hand Landmarker have made it possible to track 21 skeletal hand landmarks in real time from regular RGB video streams [12], [13], [14], [15], [16]. Because MediaPipe learns the anatomical structure of the human hand, it tracks joint positions across different skin tones and lighting conditions much better than older skin-color segmentation methods [17]. Even with reliable hand tracking, detecting the exact moment a finger touches a flat sheet of paper from a single standard camera remains a difficult challenge [2], [18]. A standard 2D camera does not provide depth values. Differences in hand sizes, changing distances between the hand and the camera, and camera viewing angles mean that simple position thresholds cannot reliably distinguish between a finger hovering slightly above the paper and an actual surface press [19], [20].

Furthermore, many existing virtual typing research projects use fixed, hard-coded QWERTY layouts without giving users tools to design custom interfaces. If the paper sheet moves slightly or the camera tilts, fixed pixel coordinate mappings fail, causing key clicks to land in the wrong places [21], [22], [23], [24], [25], [26]. Therefore, there is a clear need for an integrated system that combines custom layout design, printable paper with visual fiducial markers, real-time planar homography tracking, scale-normalized hand joint features without temporal delay, and lightweight deep learning touch detection that runs on standard computer processors.

## 1.3	Problem Statement

Existing virtual keyboard approaches face a significant practical trade-off. They either depend on expensive specialized hardware (such as 3D depth cameras, Time-of-Flight sensors, infrared receivers, stereo camera pairs, or laser projection units) or require high-end graphics processing units (GPUs) processing high-resolution video streams. Because of these constraints, ordinary users with standard laptops and low-cost USB webcams cannot easily use a real-time, customizable virtual keyboard on plain paper.

### 1.3.1	General Problem

At a broad level, traditional physical keyboards and dedicated input peripherals are expensive to replace, can be damaged by spilled liquids and dust, and present hygiene concerns in shared public computers or clinical environments [1]. Although vision-based virtual keyboards offer a flexible, low-cost alternative, current solutions often rely on specialized depth sensors or active projection equipment that regular users do not own [7], [27]. Meanwhile, many deep learning vision models require powerful GPU hardware and high-resolution camera feeds to run at acceptable speeds. This reliance on expensive hardware prevents paper-based virtual keyboards from being widely adopted in everyday home, school, and low-resource settings.

### 1.3.2	Specific Problem

Within the field of monocular computer vision and human-computer interaction, building an accurate, real-time, and flexible virtual keyboard that works on plain printed paper using only a single regular RGB webcam on a standard CPU involves six specific technical problems:

- Single-Finger Limitation and Inability to Track Multiple Fingers Independently: Traditional monocular camera keyboards track only a single active finger (such as the index finger or whichever fingertip moves fastest) [4], [6], [9]. In shadow-based systems, when multiple fingers move near the surface together, their shadows merge and occlude each other [10], causing the tracker to fail. Consequently, existing vision systems cannot support natural typing across all five fingers of the hand.

- Detecting Surface Touch Without Depth Hardware or Dwell-Time Delays: A single standard RGB camera does not provide direct depth measurements along the Z-axis. Telling whether a fingertip is touching a sheet of paper or hovering a few millimeters above it is difficult using simple position limits or fragile shadow analysis [9], [10]. Furthermore, to avoid false triggers, many earlier systems forced users to pause their finger over a key for a fixed duration (dwell-time of 500 ms to 1000 ms) [5], [28], causing high latency and breaking natural typing flow.

- High Computational Latency on Commodity Hardware: Modern deep learning vision models often require high-end GPUs to achieve real-time speeds. Running hand tracking and deep sequence models together on a standard commodity CPU frequently causes high processor load, latency delays, and dropped frames if the pipeline is not carefully optimized [6], [29].

- Environmental Brittleness to Lighting, Shadows, and Skin Tones: Shadow-based and color-thresholding methods break down under diffuse room lighting, multiple light bulbs, soft hand shadows, and diverse skin tones or cluttered backgrounds [9], [11].

- Perspective Distortion, Camera Tilt, and Paper Movement: When a paper layout is placed on a table, camera tilt, user distance, and small paper movements change key positions in the camera view [21], [30]. Without continuous planar homography tracking (H), the software maps touches to incorrect keys [22], [24].

- Layout Inflexibility and Lack of Action Multiplexing: Most physical keyboards and research virtual keyboards use fixed QWERTY layouts with hard-coded key actions [1]. They lack software tools that let users design custom layouts and link the same printed paper sheet to different action sets (such as regular typing, gaming controls, audio editing macros, or terminal commands) without reprinting the page.

##### 1.3.2.0.1	Research Gap:

A clear research gap exists in developing a complete monocular vision framework that works on standard plain printed paper using only a single regular RGB camera (without requiring depth sensors, ToF cameras, stereo setups, laser projectors, or wearable markers), detects independent multi-finger touch events without artificial dwell-time delays, runs in real time on standard commodity CPUs without requiring a dedicated GPU, supports custom user-designed layouts and dynamic action profile switching on the same printed sheet, and maintains reliable touch accuracy across regular and low camera resolutions using AprilTag homography tracking [21], [22], unitless hand-length scale normalization [12], and a lightweight PyTorch LSTM touch classifier [31], [32].

## 1.4	Research Question

To address this research gap, the project is guided by the following primary research question:

How can a monocular computer vision framework using strictly a single regular RGB camera and plain printed paper enable accurate, real-time, and flexible virtual keyboard interaction on flat surfaces (allowing users to design custom layouts and link different action combinations to the same physical printed sheet), executing entirely on standard commodity CPUs without requiring specialized hardware or dedicated GPUs, across regular and low camera resolutions?

To systematically investigate this primary research question, the research addresses the following five specific secondary sub-questions:

- Fiducial Marker Selection and Robust Layout Tracking: Which visual fiducial marker system (such as AprilTag, ArUco, ARTag, or STag) provides the highest planar homography accuracy and lowest corner jitter under camera perspective tilt and low video resolutions, and how can the paper layout be reliably tracked even when markers are partially occluded?

- Hand Landmark Subsets and Kinematic Feature Representation: Which combination and subset of the 21 MediaPipe hand joint landmarks (combined with unitless hand-length scale normalization and joint velocities) provide the most discriminative signals to identify finger touch contact without temporal smoothing delays?

- Deep Learning Sequence Architecture Optimization: Which neural network architecture family (such as 1D CNN, Attention / Transformer, ResNet, BiLSTM, or LSTM) achieves the highest touch classification accuracy (F1-score) while maintaining near real-time inference speeds (under 3 ms) on a standard commodity CPU?

- Temporal Windowing and Real-Time CPU Synchronization: What temporal sliding window length, step size, and sampling rate allow the multi-threaded vision pipeline to capture impact deceleration dynamics reliably while running at a 12 FPS rate on consumer CPUs without a GPU?

- Layout Decoupling and Action Multiplexing: How can physical paper layout geometry be separated from digital software semantics so that a single printed paper sheet can be dynamically bound to multiple distinct action profiles (typing, shortcuts, shell commands) without reprinting the page?

## 1.5	Research Motivation

This research is motivated by several practical and technical factors in human-computer interaction and accessible computing:

- Low Cost and Broad Accessibility: By avoiding expensive depth sensors, laser projectors, or special hardware in favor of plain paper, an ordinary webcam, and a standard CPU, this system can be used on low-cost computers in schools, homes, and developing regions [5], [13].

- Custom Layouts and Reusable Paper Sheets: Users can design custom key layouts (such as number pads, gaming grids, or accessibility layouts) and print them on paper. The same physical printed sheet can be switched between different software modes (like standard typing, programming shortcuts, or audio production macros) without printing new sheets.

- Hardware Independence Across Standard CPUs and Budget Cameras: Ensuring that the software runs in real time on standard consumer CPUs without requiring a GPU, while working across regular and low camera resolutions (such as 360p, 480p, and 720p), allows the system to run on older laptops and budget computers.

- Inexpensive and Disposable Paper Interfaces: In industrial workshops, chemistry labs, and classrooms, custom keypads can be printed on paper for very low cost and replaced easily if damaged or soiled.

- Hygienic Input for Medical Environments: In hospital wards and operating rooms, physical keyboards collect bacteria; paper keyboard sheets with printed AprilTag anchors can be laminated for easy sanitization or discarded after single use.

- Real-Time Deep Learning on Standard CPUs: Showing that a compact PyTorch LSTM network can detect subtle finger deceleration patterns during surface impact in real time on a CPU without a GPU provides an effective approach for lightweight vision systems [12], [14], [31], [32].

## 1.6	Research Aim

The primary aim of this research is to design, develop, and evaluate a customizable, paper-based virtual keyboard system. The system allows users to create custom layouts on plain paper and dynamically connect the same printed sheet to multiple distinct action profiles (keystrokes, keyboard shortcuts, and executable shell commands). It operates using only a single regular monocular RGB webcam without specialized hardware (such as depth cameras, Time-of-Flight sensors, or stereo rigs), tracks plain paper layouts with AprilTag fiducial anchors, and runs in real time on standard commodity CPUs without requiring a GPU across regular and low camera resolutions.

## 1.7	Research Objectives

To accomplish the research aim, the work is divided into four research objectives:

### 1.7.1	1.7.1 To Identify...

To identify and review theoretical principles, planar homography algorithms, hand tracking models, and feature normalization methods in existing literature, establishing the baseline requirements for single RGB camera tracking, plain paper homography, decoupled action profile mapping, and real-time CPU execution without specialized hardware.

### 1.7.2	1.7.2 To Analyze...

To analyze hand joint kinematic features, unitless hand-length scale normalization, direct feature propagation, and temporal sliding windows (5 frames with a 2-frame overlap at a standardized 12 FPS rate) to achieve camera resolution independence and real-time CPU performance without needing a GPU.

### 1.7.3	1.7.3 To Design and Develop...

To design, implement, and integrate an end-to-end two-application software suite comprising:

- A PySide6 desktop layout designer (Application 1) for interactive key layout creation, XML file generation, and printable PDF export with embedded AprilTag markers on standard plain paper.

- A runtime setup and command mapper GUI (Application 2, Component A) that allows users to select regular RGB webcam feeds, load layout XML files, map buttons to keystrokes or shell commands, and save or switch different action profiles for the same printed paper layout.

- An AprilTag visual tracking module to compute a real-time 3×3 Planar Homography matrix (H) from single camera video, correcting for perspective tilt and paper shifts across regular and low camera resolutions [21], [22].

- A MediaPipe Hand Landmarker tracking module paired with a hand-length scale normalizer that processes raw keypoints directly without temporal smoothing [12], [17].

- A lightweight PyTorch LSTM touch detection model (under 1 MB file size) trained on 12 FPS 5-frame temporal windows to detect finger touch contact in real time on standard CPUs without a GPU [14], [31], [33].

### 1.7.4	1.7.4 To Evaluate...

To evaluate the complete system in terms of touch classification accuracy (precision, recall, F1-score), homography coordinate mapping precision, multi-profile action dispatch reliability on the same printed layout, CPU real-time latency across different camera resolutions (360p, 480p, 720p, 1080p), typing speed (Words Per Minute), Character Error Rate (CER), and overall usability on plain paper setups.

## 1.8	Rich Picture of the Proposed Solution

The overall operational workflow of the proposed customizable paper-based virtual keyboard system is depicted in Figure 1.1. The system architecture spans two primary phases: Layout Design and Export Phase (offline design) and Real-Time Camera Pipeline and Key Execution Phase (online runtime).

Rich Picture of the Proposed Solution and Operational Pipeline Architecture.

### 1.8.1	Detailed Operational Architecture Steps

- Layout Design and Export: The user creates a custom key layout using the PySide6 desktop designer program. The software outputs two synchronized files: an XML file defining key regions, button values, action commands, and marker anchor locations; and a printable PDF sheet showing the layout grid with AprilTag markers placed along the borders.

- Physical Paper Setup and Capture: The user prints the PDF on standard paper and places it on a flat table. A normal USB webcam or laptop camera captures the live video feed at any standard resolution.

- AprilTag Tracking and Homography Calculation: The AprilTag tracking module detects the four corner fiducial markers on the paper and extracts their 2D pixel coordinates [21], [22], [23], [24]. It computes a 3×3 Planar Homography matrix (H) to map camera pixel coordinates directly into the XML layout coordinate system [25], [34], [35].

- Hand Tracking and Scale Normalization: MediaPipe Hand Landmarker extracts 21 skeletal joint keypoints for detected hands [12], [13], [15], [16], [17]. The coordinates are normalized relative to hand length (Lhand, the distance between the wrist and the middle MCP joint) to make features invariant to hand size, camera distance, and image resolution. Raw scale-normalized features are passed directly to the model without temporal smoothing to keep contact deceleration signals crisp.

- Temporal Windowing: Normalized joint positions and derived velocities are gathered at a 12 FPS processing rate into sliding temporal windows of 5 frames with a 2-frame overlap (a step size of 3 frames) [32], [33].

- LSTM Touch Classification on CPU: The custom PyTorch LSTM network evaluates each 5-frame window on the CPU to classify whether a finger touched the physical surface [14], [31], [36], [37], [38].

- Homography Coordinate Mapping and Key Execution: When a touch event is confirmed, the active fingertip pixel location (Ppixel) is mapped using H into the XML layout frame (PXML=H⋅Ppixel). The coordinate is matched with key boundaries to trigger the mapped keystroke or system shell command.

## 1.9	Resource Requirements

The hardware and software specifications needed to build, run, and test the customizable paper-based virtual keyboard system are listed below.

### 1.9.1	Hardware Requirements

- Monocular Camera: Any ordinary USB webcam, low-end camera, or built-in laptop camera working at regular or low resolutions (such as 360p, 480p, 720p, or 1080p). High frame-rate or high-resolution cameras are not needed.

- Host Computer: A standard PC or laptop with an x86_64 or ARM processor (such as an Intel Core i3/i5, AMD Ryzen, or basic equivalent, with at least 4 GB of RAM). The system runs in near real time on standard consumer CPUs without requiring high-end processors.

- Graphics Card (Optional): Live inference runs entirely on the CPU. A dedicated GPU is optional and not required for daily runtime execution.

- Physical Materials: A standard desktop printer with normal A4 or Letter paper, and a flat desk or table surface.

### 1.9.2	Software Requirements

- Operating System: Linux (Ubuntu 22.04 LTS or 24.04 LTS), macOS, or Windows 11.

- Programming Language and Tools: Python 3.10 or higher, managed using the ‘uv‘ package manager.

- GUI Framework: PySide6 (Qt for Python) for building desktop layout design and setup interfaces.

- Computer Vision Libraries: OpenCV (‘opencv-python‘) and Pupil AprilTag (‘apriltag‘ Python bindings).

- Hand Pose Estimation: Google MediaPipe Hand Landmarker (‘mediapipe‘).

- Deep Learning Framework: PyTorch (‘torch‘, ‘torchvision‘) for LSTM model definition, training, and real-time CPU inference.

- Document Tools: TeX Live full suite (‘pdflatex‘, ‘latexmk‘, ‘biber‘) for thesis compilation.

## 1.10	Project Scope

To keep the project focused and practical, clear project boundaries were defined. Table 1.1 outlines what is included in the project scope and what is kept out of scope.

System Project Scope Boundaries (In-Scope vs. Out-of-Scope)

| In-Scope System Elements | Out-of-Scope System Elements |
| --- | --- |
| Single regular monocular RGB camera input (standard USB webcams, budget sensors, regular/low resolutions: 360p, 480p, 720p, 1080p, at 12 FPS). | Specialized 3D depth sensors (RGB-D), Time-of-Flight (ToF) cameras, infrared emitters, or stereo camera setups. |
| Single-hand active typing and touch tracking (extracting 21 joint landmarks for one active hand, classifying touches across all 5 individual fingers). | Two-handed simultaneous (bimanual) typing or multi-user typing (reserved for future extensions to avoid marker occlusion on A4 paper). |
| Standard plain printed paper layout sheets (A4/Letter) with AprilTag markers printed using regular desktop printers. | Active capacitive sheets, conductive ink traces, wired sensor pads, or specialized physical touch hardware. |
| Standard commodity CPU execution for real-time inference without requiring dedicated GPU acceleration (GPU remains optional). | Mandatory GPU requirements or expensive high-end workstation clusters. |
| PySide6 desktop GUI tool for custom layout design, XML export, and AprilTag PDF printing. | Mobile native app ports (iOS/Android) or web browser extensions. |
| AprilTag fiducial marker tracking and 2D planar 3×3 Homography transformation under perspective tilt. | Non-planar or 3D curved surface tracking (such as cylinders or spheres). |
| Unitless hand-length scale normalization (Lhand) with direct feature propagation for complete resolution invariance. | Full 3D biomechanical joint muscle force or physical pressure distribution modeling. |
| Custom lightweight PyTorch LSTM neural network evaluating 5-frame sliding feature windows for touch classification on CPU. | Large-scale end-to-end language model (LLM) auto-correct context generators. |
| Real-time virtual key press execution and XML coordinate lookup on flat desk surfaces. | Multi-user simultaneous typing on the same printed paper keyboard. |

### 1.10.1	Elaboration of Scope Boundaries

The research explicitly restricts video acquisition to a single regular monocular RGB camera stream and uses standard plain printed paper sheets. This design maintains low cost, portability, and zero marginal hardware cost, eliminating any need for specialized depth hardware [10], [13], [29]. Hand tracking and touch classification are specifically focused on single-handed active interaction (tracking one active hand at a time). Focusing on single-hand interaction minimizes CPU processing overhead, keeping latency low (29.09 ms), while avoiding physical hand occlusion of the four corner AprilTag fiducial anchors printed on standard A4 paper sheets. The software pipeline is engineered around a 12 FPS processing target to ensure near real-time execution on standard commodity consumer CPUs without demanding dedicated GPU hardware at runtime (though GPU acceleration is supported where available) [2], [18], [36]. In addition, through unitless hand-length scale normalization, the system eliminates any dependency on high-resolution or studio-grade camera optics, operating robustly across regular and low resolutions (such as 360p, 480p, and 720p). Planar homography mapping is restricted to flat 2D paper sheets on desks or tables [21], [24]. Multi-finger touch classification is performed across all five digits of the active hand using scale-normalized skeletal keypoint features processed by a lightweight PyTorch LSTM model [12], [14], [31], optimized for standard CPU execution. Other features, such as simultaneous two-handed typing or curved surface unwrapping, are designated as out-of-scope for this phase of the project.

## 1.11	Chapter Summary

This introductory chapter established the problem background, problem statement, research questions, motivation, aim, objectives, solution architecture, resource requirements, and project scope for a customizable paper-based virtual keyboard system.

By replacing costly specialized hardware (such as depth sensors, ToF cameras, laser projectors, and high-end GPUs) with a single regular RGB camera, standard plain printed paper, AprilTag planar homography tracking [21], [22], unitless scale normalization [12], and a lightweight PyTorch LSTM touch classifier running in real time on standard commodity CPUs at 12 FPS [14], [31], the research addresses key accessibility and technical challenges in monocular vision input interfaces. The next chapter, Chapter 2 (Literature Review), provides a comprehensive critical review of existing virtual keyboard frameworks, hand pose estimation algorithms, planar homography techniques, and deep learning gesture classification models in current research.

# 2	Literature Review

## 2.1	Chapter Overview

The steady growth of mobile computing, augmented reality (AR), virtual reality (VR), and everyday embedded computer systems has changed how people expect to interact with computers. For more than a century, text input and computer control have relied almost entirely on mechanical QWERTY keyboards and capacitive touchscreens. Although physical keyboards provide clear tactile clicks and allow trained users to type quickly, their fixed physical form creates several practical problems [1], [2]. Physical keyboards take up permanent desk space, cannot change their key arrangements for specialized software tasks (such as audio mixing, industrial computer-aided design, digital painting, or video editing), can spread germs in hospital wards or shared public kiosks, and can be easily damaged by spilled liquids and dust [1].

To overcome these physical limitations, computer vision researchers have explored virtual input interfaces that turn flat physical surfaces or open air into interactive typing areas using standard camera sensors [3], [4], [5], [19]. Monocular vision-based virtual keyboards offer a promising approach: by watching a flat physical surface (such as a paper sheet or a wooden desk) with a single ordinary RGB webcam, users can interact with virtual keys without needing extra hardware switches or dedicated electronic boards.

However, creating a reliable, fast, and customizable virtual keyboard that works with a single ordinary RGB video feed presents several difficult challenges in computer vision, hand motion analysis, and sequential deep learning. The main technical challenges include:

- Detecting Surface Touch Without Depth Hardware: Standard monocular RGB webcams do not provide direct depth information along the Z-axis. Telling the exact moment a fingertip hits a flat surface (distinguishing an actual key press from a finger hovering slightly above the paper) without expensive Time-of-Flight (ToF) sensors or laser projectors remains a difficult computer vision problem [7], [9], [10], [39].

- Hand Scale Differences and Signal Quality: Hand landmark tracking models produce raw pixel coordinates that change depending on hand size, distance from the camera, and user position [12], [36]. Normalizing spatial features directly from joint movements without using temporal smoothing filters (which blur quick touch impacts and add delay) is necessary for accurate real-time touch detection [40].

- Camera Tilt and Surface Alignment: When a paper layout or the webcam shifts slightly, perspective distortion causes fixed coordinate grids to fail. Real-time planar marker tracking and homography matrix calculations (H∈R3×3) are needed to keep the camera view and the layout aligned [21], [22], [25], [34].

- Custom Layout Design and Action Mapping: Most existing virtual typing systems use fixed QWERTY layouts and lack software tools that let users design custom printable paper templates with visual markers while connecting buttons to custom keystrokes or shell commands [1].

This chapter provides a detailed, critical review of prior research related to vision-based virtual input systems, planar marker tracking, hand joint estimation, feature scale normalization, and deep learning sequence models. Section 2.2 presents a conceptual map that shows how the literature is structured. Section 2.3 gives a domain overview covering the history of text input, typing biomechanics, ergonomics, and domain drivers. Section 2.4 evaluates existing virtual keyboard systems across eight sensing categories. Section 2.5 presents a technical analysis of key algorithms (MediaPipe hand tracking, scale normalization mathematics, AprilTag planar homography, and PyTorch LSTM models), design trade-offs, and processing workflows. Finally, Section 2.6 discusses the limitations of prior systems and explains the research gap addressed by this project.

## 2.2	Conceptual Map of the Literature

To help the reader understand the organization of prior work, Figure 2.1 shows a conceptual taxonomy of the literature reviewed in this chapter.

Conceptual taxonomy and structural organization of the literature review.

As shown in Figure 2.1, the literature review is grouped into four main sections:

- HCI and Input Paradigms (§2.3): Reviews the history of typing devices and explains the motivations for paper-based virtual keyboards.

- Sensing Modalities (§2.4): Compares eight different sensing methods using a detailed summary table (Table 2.1).

- Technological Analysis (§2.5): Examines key technical components, including MediaPipe hand landmark tracking, hand scale normalization, AprilTag planar homography, PyTorch LSTM touch detection, and system workflows.

- Research Synthesis (§2.6): Synthesizes the findings to clearly define the research gap addressed in this project.

## 2.3	Domain Overview

Entering text and executing commands are the primary ways humans communicate with computer systems. For more than a century, the mechanical QWERTY layout has been the standard layout for typing. Although mechanical keyboards provide helpful physical clicks that allow skilled users to type quickly, their rigid hardware design causes several problems in modern computing [1]. Physical keyboards occupy fixed desk space, cannot change their button layouts for specific tasks (such as audio mixing, video editing, CAD design, or assistive typing), and can gather dust and bacteria in shared offices or sterile medical clinics.

### 2.3.1	Historical Evolution of Text Entry and Spatial HCI Hardware

The development of text input interfaces can be divided into four main stages:

- Mechanical and Electronic Keyboards (1870s to 1980s): Starting with the first typewriters and moving to early computer keyboards like the IBM Model M, keys used physical levers and metal switches. The QWERTY layout was originally created to prevent mechanical typewriter bars from getting stuck together, rather than to maximize typing speed or comfort [1].

- Membrane Keyboards and Touchscreens (1990s to 2000s): Home computers popularized rubber dome membrane keyboards, while smartphones and tablets introduced capacitive touchscreens. Touchscreens allowed smooth multi-touch gestures but removed physical key edges and travel feedback [1].

- Laser Projection and Infrared Curtains (2000s to 2010s): To build portable keyboards for early mobile devices, engineers developed small laser projectors that projected red key outlines onto flat tables while scanning for fingers with infrared light curtains [8], [41]. Although portable, these laser projectors drained batteries quickly and were hard to see in bright room lighting.

- Monocular Vision, Hand Tracking, and Deep Learning (2010s to Present): Recent deep learning models allow computers to track hand joints in real time using ordinary RGB webcams [12], [14], [19]. This approach makes it possible to create virtual keyboards on ordinary printed paper without needing active laser projectors or special sensors.

### 2.3.2	Kinematic Typing Dynamics and Biomechanical Surface Impact

Understanding how human fingers move during typing is very important when designing vision-based touch detection models. When a user presses a key on a physical surface, the finger goes through four sequential movement stages [27], [33], [42]:

- Approach Stage (t1 to t2): The finger speeds up as it moves downward from an idle resting height toward the paper. Downward velocity Vy increases quickly.

- Impact Stage (t2 to t3): The fingertip hits the physical paper on the desk. In a very short time window (Δt<40 ms), the downward speed drops to zero (Vy→0), producing a sharp deceleration change (Ay=dVydt≪0).

- Dwell Stage (t3 to t4): The fingertip rests briefly on the surface while the computer registers the key press.

- Recoil Stage (t4 to t5): The finger lifts back up to an idle resting position, moving in the opposite direction (Vy>0).

When typing on a sheet of paper placed on a table, the flat table provides a natural physical stop during Stage 2, bringing the downward motion to a quick halt. In contrast, typing in mid-air lacks this physical resting surface, which causes fingers to overshoot and move inconsistently [43].

### 2.3.3	Ergonomics, Fitts’ Law, and Human Input Performance Metrics

Evaluating user typing performance across different input systems involves several standard metrics:

- Words Per Minute (WPM): A standard speed metric calculated as WPM=T−1S×60×15, where T is the number of typed characters and S is the time taken in seconds.

- Keystrokes Per Character (KSPC): Measures typing accuracy by dividing the total number of keystrokes (including corrections) by the total character count. Physical keyboards usually achieve KSPC≈1.05, whereas mid-air typing systems often have KSPC>1.30 because of unintended gestures.

- Fitts’ Law Model: Predicts the movement time MT needed to reach and press a key of width W located at distance D:

MT=a+blog21+DW=a+b⋅ID

- where ID is the Index of Difficulty in bits. Monocular paper keyboards let users enlarge key widths W in the layout designer, lowering the difficulty value ID for users who need larger buttons.

### 2.3.4	Physical Surface Interaction Paradigms: Desk vs. Paper vs. Floating Air

Comparing different physical interaction surfaces highlights several practical trade-offs:

- Bare Desk Surface: Provides a solid physical stop for finger presses, but lacks visible key outlines unless drawn by hand or projected [4].

- Printed Paper Sheet: Combines a solid physical stop with clear printed key labels and AprilTag tracking markers. Paper sheets do not consume electrical power, have zero display delay, and provide a persistent physical guide for the user [3], [5].

- Floating Mid-Air Space: Does not need any surface, but keeping arms raised in the air quickly causes arm fatigue ("Gorilla Arm Syndrome") and lacks physical touch feedback, leading to higher typing errors [29], [43].

### 2.3.5	Biomechanical Kinematic Modeling of Multi-Finger Tendon Coupling

Along with single-finger movements, human hand anatomy involves natural physical connections between fingers [27], [33]. Tendons in the forearm and hand connect adjacent fingers, particularly between the middle finger (Landmarks 9 to 12) and the ring finger (Landmarks 13 to 16).

When a user presses a key with the index finger, neighboring fingers often move slightly as well. Simple algorithms that track only a single point (such as monitoring just the vertical height of an index finger) can mistakenly register these small sympathetic movements as secondary key clicks [4], [9]. Using deep learning models trained on complete 21-joint hand skeletons helps the software understand overall hand structure, allowing it to tell the difference between an intentional touch and passive movement from neighboring fingers.

### 2.3.6	Domain Drivers for Customizable Paper Virtual Keyboards

Monocular paper virtual keyboards provide practical benefits in several common situations:

- Low-Cost Accessibility: Using standard webcams and printable paper layouts removes the need for costly hardware, making virtual keypads practical for schools, home users, and developing areas [5], [13].

- Custom Macro Keypads: Programmers, sound designers, and graphic artists can design custom key sizes, specialized layouts, and macro pads tailored to specific software applications [1].

- Clean Medical and Laboratory Settings: In operating rooms, intensive care units, and cleanrooms, regular keyboards collect dust and bacteria. Inexpensive paper layouts can be laminated for easy cleaning or recycled after use [1].

## 2.4	Existing Systems, Frameworks, and Sensing Modalities

To understand the current state of virtual keyboards, prior systems are reviewed across eight different sensing approaches.

### 2.4.1	Projection-Based and Infrared Sensor Systems

Projection-based virtual keyboards use small optical projectors combined with infrared (IR) light curtains or infrared cameras [8], [41]. A typical laser keyboard projects a red keyboard pattern onto a flat table and projects an invisible infrared light layer a few millimeters above the surface. When a finger presses down, it breaks the infrared light layer, reflecting light back to an infrared sensor [41].

Kudale and Wanjale [8] built an infrared webcam tracking system using an IR-filtered camera and reflective pens, using 4-point planar homography to map projected keys to surface coordinates. Cheng et al. [41] introduced an interactive projector-camera system that separated hands from the background using geometric calibration, estimating touch by tracking the distance between a fingertip and its shadow.

Although projection systems work on plain tables without paper, they face several hardware limitations:

- Sensitivity to Ambient Light: Bright sunlight or standard office lighting washes out laser patterns and interferes with infrared sensors, causing tracking failures.

- Power Consumption and Cost: Projection units need dedicated power sources and careful optical alignment, making them less practical for battery-powered setups.

- Fixed Layouts: The projected key pattern is fixed in the projector optics, so users cannot easily change key sizes or create custom button layouts.

### 2.4.2	Shadow-Based Monocular RGB Keyboards

To avoid active laser projectors and extra infrared sensors, early monocular RGB virtual keyboard researchers studied shadow analysis to estimate finger touch from regular 2D video feeds [9], [10], [11].

Thomas [9] developed a single-camera virtual keyboard that tracked user fingertips and measured the pixel distance between a fingertip and its shadow. As a finger moves downward toward the desk, the distance between the fingertip and its shadow shrinks to zero at the moment of contact. Posner et al. [10] extended this idea by adding adaptive thresholding to detect touch on paper layouts.

While shadow analysis allows touch detection on standard cameras without depth sensors, practical testing shows several serious weaknesses [11]:

- Need for Strong Directional Lighting: Shadow tracking requires a single strong light source from a specific angle. Under diffuse ceiling lights or multiple fluorescent bulbs, shadows blur, split into multiple faint shapes, or disappear completely.

- Hand Self-Occlusion: The palm and other fingers frequently block light from reaching the table, hiding the shadows of the active typing fingers.

- Background Surface Interference: Dark, patterned, or reflective table surfaces make it hard for computer vision algorithms to separate shadows from the background, resulting in frequent missed or false key presses [11].

### 2.4.3	Hardware-Assisted and Depth Sensor Systems (RGB-D / ToF)

With the release of consumer RGB-D depth cameras (such as Microsoft Kinect, Intel RealSense, and Time-of-Flight sensors), researchers built virtual touch systems that measure 3D coordinates directly [7], [39].

Lee and Kwon [7] introduced a virtual touch sensor using a ToF depth camera. By calibrating a reference depth plane for the table surface, the system detected touch events when fingertip point clouds came within a few millimeters of the surface. Toshpulatov et al. [39] improved depth contact detection by combining 3D depth measurements with 2D optical flow motion vectors, helping filter out hovering fingers and false taps.

Depth cameras solve the shadow issues of regular RGB webcams by providing true 3D measurements. However, depth-based systems have practical drawbacks:

- Higher Equipment Cost: Depth cameras are significantly more expensive than regular webcams and are not built into budget laptops or desktop monitors.

- Close-Range Noise and Minimum Distances: Consumer depth cameras often have a minimum operating distance (around 20 to 40 cm) and produce noisy depth readings near flat table surfaces, making it hard to measure thin fingertips cleanly.

- Higher Processing Load: Processing detailed 3D point clouds in real time requires more computing power and often needs dedicated GPU hardware.

### 2.4.4	3D Mid-Air Typing and AR/VR Floating Keyboards

With the growth of virtual reality and augmented reality headsets (such as Meta Quest and Apple Vision Pro), researchers have explored mid-air typing interfaces where users type on virtual keys floating in space [2], [20], [29], [43].

Enkhbat et al. [29] introduced HandKey, a two-hand air-typing system using a single camera. HandKey tracks 3D finger joints in mid-air and uses Convolutional Neural Networks (CNNs) to recognize key selections when fingertips enter pre-set 3D virtual boxes. Lee et al. [2] designed two-handed virtual keyboards for headsets using deep learning keypoint detection, and Yoo and Lee [20] proposed word-level motion learning to classify continuous mid-air typing trajectories.

While mid-air typing does not require a table or paper surface, usability studies highlight significant operational challenges:

- Arm Fatigue: Holding hands and arms unsupported in mid-air causes quick muscle fatigue ("Gorilla Arm Syndrome") during longer typing sessions [43].

- Lack of Physical Stop: When typing on paper or a desk, the finger stops instantly upon hitting the surface. In mid-air typing, the absence of a surface causes fingers to move past target keys, resulting in higher typing errors and slower typing speeds [19], [43].

### 2.4.5	Paper-Anchored and Monocular RGB Keyboards

Paper-based virtual keyboards represent the specific area addressed by this research: using printed paper layouts with regular RGB webcams to enable typing on flat surfaces [3], [4], [5], [19].

Early work by Zhang et al. [3] introduced the "Visual Panel," which tracked paper sheet corners using monocular vision to calculate a homography matrix H and map fingertip positions to paper coordinates. Srivastava and Tripathi [4] built a paper keyboard using fingertip velocity analysis: the typing finger speeds up moving downward and slows down suddenly upon hitting the paper. Khare [5] used skin-color segmentation and fixed key region masks to detect key presses when fingertip positions paused over key areas.

Recently, Maman and Bermano [19] presented TypeNet, a deep learning framework that allows touch typing on uncalibrated flat surfaces using a single camera. TypeNet uses self-refinement and language model decoding to convert finger joint trajectories into text.

Even with these improvements, existing paper keyboard systems show several recurring weaknesses:

- Unstable Markerless Tracking: Early systems relied on manual corner selection or simple color thresholding [3], [5]. When the paper shifts or lighting changes, coordinate tracking fails.

- Fixed Pixel Thresholds: Speed analysis methods [4] rely on fixed pixel velocity thresholds. Because pixel movements change with camera distance and hand sizes, fixed limits do not work well across different users.

- Rigid Layout Designs: Systems like TypeNet [19] and HandKey [29] use fixed QWERTY layouts and do not provide tools for designing custom printable layouts with synchronized action configurations.

### 2.4.6	Video Keystroke Inference and Attack Side-Channels

An interesting related research area is video-based keystroke inference, where researchers study whether typing can be reconstructed from video recordings [11], [44].

Yue et al. [11] showed that side-view video recordings of smartphone typing could reconstruct typed text. The method used optical flow to detect finger downward slowdown, calculated a homography matrix H mapping phone screen edges to a keyboard model, and used shadow cues to confirm contact. Yang et al. [44] generalized video keystroke recovery to uncalibrated cameras, using self-supervised motion paths to extract typed characters directly from video frames without prior layout calibration.

Although these security studies focused on eavesdropping risks, the underlying computer vision techniques (such as planar homography calculations, motion deceleration tracking, and coordinate mapping) provide valuable ideas for building vision-based virtual keyboards.

### 2.4.7	Specialized Optical, Acoustic, and Sensor-Fusion Modalities

Beyond standard cameras, researchers have explored other specialized sensing techniques, including Visible Light Communication (VLC) shadow sensing [45], fingernail blood volume deformation imaging [46], and synchronized motion capture for musical instruments [27], [42].

Webber et al. [45] used indoor ceiling light sensors and photodiode arrays to recognize finger gestures by measuring light changes caused by hand movement. Sun et al. [46] developed a vision method to estimate fingertip pressing force by analyzing fingernail color changes under Linear Discriminant Analysis (LDA), showing that pressing a surface changes blood distribution under the nail. MacRitchie and McPherson [42] and Pilkov [27] combined multi-camera motion capture with piano key touch sensors, proving that finger joint speeds show clear deceleration changes upon hitting a physical surface.

These studies confirm that hitting a physical surface produces clear motion changes, supporting the idea of detecting paper touch contact directly from scale-normalized hand joint movements.

### 2.4.8	Gaze-Based and Assistive Visual Interfaces

For users with severe motor limitations, eye-gaze tracking virtual keyboards offer an alternative touchless text entry option [28]. Chen [28] developed a gaze-based typing system using standard webcams without expensive eye-tracking hardware. The software tracked facial landmarks and pupil positions using deep neural networks to estimate gaze points on virtual keys, using dwell-time pauses to select letters.

Although gaze-based keyboards offer valuable accessibility options, dwell-time selection can cause accidental clicks when users pause naturally to read ("Midas Touch" problem) and is slower than multi-finger hand typing. Combining hand landmark tracking with paper surface contact provides a faster and more natural experience for users who can move their hands.

### 2.4.9	Robotic Vision Servo Control and Fiducial Calibration Systems

Visual fiducial markers are widely used in robotics for position estimation and camera calibration [15], [30], [34], [35], [47], [48].

Ivacko et al. [15] used wrist-mounted cameras with ArUco markers to compute coordinate transformations for robot arm gesture control. Herdiansyah et al. [34] applied Zhang’s camera calibration method to improve ArUco positioning accuracy, reaching 95% localization precision. Schenck et al. [47] created an image-editing pipeline that removed markers synthetically to train markerless keypoint detectors for robot control. Garrido-Jurado et al. [35] introduced ArUco Nano, a fast marker detector achieving a 6.5 times speed increase using an optimized contour scanning algorithm.

These robotics studies show that fiducial markers provide stable and accurate planar alignment, supporting their use for paper keyboard homography calibration.

### 2.4.10	Comparative Analysis of Machine Learning Models in Virtual Input

Prior research has used various machine learning models to map visual features to typing actions:

- Heuristic Limits and Rule-Based Methods [4], [10]: Require no training time but fail when user typing speed or lighting changes.

- Support Vector Machines (SVM) and Random Forests [45]: Fast to train on handcrafted spatial features, but lack sequence memory over time.

- 1D Convolutional Neural Networks (1D CNN) [38], [49]: Extract local motion patterns quickly across short sliding windows, but do not maintain recurrent hidden states.

- Long Short-Term Memory (LSTM) Networks [14], [31], [36]: Maintain internal memory cell states across sequential time steps, allowing the network to learn approach, impact, and recoil movement patterns.

### 2.4.11	Noise Modeling and Lighting Robustness in Passive Vision Systems

Deploying monocular RGB computer vision systems in unconstrained indoor environments requires handling lighting changes and camera sensor noise:

- Handling Different Light Levels: Indoor light levels range from dim room light (100 lux) to bright office fluorescent lighting (1,000 lux) and strong sunlight (50,000+ lux). Older skin segmentation and shadow detection algorithms fail when lighting conditions change [11].

- Deep Landmark Detector Robustness: Modern deep learning keypoint models (such as MediaPipe Hands [12]) are trained on diverse image datasets with data augmentation (varying contrast, brightness, and synthetic noise). Because of this training, predicted joint positions remain reliable across changing room lighting.

### 2.4.12	Comparative Analysis Matrix of Existing Literature

To summarize the literature, Table 2.1 presents a comparative overview of prior research across sensing methods, hardware requirements, surface tracking techniques, scale normalization, temporal filtering, touch detection models, speed, and layout customizability.

Comparative analysis of existing virtual keyboard frameworks and sensing methodologies.

| Study and Year | Sensing Modality | Hardware Req. | Surface Tracking | Scale Normalization | Temporal Filter | Touch Classifier | Latency / FPS | Layout Customization |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Zhang et al. (2001) [3] | Monocular RGB | Standard Webcam | Quad Paper Bounds | None (Pixel Coords) | Moving Avg. | Spatial Proximity | High (∼15 FPS) | Rigid Paper Quad |
| Sun et al. (2009) [46] | Monocular RGB | High-Res Camera | None (Fixed Finger) | None | None | LDA Nail Coloration | Offline (<5 FPS) | None |
| Pirchheim and Reitmayr (2011) [25] | Monocular RGB | Mobile Camera | Keyframe Homography | Feature Scale | Particle Filter | Spatial Tracking | Real-Time (30 FPS) | None (SLAM Map) |
| Srivastava and Tripathi (2012) [4] | Monocular RGB | Standard Webcam | Manual Quad Contour | None (Raw Pixels) | Static Smoothing | Speed Deceleration | Real-Time (25 FPS) | Manual Paper Drawing |
| Posner et al. (2012) [10] | Monocular RGB | Standard Webcam | Fixed Desk Layout | None | Dynamic Threshold | Fingertip-Shadow Dist. | Real-Time (30 FPS) | Fixed Paper Template |
| Thomas (2013) [9] | Monocular RGB | Standard Webcam | Fixed Area | None | None | Shadow Gradient Vector | Real-Time (30 FPS) | Fixed Layout |
| Yue et al. (2014) [11] | Monocular RGB | Smartphone RGB | Screen Border H | None | Optical Flow | Shadow + k-Means | Offline Analysis | Fixed Smartphone Screen |
| Cheng et al. (2015) [41] | Proj. + Camera | Projector + Camera | Photometric Calib. | None | Background Sub. | 3D Shadow Distance | Real-Time (20 FPS) | Fixed Projected GUI |
| MacRitchie and McPherson (2015) [42] | Optical MoCap | MoCap + Touch | Surface Calibration | 3D Skeleton mm | Moving Average | Velocity Inversion | Real-Time (100 FPS) | Fixed Piano Keys |
| Kudale and Wanjale (2016) [8] | IR Webcam | IR Filtered Camera | 4-Point Homography | None | Thresholding | IR Pen Tap Contact | Real-Time (30 FPS) | Projected Display |
| Simao et al. (2017) [33] | Motion Capture | Leap Motion Sensor | None | Bone Lengths | Sliding Window | Velocity Inversion | Real-Time (60 FPS) | None |
| Ji et al. (2018) [6] | Monocular RGB | Standard Webcam | Skin Color Mask | None | Moving Avg. | k-Means Clustering | Real-Time (25 FPS) | Dynamic Key Clusters |
| Nunez et al. (2018) [37] | 3D Skeleton | Depth Sensor | None | Normalized Skeleton | Sequence Window | CNN + LSTM Network | Real-Time (30 FPS) | None |
| Lai and Yanushkevich (2018) [38] | Multimodal RGB-D | Depth Sensor | None | Skeleton Joint Map | Temporal Buffer | CNN + RNN Fusion | Real-Time (30 FPS) | Gesture Set |
| Khare (2019) [5] | Monocular RGB | Standard Webcam | Static Bounds | None | Color Threshold | Pause Over Bounds | Real-Time (20 FPS) | Fixed QWERTY Paper |
| Lee and Kwon (2019) [7] | RGB-D Depth | ToF Depth Camera | Depth Plane Calib. | 3D Metric mm | Weight Filtering | Z-Depth Threshold | Real-Time (30 FPS) | Fixed Virtual Touch |
| Boletsis and Kongsvik (2019) [43] | VR Controller | VR Headset | Virtual 3D Grid | VR World Scale | None | Striking Velocity | Real-Time (90 FPS) | Fixed VR Layout |
| Enkhbat et al. (2020) [29] | Monocular RGB | Standard Webcam | Air Typing Zone | Hand Size Scaling | None | CNN Spatial Classifier | Real-Time (30 FPS) | Fixed Air QWERTY |
| Cui et al. (2020) [50] | Vision Review | Various Sensors | Various | Various | Various | Deep Learning Survey | N/A | N/A |
| Zhang et al. (2020) [12] | Monocular RGB | Commodity Camera | Palm Detector | Normalized 21 Keypoints | Single-frame Regress | Skeletal Keypoint Reg. | High Speed (>60 FPS) | General Hand Tracking |
| Ikram and Liu (2020) [49] | 3D Skeleton | Leap Motion Sensor | None | Hand Scale | Sliding Window | 1D CNN + LSTM | Real-Time (50 FPS) | Mid-air Gestures |
| Wang et al. (2020) [51] | Monocular RGB | Stereo Pair | Superpixel Homography | Superpixel Scale | Bundle Adjust | Multi-Model RANSAC | Real-Time (25 FPS) | Planar Maps |
| Li et al. (2021) [18] | Monocular RGB | Embedded Camera | Grid Mask | None | CNN Feature Map | Lightweight CNN | Edge Real-Time (30 FPS) | Fixed Key Grid |
| Webber et al. (2021) [45] | Visible Light | VLC Photodiodes | Optical Intensity Mask | None | Intensity Features | SVM / Random Forest | Real-Time (30 FPS) | Light Grid Keys |
| Gammulle et al. (2021) [32] | Multimodal RGB | Single Camera | None | Sequence Normalization | Multi-Modal Fusion | Single-Stage TMMF Net | Real-Time (30 FPS) | Continuous Gestures |
| Zhang et al. (2022) [26] | Head-Mounted RGB | Smart Glasses | Gyro + Homography | None | Motion Compensation | Downward Trajectory | Real-Time (30 FPS) | Drawn Keyboard Bounds |
| Lee et al. (2022) [2] | RGB-D / HMD | AR/VR Headset | HMD Frame | 3D Joint Scaling | Temporal Buffer | 7-Class Deep Network | Real-Time (60 FPS) | Ambidextrous Layouts |
| Doan (2022) [36] | Monocular RGB | Commodity RGB | None | Scale Normalized | Coordinate Smooth | Multi-layer LSTM | Real-Time (30 FPS) | Body Activity |
| Yang et al. (2022) [44] | Smartphone RGB | Commodity Phone | Uncalibrated Video | Self-Supervised Scale | Motion Trajectory | Deep Inference Model | Offline Analysis | Uncalibrated Keyboards |
| Maman and Bermano (2023) [19] | Monocular RGB | Front Camera | Uncalibrated Plane | Hand Skeleton Scale | Self-Refinement | Deep Net + LM Decoding | Real-Time (30 FPS) | Fixed QWERTY Flat |
| Gil-Martin et al. (2023) [14] | Monocular RGB | Commodity Camera | None | MediaPipe Scale | Sequence Buffer | Deep Recurrent Net | Real-Time (60 FPS) | General Gestures |
| Sanchez-Brizuela et al. (2023) [17] | Monocular RGB | Commodity Camera | None | Landmark Distance | Morphological Mask | Landmark Rule Classifier | Ultra High (90 FPS) | Skin Segmentation |
| Ivacko et al. (2024) [15] | Monocular RGB | Wrist Camera | ArUco Calibration | Metric Scaling | Low-Pass Alpha | Rule-Based Kinematics | Edge Real-Time (30 FPS) | Robot Control Pad |
| Herdiansyah et al. (2024) [34] | Monocular RGB | Single RGB Camera | ArUco PnP | Metric Tag Size | Zhang Calibration | Geometric Pose Engine | Real-Time (30 FPS) | Navigation Targets |
| Pilkov (2024) [27] | Multi-Camera RGB | 3 RGB Cameras | None | 3D Skeleton mm | Temporal Alignment | Deep Keypoint Trajectory | Real-Time (30 FPS) | Piano Keyboard |
| Toshpulatov et al. (2024) [39] | Multimodal RGB-D | Depth + RGB | Surface Calib. | 3D Spatial mm | Fusion Filter | Temporal Depth-Motion Net | Real-Time (30 FPS) | VR/AR GUI Objects |
| Chen (2024) [28] | Monocular RGB | Commodity Webcam | Screen Grid | Pupil Vector Scale | Dwell Time Filter | Eye-Gaze Deep Model | Real-Time (30 FPS) | Gaze Keypad |
| Schenck et al. (2024) [47] | Monocular RGB | Single RGB Camera | ArUco Inpainting | Metric Scale | Inpainting Mask | Deep Keypoint Detector | Real-Time (30 FPS) | Robotic Links |
| Andriyanov et al. (2025) [16] | Monocular RGB | Commodity RGB | Bounding Box | Normalized Bounds | Temporal Window | MediaPipe + YOLO-Pose | Real-Time (45 FPS) | Gesture Set |
| Yoo and Lee (2026) [20] | Monocular RGB | Single RGB Camera | None | Trajectory Scaling | Displacement Accum. | Word Motion Network | Real-Time (30 FPS) | Contactless QWERTY |
| Kumar (2026) [40] | Monocular RGB | Commodity Camera | Canvas Plane | Landmark Ratio | Adaptive Low-Pass | Motion Threshold | Real-Time (60 FPS) | Air Canvas Drawing |
| Garrido-Jurado et al. (2026) [35] | Monocular RGB | Commodity Camera | ArUco Nano Tags | Metric Tag Scale | Sub-pixel Sampling | Visited-Aware Contours | Edge High (>100 FPS) | Robotics Targets |

The comparative evaluation in Table 2.1 shows how virtual input technology has progressed over time: moving from early image thresholding [3], [4] and fragile shadow methods [9], [10] toward modern deep learning models using scale-normalized hand joint keypoints [12], [14], [19] and visual marker tracking [21], [35].

## 2.5	Technological Analysis

This section provides a detailed technical analysis covering the three core algorithmic and engineering areas of the thesis: hand pose estimation and scale normalization, planar homography tracking using visual fiducial markers, and deep learning touch classification models.

### 2.5.1	Algorithmic Analysis

#### 2.5.1.1	Hand Pose Estimation and Skeletal Joint Tracking

Tracking 2D and 3D hand joint positions from video streams in real time without physical gloves or colored markers is central to vision-based interaction [12], [50].

In early research, hand tracking relied on skin-color thresholding in color spaces like HSV or YCbCr [5], [6]. However, skin-color segmentation is very sensitive to room lighting changes, cluttered backgrounds, and diverse skin tones. Modern deep learning models, such as OpenPose, YOLO-Pose [16], and Google MediaPipe Hand Landmarker [12], improved hand tracking by predicting skeletal joint positions directly from image pixels.

Google MediaPipe Hands [12] uses a two-stage detection and tracking pipeline designed to run efficiently on standard CPUs:

- Single-Shot Palm Detector: The model first scans the full video frame to find palm bounding boxes. Instead of searching for moving fingers directly (which can block each other or bend in complex ways), the detector finds the palm and wrist. It uses focal loss Lfocal=−αt1−ptγlogpt to find hands accurately across different sizes.

- 21-Landmark Keypoint Network: Once the palm area is cropped, a landmark model predicts 2D coordinates xi,yi and relative depth values zi for 21 anatomical hand joints, as shown in Figure 2.2.

MediaPipe 21 hand landmark anatomical topology.

The 21-keypoint model provides detailed hand tracking, locating all five fingertips (Landmarks 4, 8, 12, 16, 20) as well as the finger joints (MCP, PIP, DIP). Studies by Gil-Martín et al. [14] and Sánchez-Brizuela et al. [17] confirm that MediaPipe Hands runs at high frame rates (over 60 FPS) on standard CPUs while maintaining reliable joint tracking under normal room lighting.

#### 2.5.1.2	Spatial Kinematic Scale Normalization vs. Temporal Smoothing

Raw pixel coordinates xi,yi produced by landmark detectors vary when the hand moves closer to or further from the camera, or when the camera resolution changes [36], [40]. If raw pixel values are sent directly into a neural network, a tap by a large hand near the camera appears as a much larger coordinate movement than the exact same tap performed by a smaller hand located further away.

##### 2.5.1.2.1	Mathematical Proof of Perspective Scale Invariance:

Under a standard pinhole camera projection model, 3D world coordinates X,Y,Z map to 2D image pixel coordinates xpixel,ypixel using camera focal length f:

xpixel=fXZ, ypixel=fYZ

When the distance between the hand and the camera Z changes by a factor k (Z′=kZ), the resulting pixel coordinates scale inversely by k:

x′pixel=xpixelk, y′pixel=ypixelk

In the same way, the measured hand length Lhand on the image plane also scales by the factor k:

L′hand=Lhandk

When landmark coordinates relative to the wrist are divided by Lhand, the distance factor k cancels out:

Pinorm′=Pi′−P0′L′hand=Pi−P0kLhandk=Pi−P0Lhand=Pinorm

Equation ([eq:scale_invariance_proof]) proves that unitless hand-length normalization makes feature values invariant to camera distance Z and individual hand sizes.

Let P0=x0,y0 represent the wrist (Landmark 0) and P9=x9,y9 represent the middle MCP joint (Landmark 9). The hand length scale factor Lhand is defined as:

Lhand=x9−x02+y9−y02

Each landmark coordinate Pi=xi,yi is normalized relative to the wrist origin P0 and divided by Lhand:

Pinorm=xi−x0Lhand,yi−y0Lhand

Numerical velocities Vit between consecutive frames are calculated from these scale-normalized coordinates:

Vit=Pinormt−Pinormt−ΔtΔt

##### 2.5.1.2.2	Direct Raw Kinematic Feature Propagation and Information Value:

In touchless interaction research (such as air-drawing or mid-air gesture recognition), low-pass smoothing filters or moving averages are often applied to remove small hand tremors [40].

However, for physical surface typing, landmark coordinates and calculated velocities must be sent directly to the model without temporal low-pass smoothing. The reasons for using direct feature propagation in touch detection are:

- Resolving the Spatial Ambiguity Bottleneck (Hover vs. Contact): A stationary finger hovering 2 mm above a paper key produces virtually the same x,y coordinates in a 2D camera image as a finger in physical contact. Pure spatial coordinates alone cannot distinguish hover from touch. Touch is fundamentally an impact dynamic event characterized by a rapid deceleration spike (Vy→0) upon desk impact [4], [42].

- Preserving Touch Deceleration Signals: Deliberate keystrokes involve sudden deceleration impacts that occur within a very short time window (under 40 ms). Low-pass smoothing filters smooth out these sharp velocity changes, weakening the key signal that separates a true surface tap from hovering finger motion.

- Biomechanical Joint Tiers for Intent Discrimination: Incorporating multiple joint tiers (terminal fingertips for contact impact, intermediate DIP/PIP knuckles for curvature and curl angles, and MCP/wrist base points for palm reference) allows the deep sequence model to separate intentional individual finger strikes from whole-hand posture shifts.

- Avoiding Latency Delay: Low-pass filters and moving averages introduce time delay across consecutive frames. In real-time typing, this introduces extra lag of several frames (30 to 90 ms), making the system feel less responsive to the user.

By using unitless hand-length scale normalization (Pinorm) with direct feature propagation, the pipeline preserves sharp velocity changes, allowing the recurrent neural network to detect surface contact quickly and accurately.

#### 2.5.1.3	Planar Homography Mathematics and AprilTag Fiducial Tracking

Mapping 2D webcam pixel coordinates xpixel,ypixel to a printed paper layout xXML,yXML requires correcting for camera tilt, perspective distortion, and paper movement [25], [51], [52].

Because both the physical paper sheet and the layout coordinate system are flat 2D planes, their geometric mapping is defined by a 3×3 Planar Homography Matrix H∈R3×3 in projective geometry [25], [52]:

xXMLwyXMLww=HPpixel=h11h12h13h21h22h23h31h32h33xpixelypixel1

Dividing by the scale factor w=h31xpixel+h32ypixel+h33 gives the final XML coordinates:

xXML=h11xpixel+h12ypixel+h13h31xpixel+h32ypixel+h33, yXML=h21xpixel+h22ypixel+h23h31xpixel+h32ypixel+h33

##### 2.5.1.3.1	Direct Linear Transform (DLT) Matrix Formulation:

Given N≥4 corresponding point pairs xk,yk↔x′k,y′k, each point pair produces two linear equations:

−xkh11−ykh12−h13+0+0+0+xkx′kh31+ykx′kh32+x′kh33=00+0+0−xkh21−ykh22−h23+xky′kh31+yky′kh32+y′kh33=0

Combining these equations yields the system A2N×9h=0. Solving this system using Singular Value Decomposition (SVD, where A=UΣVT) gives vector h from the column in V associated with the smallest singular value.

##### 2.5.1.3.2	Comparative Analysis of Visual Fiducial Marker Systems:

To provide reliable reference points for homography calculation, visual fiducial markers are placed at the corners of the printed layout [21], [22], [23], [24], [35], [53], [54]. The field of planar fiducial tracking has evolved across three key generations:

- Foundational Edge-Based Digital Markers (ARTag): Fiala [53] introduced the seminal ARTag architecture, establishing a two-stage pipeline consisting of edge-gradient based candidate quad hypothesis generation and forward-error-correcting digital verification. This eliminated the lighting sensitivity and high false-positive rates of earlier correlation-based templates.

- Sub-Pixel Line Clustering and Hamming Code Systems (AprilTag and ArUco): AprilTag 2/3 [21] and ArUco [35] improved corner localization through sub-pixel gradient line clustering and adaptive thresholding. Comparative studies by Shabalina et al. [23], Kallwies et al. [22], and Kalaitzakis et al. [24] showed that AprilTag achieves a low median corner error of 0.017 pixels even at camera tilt angles up to 75∘, outperforming ArUco under steep viewing angles and low image resolutions.

- Deep Learning Keypoint Marker Detectors (DeepTag): Zhang et al. [54] developed DeepTag, an end-to-end CNN framework that regresses dense internal keypoints (such as all internal grid cell centroids) and decodes digital IDs under severe motion blur and extreme viewing angles.

While deep learning marker frameworks like DeepTag [54] offer high robustness, traditional AprilTag detection [21] runs in under 1.0 ms on commodity CPUs with sub-pixel precision. This leaves computational headroom on standard CPUs for concurrent MediaPipe joint extraction and PyTorch touch classification without requiring a GPU. Therefore, border AprilTag markers were chosen for our real-time paper keyboard system.

#### 2.5.1.4	Deep Sequential Temporal Touch Detection Models

Detecting finger surface touches from continuous normalized landmark sequences requires analyzing motion patterns across multiple frames over time [31], [32], [37].

Simple limit-based methods (such as triggering a key whenever downward velocity Vy>θ) fail because non-touch hand adjustments, hovering movements, and natural finger coupling create velocity variations that trigger false presses [33], [39].

Recurrent Neural Networks (RNNs), especially Long Short-Term Memory (LSTM) networks [31], are well-suited for classifying time-series motion data [14], [36], [37], [49]. The standard LSTM architecture uses internal memory cells and gating equations to learn sequential patterns:

ft=σWfht−1,xt+bf (Forget Gate)it=σWiht−1,xt+bi (Input Gate)Ct=tanhWcht−1,xt+bc (Candidate State)Ct=ft⊙Ct−1+it⊙Ct (Cell State Update)ot=σWoht−1,xt+bo (Output Gate)ht=ot⊙tanhCt (Hidden Output)

##### 2.5.1.4.1	Comparative Deep Neural Architecture Evaluation:

When modeling 1D time-series skeletal hand sequences, researchers evaluate five primary deep learning architectures [32], [36], [37], [38], [49]:

- Uni-directional LSTM Networks: Process sequence X=x1,…,xT forward in time. Uni-directional LSTMs rely only on current and past frames, making them ideal for live real-time streams where future frames are not yet available.

- Bidirectional LSTMs (BiLSTM): Combine a forward hidden state ht with a backward hidden state ht. While BiLSTMs achieve high accuracy on pre-recorded datasets, the backward pass requires waiting for future frames, which introduces unwanted buffering latency in live typing.

- 1D Convolutional Neural Networks (1D CNN): Apply 1D temporal convolution filters across landmark features. 1D CNNs extract local patterns quickly, but do not maintain recurrent memory states over longer motions.

- 1D Residual Networks (1D ResNet): Use skip-connections y=Fx+x to help train deeper temporal convolutional layers while avoiding vanishing gradients.

- Self-Attention and Transformer Networks (Attention): Use multi-head self-attention mechanisms AttentionQ,K,V=softmaxQKTdkV to capture pairwise dependencies across all temporal frames in a sequence.

Our experimental benchmark using run_all.py across 22 model combinations confirmed that a uni-directional PyTorch LSTM model trained on 5-frame scale-normalized sequences provides the best balance of high classification accuracy and low real-time CPU latency.

##### 2.5.1.4.2	Temporal Windowing Architecture:

To evaluate incoming landmark data, input features are grouped into sliding temporal windows [32], [33], [36]. The sliding window configuration balances detection accuracy with processing speed:

- 5-Frame Window with 2-Frame Overlap (Stride of 3 Frames): When running at a standardized 12 FPS video rate, a 5-frame temporal window covers a duration of T=416.6 ms. A 2-frame overlap (step size of 3 frames) means the PyTorch LSTM evaluates touches every 250 ms. Experimental testing shows that 5 frames provide enough temporal context to capture finger descent, surface impact, and finger recoil while keeping processing latency low.

Model benchmarks across 2D architectures (LSTM, BiLSTM, 1D CNN, 1D ResNet, Self-Attention) confirm that a multi-layer PyTorch LSTM model trained on 5-frame scale-normalized sequences achieves strong classification performance for multi-finger touch detection on standard computer processors.

### 2.5.2	Design Analysis

Designing a vision-based virtual keyboard involves balancing practical trade-offs between hardware cost, software complexity, user comfort, and computing requirements.

#### 2.5.2.1	Architectural Sensing Trade-Offs

Table 2.2 summarizes the key engineering trade-offs between monocular RGB vision systems and alternative virtual input methods.

System architecture design trade-offs across sensing modalities.

| Architectural Dimension | Monocular RGB + AprilTag (Proposed) | RGB-D / ToF Depth Sensing | Laser Projection + IR | 3D Mid-Air Typing (VR) |
| --- | --- | --- | --- | --- |
| Hardware Sensor Cost | Low (Commodity Webcam) | High ($150 to $400 Depth Camera) | Medium ($80 to $150 Projector) | High ($400 to $3500 Headset) |
| Ambient Light Sensitivity | Moderate (Robust to indoor lux) | High (Sunlight IR interference) | Extreme (Sunlight washes pattern) | Low (Built-in HMD sensors) |
| Surface Alignment | Dynamic (AprilTag Homography) | Fixed Depth Plane Calibration | Fixed Desktop Projection | None (Mid-air tracking) |
| Tactile Feedback | Passive Surface Tactile Stop | Passive Surface Tactile Stop | Passive Surface Tactile Stop | Complete Absence (Fatigue) |
| Layout Customization | High (Printable XML Layouts) | Moderate (Fixed Spatial GUI) | Low (Hardcoded Laser Pattern) | High (Software VR Grids) |
| Edge Compute Latency | Low (>60 FPS on CPU) | Medium (Point Cloud Compute) | Low (Hardware IR Processing) | High (Heavy HMD Pipeline) |

As shown in Table 2.2, using a single monocular RGB webcam with AprilTag markers gives a good combination of low equipment cost, reliable surface tracking using homography matrix H, physical touch feedback from the desk, and easy layout customization.

#### 2.5.2.2	Layout Customization and Synchronized XML Artifacts

A key limitation found in earlier research [1], [19] is that layout design tools are rarely integrated with runtime typing engines. Most prior virtual keyboards hardcode fixed QWERTY key coordinates directly into the source code.

To make the system flexible, the architecture separates layout creation into a dedicated Layout Designer application (designer/designer_app.py). This tool lets users draw custom button shapes, assign keys to specific keystrokes or shell commands, and automatically place AprilTag fiducial markers at layout corners. When the user finishes a design, the tool exports two synchronized files:

- Printable Layout PDF: A PDF file showing the key boundaries and embedded AprilTag visual markers, ready for printing on standard paper.

- Layout XML Specification: A structured XML file containing the coordinates of every key xmin,ymin,xmax,ymax, key identifiers, labels, command actions, and marker anchor locations.

During live operation, the runtime software reads this XML file to set up key lookup boundaries and homography target spaces automatically, without needing to recompile the program.

#### 2.5.2.3	Edge Computing, Parameter Optimization, and Inference Real-Time Benchmarks

Running deep learning models on standard desktop CPUs or budget laptops requires keeping model size small and computational latency low [15], [55], [56].

Jiang et al. [55] showed that replacing heavy convolutional layers with lightweight residual blocks reduces model parameter counts by 65% while keeping real-time speeds on embedded processors. Tariq and Javed [56] tested YOLO detection models on embedded hardware, showing that compact models run efficiently on integrated CPUs when detecting small objects.

Table 2.3 shows the per-frame processing time for each stage of our runtime pipeline running on a single standard CPU thread.

Runtime execution latency breakdown per frame on standard CPU hardware.

| Pipeline Component | Underlying Execution Module | Execution Time (ms) |
| --- | --- | --- |
| Video Frame Capture | OpenCV VideoCapture Thread (camera_thread.py) | 8.33 ms |
| Hand Joint Pose Regression | Google MediaPipe Hand Landmarker | 12.50 ms |
| Unitless Scale Normalization | Distance Processor (normalize_landmarks.py) | 0.21 ms |
| Sliding Window Buffer | 5-Frame Temporal Buffer (create_windows.py) | 0.15 ms |
| Deep Touch Classification | PyTorch LSTM Model (best_finger_touch_lstm.pth) | 3.12 ms |
| Fiducial Tag Localization | AprilTag Tracker Engine | 4.65 ms |
| Homography Transformation | Matrix Mapping Engine (H⋅Ppixel) | 0.08 ms |
| Key Lookup and Execution | XML Action Dispatcher | 0.05 ms |
| Total Pipeline Latency | End-to-End Latency per Frame | 29.09 ms (≈34.4 FPS) |

As shown in Table 2.3, the complete end-to-end processing time per frame is 29.09 ms. This allows the system to run at approximately 34.4 FPS on a standard CPU without requiring an external GPU.

In our system design, evaluating 2D deep temporal models (LSTM, BiLSTM, 1D CNN, 1D ResNet) using run_all.py confirmed that a lightweight PyTorch LSTM model processes 21 2D joint landmarks with very little computing overhead (around 3.12 ms per frame), ensuring smooth real-time execution on standard CPUs.

### 2.5.3	Workflow Analysis

The virtual keyboard system works through two main workflows: the Offline Dataset Creation and Model Training Pipeline, and the Real-Time Runtime Execution Pipeline.

#### 2.5.3.1	Dataset Creation and Model Benchmark Pipeline Workflow

Figure 2.3 shows the step-by-step data processing workflow used to create the scale-normalized training dataset and train the PyTorch LSTM classifier (best_finger_touch_lstm.pth).

Data creation, feature engineering, and model training workflow pipeline.

As shown in Figure 2.3, raw video recordings are first resampled to a standardized 12 FPS rate (resample_12fps.py) to keep CPU load low. MediaPipe extracts 21 hand joint positions, which are normalized using the hand length Lhand (normalize_landmarks.py) so features do not depend on camera distance. Joint velocities are then calculated (calculate_velocities.py), structured into 5-frame sliding windows with a 2-frame overlap (create_windows.py), validated for quality (filter_window_quality.py), and trained across 22 model architectures using run_all.py to produce the final LSTM touch classifier.

#### 2.5.3.2	Real-Time Runtime Watch and Key Execution Pipeline Workflow

Figure 2.4 illustrates the real-time processing workflow inside Application 2 (Runtime Engine).

Real-time runtime vision, touch classification, homography mapping, and key execution workflow.

During live operation (Figure 2.4), the camera thread continuously captures video frames from a single ordinary RGB webcam and sends them in parallel to the AprilTag tracker and MediaPipe Hand Landmarker. AprilTag corner tracking computes the updated homography matrix H for the paper surface. At the same time, MediaPipe joint positions are normalized using the hand length Lhand and placed into a 5-frame sliding window buffer operating at 12 FPS. The lightweight PyTorch LSTM model classifies the window sequence in real time on a standard CPU without needing a GPU. When a touch event is detected, the fingertip pixel coordinates Ppixel are mapped through H into the layout space (PXML=H⋅Ppixel), triggering the corresponding key press or system command.

## 2.6	Reflection and Synthesis of Research Gap

A careful review of prior work shows important achievements along with several remaining limitations in vision-based human-computer interaction.

### 2.6.1	Synthesis of Prior Literature Limitations

The review of existing systems reveals six major limitations and research gaps in previous research:

- Inability to Track Multiple Fingers Independently: Traditional monocular camera keyboards are restricted to tracking only a single active finger (usually the index finger or the single fastest-moving fingertip) [4], [6], [9]. In shadow-based systems, when multiple fingers move near the surface together, their cast shadows merge and occlude one another [10], causing the touch detector to fail. Consequently, prior systems cannot recognize natural touch events across all five individual fingers of the hand.

- Reliance on Artificial Dwell-Time Delays: Because standard 2D webcams cannot measure physical contact directly, earlier vision keyboards forced the user to hover and freeze their finger over a key for a fixed duration (dwell-time of 500 ms to 1000 ms) to register a press [5], [28]. This artificial delay introduces high input latency, causes typing fatigue, and prevents natural, continuous typing.

- High Computing Overhead and GPU Requirements on Commodity Hardware: Existing deep learning vision models that process high-resolution frames or complex 3D grids drop below 15 FPS on standard CPUs [6], [29], while other systems require dedicated GPUs or high-end workstations to achieve real-time responsiveness.

- Environmental Brittleness Under Varying Lighting and Shadows: Shadow-based and color-thresholding methods break down under diffuse room lighting, multiple light bulbs, soft hand shadows, and diverse skin tones or cluttered backgrounds [1], [9], [11].

- Reliance on Expensive or Specialized Hardware: To avoid monocular depth ambiguity, many systems rely on specialized hardware such as Time-of-Flight depth cameras, stereo camera rigs, laser projectors, or wearable sensor gloves [7], [8], [27], [39]. This reliance makes systems expensive and prevents them from running on standard computers with only a regular webcam.

- Lack of Integrated Layout Customization, Marker Tracking, and Action Multiplexing: Existing deep learning typing systems (such as TypeNet [19] and HandKey [29]) use fixed QWERTY layouts and lack design tools to create printable paper sheets with tracking markers. Without continuous homography tracking (H), any slight shift of the paper breaks coordinate accuracy [21], [22], [25]. Furthermore, prior systems cannot dynamically multiplex the same physical printed sheet across multiple software profiles without reprinting.

### 2.6.2	Formal Rationale and Research Gap Justification

These limitations clearly define the central research gap addressed by this thesis:

There is an absence of an integrated, real-time virtual keyboard architecture that operates using strictly a single regular monocular RGB camera on standard plain printed paper layouts without specialized hardware (no depth sensors, no lasers, no stereo pairs), detects independent multi-finger touch events without artificial dwell-time delays, executes in real time entirely on standard commodity CPUs without requiring a dedicated GPU (while remaining GPU-accelerable), supports dynamic layout customization and action multiplexing on the same printed sheet, and maintains robust touch detection and homography mapping across regular and low camera resolutions at a 12 FPS operational standard.

By combining custom paper layout design, AprilTag homography tracking, unitless hand-length scale normalization, direct feature propagation, and a lightweight PyTorch LSTM touch classifier, the system built in this research overcomes the lighting issues, hardware costs, single-finger restrictions, dwell-time latency, high computing demands, and rigid layouts found in earlier systems.

## 2.7	Chapter Summary

This chapter presented a detailed literature review of vision-based virtual keyboards, hand joint tracking, scale normalization, planar marker homography, and deep learning sequence models.

Section 2.2 showed the conceptual taxonomy of the literature. Section 2.3 reviewed the evolution of text entry and the motivations for paper-based virtual keyboards. Section 2.4 compared existing systems across eight sensing categories. Section 2.5 analyzed core technical components including MediaPipe hand landmarks, scale normalization without temporal smoothing, AprilTag planar homography, PyTorch LSTM classifiers, CPU latency measurements, and system workflows. Finally, Section 2.6 summarized prior limitations and clearly stated the research gap.

The ideas and technical foundation discussed in this chapter guide the research methodology, system requirements, and software design presented in the following chapters.

# 3	Research Methodology

## 3.1	Chapter Overview

Designing, implementing, and testing a customizable paper-based virtual keyboard using a single webcam, planar homography, and deep learning requires a clear and organized research methodology. Building a touch interface that uses a single regular camera without special hardware brings together computer vision, hand movement analysis, real-time software development, and user performance testing [1], [19].

To guide this work step by step, this chapter presents the overall research framework used in this project. Section 3.2 explains the philosophical research paradigm and why a quantitative approach is suitable. Section 3.3 details the deductive approach and the Design Science Research Methodology (DSRM) framework. Section 3.4 describes the partitioned engineering strategy used to test individual technical components before putting the complete system together. Section 3.5 explains the data collection setup, 12 FPS video resampling, hand scale normalization, and dataset preparation. Section 3.6 provides an execution workflow table connecting the research objectives to technical activities, software components, and quantitative evaluation metrics. Section 3.7 covers the Agile Scrum management process, project schedule, risk management matrix, and ethical data privacy practices. Finally, Section 3.8 summarizes the chapter.

## 3.2	Research Paradigm

A research paradigm is the basic set of beliefs and philosophical principles that guide scientific investigation [50]. In computer science and software engineering research, paradigms are generally divided into three main perspectives: Positivism, Interpretivism, and Critical Realism [1].

### 3.2.1	Philosophical Categorization and Paradigm Selection

Scientific research is based on three main philosophical ideas:

- Ontology: Looks at the nature of reality (whether reality exists independently in the physical world or is created by human thoughts and social agreements).

- Epistemology: Concerns knowledge and understanding, defining what counts as valid knowledge and how it can be proven.

- Axiology: Relates to values and ethics in research, assessing whether the researcher is an objective observer or influenced by personal perspective.

In computer science and human-computer interaction (HCI), researchers usually consider three main research paradigms:

- Positivism (Empiricism): Assumes an objective physical reality governed by predictable natural and mathematical laws. Knowledge is developed through numerical measurements, controlled laboratory tests, mathematical equations, and statistical analysis. The researcher stays objective.

- Interpretivism (Constructivism): Assumes that reality is formed through human experiences and social context. It mostly relies on qualitative data such as open interviews, group discussions, and user observations.

- Critical Realism: Recognizes that an objective physical world exists, but acknowledges that human observation of this world is shaped by social context and mental models.

For this research project, a Positivist (Empirical) Research Paradigm is selected and applied throughout.

### 3.2.2	Justification of the Positivist Quantitative Paradigm

Choosing a positivist empirical paradigm fits the engineering nature of this research project:

- Objective Measurement of Physical and Motion Data: Hand joint movements, camera coordinate transformations, planar homography matrix math (H∈R3×3), and finger deceleration upon touching paper are physical and mathematical events. Evaluating these components requires exact pixel coordinates, scale ratios, and millisecond latency measurements [12], [22].

- Quantitative Evaluation Metrics: Testing deep learning touch detection models (run_all.py) across 22 model variations relies on numerical performance metrics: Accuracy, Precision, Recall, F1-Score, ROC curves, and inference time on standard CPUs [31], [36].

- Repeatable Controlled Experiments: A positivist approach supports controlled, repeatable testing across different camera tilt angles, hand sizes, typing speeds, and room light levels to evaluate system reliability [14], [39].

### 3.2.3	Epistemological Threats to Validity and Mitigation Strategies

To keep the research reliable and scientifically valid, several possible threats were identified and addressed:

- Construct Validity: Addressed by using clear mathematical metrics (F1-score, sub-pixel corner location error, end-to-end frame processing time in milliseconds) instead of subjective user ratings.

- Internal Validity: Addressed by testing components in five separate research partitions, making sure changes in model accuracy came directly from feature design and model architectures rather than camera noise.

- External Validity: Supported by gathering data from 15 participants with different hand sizes (Lhand between 14.2 cm and 21.5 cm) and testing the system across various indoor light levels (150 lux to over 2,500 lux).

- Reliability: Supported by writing clear and deterministic data preparation scripts (normalize_landmarks.py, create_windows.py) and automated training scripts (run_all.py).

## 3.3	Research Approach

Scientific research generally uses one of three logical approaches: Inductive reasoning (building new theories from observations), Deductive reasoning (testing hypotheses derived from established theories), or Pragmatic mixed methods [50].

### 3.3.1	Deductive Logical Reasoning Framework

This study uses a Deductive Research Approach. The research starts from established theories in computer vision, motion analysis, and deep learning:

- Planar Geometry and Homography Theory [25], [52]: Deduce that layout coordinates PXML can be calculated from webcam pixel coordinates Ppixel using a planar homography matrix H, as long as four clear AprilTag corner points are detected.

- Scale Invariance in Perspective Projection [36]: Deduce that dividing hand joint coordinates by the middle MCP hand length Lhand removes differences caused by hand size and distance from the camera Z.

- Sequence Learning with Recurrent Neural Networks [31]: Deduce that sliding windows of scale-normalized joint velocities capture the sudden slowdown of a finger hitting paper, allowing an LSTM model to detect touch contact without depth cameras.

These theoretical principles were turned into specific software designs and verified through experimental testing.

### 3.3.2	Design Science Research Methodology (DSRM)

To organize the practical software engineering work, this project follows the Design Science Research Methodology (DSRM) described by Peffers et al. [1]. DSRM is widely used in computing research, focusing on creating and testing software artifacts to solve practical problems.

Figure 3.1 shows the six stages of the DSRM framework used in this project.

Design Science Research Methodology (DSRM) execution framework.

The implementation of each of the six DSRM phases in this study is detailed below:

- Phase 1: Problem Identification and Motivation: Identified the research gap in monocular paper keyboards by reviewing 57 prior papers [1], [9], [19]. Most existing systems require expensive depth cameras, active lasers, or powerful GPUs, while single-camera setups struggled with lighting changes, camera perspective tilt, and rigid key layouts.

- Phase 2: Define Objectives of Solution: Defined core system goals: (1) work with a single regular RGB webcam without special hardware, (2) work on plain printed paper, (3) run in real time at a 12 FPS standard on standard CPUs without requiring a GPU (while remaining GPU-accelerable), (4) achieve high touch detection accuracy (F1≥0.95) across regular and low resolutions (360p, 480p, 720p), (5) achieve accurate marker corner tracking (<0.05 pixels), and (6) export synchronized printable PDF layouts and XML configuration files.

- Phase 3: Design and Development: Built two main software applications:

  - Application 1 (Layout Designer): A PySide6 desktop GUI tool (designer/designer_app.py) for drawing custom button layouts, placing AprilTag markers automatically on borders, exporting printable PDF files, and saving XML layout definitions.

  - Application 2 (Runtime Virtual Keyboard Engine): A multi-threaded runtime engine combining AprilTag homography estimation (H∈R3×3), MediaPipe 21 hand joint tracking, scale normalization (Lhand), 12 FPS 5-frame sliding window PyTorch LSTM evaluation (best_finger_touch_lstm.pth) on CPU, coordinate mapping (PXML=H⋅Ppixel), and keypress dispatching.

- Phase 4: Demonstration: Tested the full system on a normal desk using standard paper printed from an office printer, paired with an ordinary webcam, triggering single keystrokes and multi-key commands on a standard desktop CPU.

- Phase 5: Quantitative Evaluation: Carried out thorough benchmarks: comparing 22 deep learning models (run_all.py), measuring CPU latency across resolutions (camera_thread.py), and testing coordinate mapping accuracy (designer/analyzer/main.py).

- Phase 6: Communication: Documented the theory, mathematics, software architecture, and test results across the thesis chapters.

## 3.4	Research Strategy: Partitioned Development and Evaluation

Because single-camera paper touch detection is an experimental computer vision task, the system components were built and tested in **isolated experimental partitions** to verify feasibility before assembling the final applications.

### 3.4.1	Partitioned Research Rationale

Trying to build the entire pipeline at once makes debugging difficult: if a keystroke fails, the issue could be caused by bad hand joint detection, incorrect scale normalization, poor homography math, or model errors [36], [50]. By dividing the project into five distinct partitions, each part was developed, tested, and refined independently.

Table 3.1 summarizes the five research partitions, their codebase folders, main technical tasks, and validation criteria.

Partitioned research strategy, component modules, and validation criteria.

| Research Partition | Codebase Directory | Core Technical Tasks | Validation Criteria |
| --- | --- | --- | --- |
| Partition 1: Layout Design and | designer/ | - PySide6 drag-and-drop designer (designer_app.py) | - Valid vector PDF export with AprilTags |
| Homography Simulation | designer/analyzer/ | - XML layout schema exporter | - Sub-pixel mapping accuracy in simulation |
|  |  | - Homography analyzer (main.py, homography_engine.py) | - Correct H transformation (PXML=H⋅Ppixel) |
| Partition 2: Dataset Creation and | mediapipeDetector/ | - 12 FPS video sub-sampling (resample_12fps.py) | - Scale-invariant landmark features |
| Pipeline Engineering | datacreator/ | - 21 MediaPipe landmark extraction | - Elimination of distance scale variance |
|  |  | - Unitless hand scale norm (normalize_landmarks.py) | - Preserved kinetic deceleration spikes |
|  |  | - Numerical velocity derivation (calculate_velocities.py) | - Clean dataset window extraction |
|  |  | - 5-frame sliding window generation (create_windows.py) |  |
|  |  | - Data quality filtering (filter_window_quality.py) |  |
| Partition 3: Deep Learning Model | mediapipeDetector/ | - Automated multi-architecture benchmark (run_all.py) | - Highest F1-score model selection |
| Benchmarking | deepLearningModels/ | - Evaluated 22 pure 2D models across 5 families (1D CNN, Attention, ResNet, BiLSTM, LSTM) | - Optimal PyTorch model trained |
|  |  | - Trained PyTorch model (best_finger_touch_lstm.pth) | - Low MAC operational counts |
| Partition 4: Real-Time | mediapipeDetector/ | - Multi-threaded camera feed capture (camera_thread.py) | - Low per-frame latency (<30 ms) |
| Responsiveness Evaluation | realtimeprocess/ | - Live model inference manager (model_manager.py) | - Smooth thread concurrency |
|  |  | - Real-time UI evaluation (main_realtime_ui.py) | - Zero frame dropping or buffer lag |
| Partition 5: AprilTag | aprilTag/ | - Camera calibration engine (main.py) | - High tag corner localization precision |
| Fiducial Tracking |  | - AprilTag corner point localization (estimater.py) | - Stable SVD homography calculation |
|  |  | - Real-time homography estimation (homography.py) | - Robust tracking under 75∘ planar tilt |

### 3.4.2	Detailed Partition Specifications and Technological Frameworks

#### 3.4.2.1	Partition 1: Layout Designer GUI and Homography Simulation Suite

Located in designer/ and designer/analyzer/, this partition focused on layout design and coordinate transformation math:

- Designer Application (designer_app.py): A PySide6 canvas where users place button boxes on an A4 page (210×297 mm), assign button identifiers, configure actions or shell commands, and automatically place AprilTag markers at the four corners.

- Export Engine: Generates printable PDF files containing key boundaries with AprilTag markers, along with matching XML files defining key coordinates xmin,ymin,xmax,ymax.

- Simulation Suite (analyzer/main.py, homography_engine.py): Simulated synthetic finger touches under various camera angles (0∘ to 75∘), verifying that the homography matrix H correctly maps camera coordinates back to XML key areas.

#### 3.4.2.2	Partition 2: Dataset Creation, Resampling, Landmark Extraction, and Scale Normalization

Located in mediapipeDetector/datacreator/, this partition processed hand motion videos into clean training data:

- Video Resampling (resample_12fps.py): Down-sampled video recordings to a uniform 12 FPS rate. This frame rate was chosen to capture finger motion clearly while keeping CPU load light for real-time processing on standard processors without a GPU.

- MediaPipe 21 Landmark Extraction and Anatomical Groupings: Extracted 2D pixel coordinates xi,yi for all 21 hand joints of the active interacting hand in each frame (num_hands=1). Single-hand tracking was used to avoid visual marker occlusion on A4 paper and to maintain low computational latency on standard CPUs. The landmarks span three functional tiers: terminal fingertips (4, 8, 12, 16, 20) for impact detection; intermediate DIP/PIP knuckle joints for flexion curvature; and MCP knuckles/wrist for the palm reference frame.

- Scale Normalization (normalize_landmarks.py): Shifted coordinates relative to the wrist P0 and divided by hand length Lhand=∥P9−P0∥2, producing unitless coordinates Pinorm. These values were sent directly without smoothing filters to preserve quick deceleration signals.

- Velocity Calculation (calculate_velocities.py): Computed numerical inter-frame velocity vectors Vit=Δxi/Δt,Δyi/Δt across consecutive frames to capture sudden impact deceleration spikes.

- Sliding Window Generation (create_windows.py): Created 5-frame temporal windows with a 2-frame overlap (stride of 3 frames), yielding input matrices XW∈R5×84 representing the single active hand (21 joints×4 values per joint: x,y,vx,vy).

- Quality Filtering (filter_window_quality.py): Removed incomplete or noisy windows where landmarks were missing or had low confidence scores.

#### 3.4.2.3	Partition 3: Deep Learning Model Benchmarking and Selection

Located in mediapipeDetector/deepLearningModels/, this partition tested multiple model designs and data views to find the best touch detector:

- Master Benchmark Engine (run_all.py): An automated cross-validation script testing 22 pure 2D model architecture combinations across five model families (1D CNNs, Attention / Transformers, 1D ResNets, BiLSTMs, and uni-directional LSTMs) across multiple feature inputs (raw pixels, wrist-relative coordinates, scale-normalized positions, velocities, and combined representations).

- PyTorch Model Selection: Testing showed that a uni-directional multi-layer PyTorch LSTM model trained on 5-frame normalized position and velocity features achieved the best F1-score (0.963) with a compact model size (0.44 MB) and ultra-low CPU inference latency (2.08 ms), saved as best_finger_touch_lstm.pth.

#### 3.4.2.4	Partition 4: Real-Time Execution, Latency, and Threading Evaluation

Located in mediapipeDetector/realtimeprocess/, this partition tested live camera capture and real-time inference speed:

- Multi-Threaded Camera Capture (camera_thread.py): Ran OpenCV camera reading in a separate thread with a safe queue to prevent frame drops.

- Inference Manager (model_manager.py): Prepared incoming 5-frame arrays and ran the PyTorch LSTM model.

- Latency Evaluation (main_realtime_ui.py): Measured total per-frame pipeline latency, achieving an end-to-end processing time of 29.09 ms (≈34.4 FPS) on a standard CPU.

#### 3.4.2.5	Partition 5: AprilTag Fiducial Localization and Planar Homography Tracking

Located in aprilTag/, this partition handled paper surface tracking:

- Camera Calibration Engine (main.py): Estimates the camera intrinsic matrix K and distortion values using checkerboard calibration images.

- AprilTag Corner Estimator (estimater.py): Detects AprilTag visual markers placed at layout corners and finds their sub-pixel corner coordinates even under steep camera angles.

- Real-Time Homography Engine (homography.py): Solves the 3×3 Planar Homography matrix H using SVD (Ah=0) to map fingertip pixels Ppixel into layout coordinates PXML=H⋅Ppixel.

## 3.5	Fact Collection and Data Gathering Mechanisms

Collecting reliable experimental data requires structured recording setups, careful labeling, feature extraction methods, and experimental controls.

### 3.5.1	Typing Interaction Dataset Capture Protocol

To train and evaluate the deep sequential touch detection models, a dataset of multi-finger typing on paper was recorded in a laboratory environment:

- Hardware Setup: Video was recorded using standard 1080p monocular RGB webcams mounted on adjustable stands at heights between 35 cm and 65 cm above the desk. Camera tilt angles ranged from 30∘ (slanted view) to 90∘ (top-down view).

- Lighting Variations: Video data was recorded under three common indoor lighting conditions: Dim Ambient (150 lux), Standard Office (500 lux), and Bright Sunlight (2,500+ lux).

- Participant Diversity: Typing samples were gathered from 15 participants with different hand sizes (Lhand between 14.2 cm and 21.5 cm), different skin complexions (Fitzpatrick scale Types I to VI), and various typing habits (from single-finger typing to multi-finger touch typing).

- Action Gesture Protocol: Participants performed thousands of deliberate touch events and non-touch hovering movements across all five fingers (Thumb, Index, Middle, Ring, Pinky). Actions included intentional surface taps, hovering over keys, lateral hand adjustments, and resting hands on paper.

### 3.5.2	Ground Truth Annotation and Temporal Alignment Procedure

Training supervised deep learning models requires exact alignment between video frames and binary touch labels yt=y1,y2,y3,y4,y5∈{0,1}5 for the five fingers (Thumb, Index, Middle, Ring, Pinky).

Ground truth touch labels were established using a two-step labeling process:

- Contact Sensor Synchronization: Physical paper pads were connected to low-voltage contact sensors, giving precise physical contact timestamps Timpact.

- Frame-by-Frame Video Inspection: Video recordings were examined frame-by-frame using video annotation tools to confirm that fingertip contact matched the sensor timestamps Timpact.

- Binary Target Labeling: For every 5-frame sliding window, a binary vector y∈{0,1}5 was assigned to show which finger or fingers touched the paper during that window.

### 3.5.3	Feature Vector Pipeline Formulation

For each video frame t, MediaPipe extracts 21 2D joint coordinates Pit=xit,yit for i∈{0,…,20}.

The hand length Lhandt is measured from the wrist (Landmark 0) to the middle MCP knuckle (Landmark 9):

Lhandt=x9t−x0t2+y9t−y0t2

Unitless scale-normalized joint coordinates Pinormt are calculated as follows:

Pinormt=xit−x0tLhandt,yit−y0tLhandt

Numerical velocity vectors Vit=vx,it,vy,it between consecutive frames are calculated at Δt=112 s:

vx,it=xinormt−xinormt−1Δt, vy,it=yinormt−yinormt−1Δt

Combining the normalized position Pinorm and velocity Vi for all 21 joints creates an 84-dimensional feature vector ft∈R84 per frame. A 5-frame temporal sliding window stacks these vectors into an input matrix XW∈R5×84.

## 3.6	Research Methodology Execution Workflow

To show how each research objective defined in Chapter 1 is systematically fulfilled, Table 3.2 presents the complete execution workflow mapping objectives to DSRM phases, research activities, software components, and evaluation metrics.

Comprehensive research methodology execution workflow mapping research objectives to technical activities, engineered artifacts, and evaluation metrics.

| Research Objective | DSRM Phase | Technical Research Activities | Engineered Artifacts / Code | Quantitative Evaluation Metrics |
| --- | --- | --- | --- | --- |
| Obj 1: Identify and Analyze | Phase 1: Problem ID | - Systematic literature review | - Literature Database (research-db/) | - 57 Cited Papers mapped |
| Existing Virtual Input | and Phase 2: Objectives | - Categorized 8 sensing modalities | - Summary Matrix (summary-matrix.md) | - Comparative Matrix (Table 2.1) |
| Frameworks and Gaps |  | - Formulated research gap | - Thesis Chapter 02 (chapter02.tex) | - Performance baseline metrics |
| Obj 2: Design Software | Phase 3: Design and | - Developed PySide6 drag-and-drop GUI | - Layout Designer App (designer_app.py) | - PDF vector alignment error (<0.1 mm) |
| Layout Designer and PDF / XML | Development | - Automated AprilTag anchor embedding | - Layout XML Schema (layout.xml) | - XML parsing latency (<5 ms) |
| Synchronized Exporter |  | - Implemented dual PDF/XML exporter | - Printable AprilTag PDF Layouts | - 100% schema validation rate |
| Obj 3: Implement Data | Phase 3: Design and | - 12 FPS sub-sampling pipeline | - Resampler (resample_12fps.py) | - Hand scale distance invariance |
| Pipeline and Hand Scale | Development | - MediaPipe 21 landmark extractor | - Normalizer (normalize_landmarks.py) | - 0% low-pass phase lag |
| Normalization Engine |  | - Unitless hand scale normalization | - Window Generator (create_windows.py) | - Feature vector dimension (84×5) |
|  |  | - Direct raw feature propagation | - Quality Filter (filter_window_quality.py) |  |
| Obj 4: Benchmark and Train | Phase 3: Design and | - Multi-model benchmark suite | - Benchmark Engine (run_all.py) | - Model F1-Score (>0.95) |
| Deep Learning Temporal Touch | Development | - Evaluated 22 pure 2D architectures | - PyTorch LSTM Model File | - Precision, Recall, ROC-AUC |
| Detection Classifier |  | - Trained PyTorch LSTM classifier | (best_finger_touch_lstm.pth) | - Model parameter size (<1 MB) |
| Obj 5: Implement Real-Time | Phase 3: Design and | - AprilTag sub-pixel corner tracker | - AprilTag Engine (aprilTag/) | - Sub-pixel corner error (<0.05 px) |
| AprilTag Homography Tracking | Development | - SVD Planar Homography calculator | - Homography Matrix (H∈R3×3) | - Mapping error under tilt (<0.8 mm) |
| and Coordinate Transformation |  | - Planar transformation engine | - Coordinate Mapper (PXML=H⋅Ppixel) | - Real-time tracking speed (>60 FPS) |
| Obj 6: Assemble and Evaluate | Phase 4: Demo | - Integrated Application 1 + App 2 | - Runtime Engine GUI | - System end-to-end latency (<30 ms) |
| Integrated Two-Application | Phase 5: Evaluation | - Multi-threaded camera watch | - Thread Manager (camera_thread.py) | - System frame rate (>34 FPS) |
| Virtual Keyboard Suite | Phase 6: Comm. | - Empirical latency and throughput test | - XML Action Mapper | - Keystroke trigger accuracy (>96%) |

### 3.6.1	Detailed Sub-Phase Execution Breakdown

Each phase of the methodology was executed systematically:

- Problem Identification and Relevance Justification (Obj 1): Established the research gap by studying 57 prior papers across 8 sensing methods, defining the baseline parameters for single-camera paper keyboards.

- Layout Designer and PDF/XML Exporter (Obj 2): Built Application 1 Layout Designer (designer/designer_app.py) using PySide6. The software produces printable PDF sheets with embedded AprilTag markers and matching XML layout definition files.

- Data Pipeline and Scale Normalization Engine (Obj 3): Implemented 12 FPS video resampling (resample_12fps.py), 21 landmark extraction, scale normalization (Lhand) without low-pass filtering (normalize_landmarks.py), velocity calculation (calculate_velocities.py), 5-frame sliding windowing (create_windows.py), and quality filtering (filter_window_quality.py).

- Deep Learning Model Benchmarking (Obj 4): Used run_all.py to benchmark 22 pure 2D model architecture combinations under 5-fold cross-validation to select the best PyTorch LSTM model (best_finger_touch_lstm.pth).

- AprilTag Homography Engine (Obj 5): Developed real-time AprilTag corner detection and SVD homography calculation (H∈R3×3) in aprilTag/, achieving sub-pixel corner precision.

- System Assembly and Real-Time Evaluation (Obj 6): Integrated Application 1 and Application 2 Runtime Engine with multi-threaded camera acquisition (camera_thread.py), achieving an end-to-end processing time of 29.09 ms (34.4 FPS).

## 3.7	Project Management Methodology and Governance

Managing a research project that involves computer vision, deep learning, GUI development, and planar geometry requires an organized project management framework.

### 3.7.1	Agile Scrum Research Adaptation

This project used an adapted Agile Scrum Methodology organized around research modules. Work was structured into two-week Sprint cycles:

- Sprint Backlog Management: Technical tasks were organized into user stories in a sprint backlog (for example, "Implement scale normalization and direct feature propagation", "Benchmark 1D CNN vs LSTM models", "Optimize 12 FPS pipeline for CPU performance", "Integrate AprilTag homography matrix").

- Sprint Reviews and Incremental Demos: At the end of every two weeks, code modules were tested and checked against numerical goals.

- Iterative Refinement Loops: If model accuracy fell below expectations, the workflow returned to feature normalization tuning or hyperparameter adjustments.

### 3.7.2	Project Timeline and Gantt Chart Schedule

The project was conducted over a 9-month timeframe divided into sequential research phases. Figure 3.2 shows the project timeline Gantt chart.

Project execution Gantt chart schedule across 9 monthly research milestones.

### 3.7.3	Risk Mitigation Matrix

To ensure the project was completed on time, potential technical risks were identified, assessed for impact, and paired with proactive mitigation steps, as summarized in Table 3.3.

Technical risk management, impact assessment, and mitigation matrix.

| Identified Technical Risk | Probability | Impact | Proactive Risk Mitigation Strategy |
| --- | --- | --- | --- |
| R1: AprilTag Corner Occlusion | Medium | High | - Place multiple AprilTag markers along the layout borders. |
| By Typing Hands |  |  | - Require only 4 clear corner points across the sheet to compute homography H. |
| R2: High Inference Latency | Medium | High | - Sub-sample video streams to 12 FPS (resample_12fps.py). |
| Degrading Real-Time FPS |  |  | - Choose lightweight PyTorch LSTM models over heavy 3D CNN architectures. |
| R3: False Positive Taps | High | High | - Use 5-frame sliding temporal windows to capture motion context. |
| From Hovering Fingers |  |  | - Train classifier on normalized velocity vectors capturing touch deceleration. |
| R4: Scale Distortion Across | High | Medium | - Normalize landmark coordinates by unitless hand length Lhand. |
| Diverse Hand Sizes and Distances |  |  | - Avoid fixed pixel limits and use scale-invariant feature ratios. |
| R5: Hardware Dependency on | Medium | High | - Use a compact LSTM model (<1 MB) that runs efficiently on standard CPUs. |
| High-End GPUs or Fast Sensors |  |  | - Standardize pipeline at 12 FPS for CPU execution with any standard webcam. |

### 3.7.4	Ethical Considerations and Data Privacy Protocols

Because video recording involves human participants, strict ethical and data privacy practices were followed throughout the research:

- Informed Consent: All 15 participants received information sheets explaining the study and signed voluntary consent forms before video recording.

- Data Privacy and Anonymization: Raw video recordings containing participant faces were stored only on secure local drives. Once MediaPipe extracted the 2D joint coordinates, raw video files were permanently removed.

- Irreversible Feature Data: The saved landmark dataset contains only unitless floating-point coordinate arrays (xinorm,yinorm). These coordinate files contain no facial images or personal identifying information, protecting participant privacy.

## 3.8	Chapter Summary

This chapter presented the research methodology for designing, implementing, and evaluating the customizable paper virtual keyboard system.

Section 3.2 explained the Positivist empirical paradigm. Section 3.3 described the Deductive approach and the 6-phase Design Science Research Methodology (DSRM). Section 3.4 detailed the five isolated development partitions. Section 3.5 described the interaction dataset collection, 12 FPS resampling, MediaPipe joint extraction, and 84-dimensional scale-normalized feature vectors. Section 3.6 presented the execution workflow table connecting objectives to software modules and metrics. Finally, Section 3.7 outlined the Agile Scrum process, project Gantt chart, risk mitigation matrix, and data privacy procedures.

The methodological foundation established in this chapter guides the system requirements, architectural design, and experimental results presented in the following chapters.

# 4	System Requirement Specification

## 4.1	Chapter Overview

Building a reliable, fast, and customizable paper-based virtual keyboard requires a clear System Requirement Specification (SRS). Developing vision-based human-computer interaction software without physical switches requires careful systems analysis. This ensures that camera tracking, hand joint normalization, homography calculation, and deep learning touch detection satisfy practical user needs [1], [19].

This chapter presents the requirements and UML models for the two-application system: Application 1 (Layout Designer Suite designer/designer_app.py) and Application 2 (Runtime Virtual Keyboard Engine and Command Mapper). Section 4.2 provides a stakeholder analysis, identifying user groups along with a Power-Interest matrix and design trade-offs. Section 4.3 explains the operationalization process, showing how research objectives and test data map to software requirements. Section [sec:req_uml_analysis] presents a full UML analysis, including Use Case diagrams with formal specifications, Class diagrams with Class Responsibility Collaboration (CRC) descriptions, Activity diagrams, Sequence diagrams, Statechart diagrams, and Deployment diagrams. Section 4.4 explains the overall system architecture framework and design patterns. Section 4.5 details the Functional and Non-Functional requirements. Finally, Section 4.6 summarizes the chapter.

## 4.2	Stakeholder Analysis

Stakeholder analysis identifies the people, groups, and systems that use or are affected by the virtual keyboard. Understanding stakeholder expectations ensures the system addresses real user needs [1].

### 4.2.1	Stakeholder Identification and Classification

Stakeholders for the paper virtual keyboard system are divided into four main groups:

- Primary End-Users: People who directly type on the printed paper sheets to enter text or control computers. This group includes everyday computer users, regular typists, and users with motor difficulties who need custom large-key layouts [28].

- Domain-Specific Specialists: Professional users in specialized fields (such as digital audio editors, video editors, industrial machine operators, and CAD designers) who need custom macro pads for specific software shortcuts [1].

- Healthcare and Clinical Staff: Doctors, nurses, and laboratory technicians working in sterile environments where regular plastic keyboards can collect dirt and spread bacteria [1].

- System Engineers and Researchers: Computer vision researchers, software developers, and system administrators who design layouts, configure XML files, train neural touch models (run_all.py), and maintain the runtime software.

Table 4.1 outlines the stakeholder groups, their roles, and their key requirements.

Comprehensive stakeholder identification, operational roles, and key requirements matrix.

| Stakeholder Group | Specific User Category | Operational Role and Context | Key System Requirements |
| --- | --- | --- | --- |
| Primary End-Users | General Desktop Typists | - Daily text entry and computer navigation | - Low entry latency (<30 ms) |
|  |  | - Desktop computing environments | - High touch trigger accuracy (>95%) |
|  | Accessibility Users | - Motor-impaired or low-dexterity users | - Enlarged key size customization |
|  |  | - Customized physical layout sheets | - Low Index of Difficulty (ID) under Fitts’ Law |
| Domain Specialists | Audio / Video Engineers | - Dynamic macro triggering in DAWs | - Multi-key shell script binding |
|  | Industrial Operators | - Machine tool control pads | - Dynamic layout XML hot-swapping |
| Healthcare / Clinical | Surgeons and ICU Staff | - Touch interaction in sterile operating rooms | - Disposable, sanitizable paper sheets |
|  | Cleanroom Technicians | - Pathogen-free computer interface | - Zero physical switch contamination |
| System Engineers | Vision Researchers | - Algorithm benchmarking (run_all.py) | - Scale-normalized feature exports |
|  | Software Administrators | - System command mapping and deployment | - Validated XML schema synchronization |

### 4.2.2	Stakeholder Power-Interest Matrix

To help prioritize requirements, Figure 4.1 maps stakeholder groups on a Power-Interest matrix.

Stakeholder Power-Interest Matrix governing project requirement prioritization.

As shown in Figure 4.1, System Engineers and Domain Specialists are Key Players (High Power, High Interest), requiring close attention to ensure architectural flexibility. Primary Typists and Accessibility Users have High Interest, guiding responsiveness and usability needs.

### 4.2.3	Stakeholder Conflict Resolution and Requirement Trade-offs

Differences in stakeholder priorities were resolved through practical engineering trade-offs:

- Typing Speed vs Computing Load (Typists vs Administrators): Typists need low delay, while administrators want the software to run on standard computers without high CPU load. This was resolved by using a 12 FPS video standard (resample_12fps.py) and a compact PyTorch LSTM model (<1 MB), ensuring real-time response on standard CPUs without requiring a GPU.

- Custom Layouts vs Tracking Stability (Domain Specialists vs Vision Researchers): Custom macro layouts can have keys placed near corners, which might cover tracking markers. This was solved by placing AprilTag markers along the outer margins, requiring only four visible corners to calculate homography matrix H.

- Paper Disposal vs Profile Persistence (Medical Staff vs System Admins): Resolved by storing layout geometries in XML files, allowing paper sheets to be thrown away or reprinted at any time without having to reconfigure the computer.

## 4.3	Operationalization Process

The operationalization process connects high-level research objectives and experimental data gathering with concrete software requirements.

### 4.3.1	Mapping Research Data Gathering to System Requirements

System requirements were developed by combining findings from 57 literature papers, video data recorded from 15 participants, and CPU performance measurements.

Empirical techniques and data sources used to formulate system requirements.

| Research Objective | Data Gathering / Empirical Technique | Requirement Validity Justification | Mapped System Requirement Module |
| --- | --- | --- | --- |
| Obj 1: Identify Gaps and | - Systematic literature review (57 papers) | - Establishes baseline latency (<30 ms) | - Core System Architectural Scope |
| Establish Baselines | - Benchmark matrix (Table 2.1) | - Justifies monocular RGB paper approach | - Hardware/Software Constraints |
| Obj 2: Software Layout | - User study interviews (N=15) | - Confirms need for arbitrary key sizing | - Layout Designer App (designer_app.py) |
| Designer and PDF/XML Exporter | - Ergonomic Fitts’ law evaluation | - Validates synchronized PDF/XML export | - PDF Vector Renderer and XML Exporter |
| Obj 3: Landmark Pose and | - 15-participant video dataset (Lhand) | - Proves scale invariance across distance Z | - MediaPipe Pose Extractor |
| Scale Normalization Engine | - Distance scaling experiments | - Confirms scale and resolution invariance | - Scale Normalizer (normalize_landmarks.py) |
| Obj 4: Deep Temporal | - Automated benchmark (run_all.py) | - Benchmarks 22 pure 2D architectures | - Sliding Window Buffer (create_windows.py) |
| Touch Detection Classifier | - 5-fold cross-validation on 5-frame windows | - Selects optimal PyTorch LSTM model | - PyTorch Classifier (best_finger_touch_lstm.pth) |
| Obj 5: AprilTag Fiducial | - Tilt camera experiments (0∘ to 75∘) | - Solves SVD Planar Homography H | - AprilTag Tracker (aprilTag/estimater.py) |
| Homography Tracking Engine | - Sub-pixel corner localization tests | - Ensures sub-millimeter mapping accuracy | - Planar Homography Solver (homography.py) |
| Obj 6: System Assembly | - End-to-end CPU latency profiling | - Validates real-time execution (34.4 FPS) | - Multi-threaded Runtime Engine |
| and Real-Time Execution | - Multi-threaded stream testing | - Confirms XML action dispatching | - Action Command Mapper GUI |

### 4.3.2	Qualitative Data Gathering from User Interactivity Studies

Understanding user requirements involved gathering feedback from 15 test participants with varying typing styles. Participants noted that fixed keyboard layouts limit efficiency when working with specialized desktop applications. Users requested customizable macro pads, validating the need for the Layout Designer tool (designer/designer_app.py).

### 4.3.3	Empirical Kinematic Data Translation to Landmark Requirements

### 4.3.4	Use Case Analysis and Formal Specifications

The system architecture includes two human actors: the Layout Designer User (using Application 1) and the Runtime Operator User (using Application 2). Secondary system actors include the Monocular RGB Camera and the Operating System Shell.

Figure 4.2 shows the Use Case Diagram covering system boundaries, actors, and use case interactions.

System Use Case Diagram covering Application 1 and Application 2 workflows.

Detailed formal Use Case Specifications for core system operations are provided in Tables 4.3, 4.4, 4.5, 4.6, 4.7, 4.8, 4.9, and 4.10.

Formal Use Case Specification: UC-01 Design Custom Key Layout.

| Use Case Attribute | Specification Detail |
| --- | --- |
| Use Case ID and Title | UC-01: Design Custom Key Layout |
| Primary Actor | Layout Designer User |
| Description | User creates an arbitrary key layout on a PySide6 A4 GUI canvas, positions buttons, assigns button IDs, and configures key dimensions. |
| Preconditions | Application 1 (designer/designer_app.py) is launched on host desktop PC. |
| Postconditions | Layout graphical elements and button metadata are initialized in GUI memory. |
| Main Success Scenario | 1. User launches Designer GUI and creates key bounding boxes on grid. |
|  | 2. User sets key properties (Button ID, default values, command bindings). |
|  | 3. System automatically places AprilTag fiducial anchors at four layout corners. |
| Alternative Flows | 1a. Overlapping key boundaries detected: System highlights error and prompts user. |

Formal Use Case Specification: UC-02 Export Synchronized PDF/XML Artifacts.

| Use Case Attribute | Specification Detail |
| --- | --- |
| Use Case ID and Title | UC-02: Export Synchronized PDF and XML Artifacts |
| Primary Actor | Layout Designer User |
| Description | System renders vector PDF sheet with embedded AprilTag anchors for printing and exports XML layout specification file defining bounding boxes. |
| Preconditions | Valid key layout designed in UC-01 without boundary overlaps. |
| Postconditions | Printable PDF file and matching XML specification file saved to disk directory. |
| Main Success Scenario | 1. User selects Export option from Application 1 file menu. |
|  | 2. System validates AprilTag anchor coordinates along layout margins. |
|  | 3. System generates vector PDF print sheet and writes XML layout schema to disk. |
| Alternative Flows | 2a. Disk write permission denied: System alerts user and prompts for alternative path. |

Formal Use Case Specification: UC-03 Select Camera Input Stream.

| Use Case Attribute | Specification Detail |
| --- | --- |
| Use Case ID and Title | UC-03: Select Camera Input Stream |
| Primary Actor | Runtime Operator User |
| Description | User selects the active monocular RGB camera input device stream from Application 2 Setup GUI. |
| Preconditions | Camera device connected to host desktop PC via USB. |
| Postconditions | OpenCV camera capture thread initialized with active device index. |
| Main Success Scenario | 1. Operator launches Application 2 Setup GUI. |
|  | 2. System enumerates connected video capture devices. |
|  | 3. Operator selects target camera device from dropdown menu. |
|  | 4. System initializes OpenCV video stream and displays live preview feed. |
| Alternative Flows | 2a. No camera detected: System displays hardware error alert. |

Formal Use Case Specification: UC-04 Load Layout XML Schema.

| Use Case Attribute | Specification Detail |
| --- | --- |
| Use Case ID and Title | UC-04: Load Layout XML Schema |
| Primary Actor | Runtime Operator User |
| Description | System parses exported XML layout specification file and constructs target coordinate lookup tables. |
| Preconditions | Valid layout XML file generated by Application 1 saved on disk. |
| Postconditions | Key bounding boxes and button ID mappings loaded into shared runtime memory. |
| Main Success Scenario | 1. Operator clicks ’Load Layout XML’ in Application 2 Setup GUI. |
|  | 2. System parses XML tags, extracting key bounding boxes and AprilTag anchor positions. |
|  | 3. System builds spatial lookup data structures for real-time key matching. |
| Alternative Flows | 2a. Corrupted XML schema: System displays validation error message. |

Formal Use Case Specification: UC-05 Map Buttons to Keystrokes/Commands.

| Use Case Attribute | Specification Detail |
| --- | --- |
| Use Case ID and Title | UC-05: Map Buttons to Keystrokes or System Shell Commands |
| Primary Actor | Runtime Operator User |
| Description | Operator maps each layout button ID to specific keystrokes or system shell commands. |
| Preconditions | Layout XML schema successfully loaded in UC-04. |
| Postconditions | Action mapping dictionary registered in runtime execution engine. |
| Main Success Scenario | 1. System populates interactive action mapping table with loaded button IDs. |
|  | 2. Operator assigns keystrokes (e.g., ‘’A’‘, ‘’Enter’‘) or shell scripts to button IDs. |
|  | 3. System registers action bindings in dispatcher queue. |
| Alternative Flows | 2a. Invalid shell command syntax: System highlights invalid command string. |

Formal Use Case Specification: UC-06 Save and Load Action Configuration Profiles.

| Use Case Attribute | Specification Detail |
| --- | --- |
| Use Case ID and Title | UC-06: Save and Load Action Configuration Profiles |
| Primary Actor | Runtime Operator User |
| Description | Operator saves custom button-to-command mappings to disk as reusable JSON configuration profiles and switches between multiple action combination profiles for the exact same physical printed layout sheet without reprinting the paper. |
| Preconditions | Action mapping table populated in UC-05. |
| Postconditions | Profile JSON file saved to disk or loaded into active mapping table. |
| Main Success Scenario | 1. Operator clicks ’Save Profile’ in Application 2 GUI. |
|  | 2. System serializes action mapping table to JSON configuration file (e.g., ’Work_Mode.json’). |
|  | 3. Operator reloads or switches between configuration profiles (e.g., ’DAW_Mode.json’, ’Gaming_Mode.json’) for the exact same physical paper layout without reprinting. |
| Alternative Flows | 3a. Profile file version mismatch: System performs automatic migration. |

Formal Use Case Specification: UC-07 AprilTag Tracking and Planar Homography Estimation.

| Use Case Attribute | Specification Detail |
| --- | --- |
| Use Case ID and Title | UC-07: Track AprilTag Anchors and Compute Planar Homography Matrix H |
| Primary Actor | Monocular RGB Camera (Hardware) / Background Runtime Engine |
| Description | The system finds sub-pixel AprilTag corner points in camera frames and calculates the 3×3 Planar Homography matrix H using SVD. |
| Preconditions | Camera feed is running; printed paper layout is placed within the camera view. |
| Postconditions | Homography matrix H∈R3×3 is updated in shared runtime memory. |
| Main Success Scenario | 1. Camera thread captures video frame and passes it to the AprilTag module. |
|  | 2. AprilTag detector finds tag corners with sub-pixel precision. |
|  | 3. System matches image corners with the XML anchor coordinates. |
|  | 4. SVD algorithm solves Ah=0, updating H for spatial mapping. |
| Alternative Flows | 3a. Tags blocked by hands (fewer than 4 tags visible): System uses previous valid H matrix. |

Formal Use Case Specification: UC-08 Hand Landmark Scale Normalization and LSTM Touch Classification.

| Use Case Attribute | Specification Detail |
| --- | --- |
| Use Case ID and Title | UC-08: Scale-Normalize Keypoints, Classify Touch via PyTorch LSTM, and Dispatch Action |
| Primary Actor | Background Runtime Engine / OS Shell Execution Engine |
| Description | System extracts 21 MediaPipe hand keypoints, normalizes coordinates using hand length Lhand, evaluates a 5-frame sliding window with PyTorch LSTM, maps the fingertip to the XML key region via H, and triggers the mapped keypress or command. |
| Preconditions | Layout XML loaded; PyTorch model (best_finger_touch_lstm.pth) loaded into memory. |
| Postconditions | Keystroke or system shell command executed; active touch status updated on GUI. |
| Main Success Scenario | 1. MediaPipe extracts 21 2D joint coordinates from RGB camera frame. |
|  | 2. System normalizes keypoints relative to wrist and middle MCP hand length Lhand. |
|  | 3. System fills 5-frame sliding window and feeds matrix to PyTorch LSTM. |
|  | 4. PyTorch LSTM classifies active finger surface contact event (P>0.90). |
|  | 5. System transforms active fingertip pixel Ppixel to XML space (PXML=H⋅Ppixel). |
|  | 6. XML key lookup locates hit key ID and dispatches mapped OS shell command. |
| Alternative Flows | 4a. Hovering movement detected (P≤0.90): System suppresses key actuation. |

### 4.3.5	System Class Diagram and Class Responsibility Breakdown

The system architecture follows object-oriented design principles, dividing layout creation, camera capture, feature processing, deep learning inference, homography mapping, and command execution into separate classes.

Figure 4.3 presents the System Class Diagram showing class structures, attributes, methods, and relationships.

System Class Diagram covering Layout Designer and Runtime Keyboard Engine modules.

Class Responsibility Collaboration (CRC) breakdowns for core system classes are explained below:

- LayoutDesignerApp: Responsible for managing the PySide6 canvas, tracking graphical button objects, and invoking export functions. Collaborates with KeyButtonGraphic and export utilities.

- RuntimeKeyboardEngine: Acts as the main coordinator for background vision processing. Manages threads, calls hand pose tracking, and dispatches key events. Collaborates with CameraThread, MediaPipePoseExtractor, and KeyActionDispatcher.

- ScaleNormalizationEngine: Shifts joint coordinates relative to the wrist P0 and computes unitless scale ratios using hand length Lhand. Collaborates with SlidingWindowBuffer.

- HomographyMappingEngine: Takes sub-pixel AprilTag corner points and calculates matrix H∈R3×3 using SVD. Collaborates with AprilTagTracker and KeyActionDispatcher.

### 4.3.6	System Activity Diagram

The runtime engine uses parallel execution threads to maintain real-time performance on standard CPUs.

Figure 4.4 presents the System Activity Diagram showing parallel workflows for video capture, AprilTag tracking, hand pose detection, sliding window LSTM touch inference, and homography action triggering.

System Activity Diagram illustrating parallel vision execution threads.

### 4.3.7	Sequence Diagrams Mapped to Key Use Cases

Sequence diagrams illustrate the order of messages between software components during execution.

Figure 4.5 shows Sequence Diagram 1 for layout design and synchronized file export (UC-01 and UC-02).

Sequence Diagram 1: Layout Design and Synchronized Export Sequence (UC-01 and UC-02).

Figure 4.6 shows Sequence Diagram 2 for real-time vision, AprilTag tracking, LSTM inference, and command execution (UC-07 and UC-08).

Sequence Diagram 2: Real-Time Vision, AprilTag Tracking, PyTorch LSTM Inference, and Action Execution Sequence (UC-07 and UC-08).

### 4.3.8	System Statechart Diagram

The lifecycle states of a finger touch interaction are modeled using a Statechart diagram.

Figure 4.7 presents the System Statechart Diagram showing states and transition triggers.

System Statechart Diagram governing finger touch lifecycle transitions.

### 4.3.9	System Deployment Diagram

Figure 4.8 presents the System Deployment Diagram showing hardware devices, execution environments, and software components.

System Deployment Diagram detailing hardware nodes, execution processes, and software artifacts.

## 4.4	Proposed System Architecture Framework

The proposed system architecture is organized as a two-application software suite.

Figure 4.9 presents the System Architecture Framework Diagram showing data flow between Application 1 (Layout Designer Suite) and Application 2 (Runtime Virtual Keyboard Engine and Command Mapper).

Proposed System Architecture Framework Diagram highlighting Application 1 and Application 2 synchronized workflows.

As shown in Figure 4.9, the design separation keeps layout authoring distinct from live camera processing:

- Application 1 (Layout Designer Suite): Desktop GUI tool that exports printable vector PDF layouts with border AprilTag markers on normal paper, together with XML layout coordinate definitions.

- Application 2 (Runtime Keyboard Engine): Contains a Setup and Command Mapper GUI (Component A) to configure key-to-command mappings and choose a camera stream, and a Background Vision Watch Engine (Component B) that tracks AprilTag homography H, extracts MediaPipe hand landmarks, performs scale normalization (Lhand), evaluates a 12 FPS 5-frame sliding window with PyTorch LSTM on CPU, transforms coordinates (PXML=H⋅Ppixel), and dispatches key commands.

### 4.4.1	Architectural Design Patterns Applied

The software design uses standard design patterns to keep the code modular and maintainable:

- Model-View-Controller (MVC): Used in the Layout Designer (designer_app.py) to separate the visual canvas (View) from the XML coordinate data (Model) and user mouse actions (Controller).

- Producer-Consumer Thread Queue: Used in Application 2 to buffer webcam frames captured by camera_thread.py into a thread-safe Queue read by the model manager (model_manager.py), avoiding dropped video frames.

- Strategy Pattern for Action Dispatching: Separates regular keyboard typing from shell script execution into modular command handlers.

- Singleton Model Manager: Ensures the trained PyTorch touch model (best_finger_touch_lstm.pth) is loaded into memory only once during startup.

## 4.5	Functional and Non-Functional Requirements

System requirements are split into Functional Requirements (FR) and Non-Functional Requirements (NFR).

### 4.5.1	Functional Requirements (FR)

Functional requirements define the core functions and services provided by the system, as shown in Table 4.11.

Detailed Functional Requirements (FR) specification matrix.

| Req ID | Requirement Title | Detailed Functional Requirement Description |
| --- | --- | --- |
| FR-01 | Interactive Layout Creation | PySide6 GUI canvas must enable users to add, resize, reposition, and delete key buttons on an A4 layout grid (210×297 mm). |
| FR-02 | AprilTag Anchor Placement | Designer app must automatically calculate and position AprilTag visual anchors at four border layout corners. |
| FR-03 | Vector PDF Layout Exporter | Application 1 must render and export high-resolution vector PDF layouts with embedded AprilTags for physical paper printing. |
| FR-04 | XML Schema Generation | Application 1 must export structural XML files specifying key IDs, bounding box coordinates xmin,ymin,xmax,ymax, default values, and tag locations. |
| FR-05 | Single Regular RGB Camera Selector | Application 2 Setup GUI must detect and allow users to select from any single regular monocular RGB camera stream (commodity webcams or laptop sensors across regular/low resolutions without specialized hardware). |
| FR-06 | XML Specification Loader | Application 2 must parse exported XML layout files and dynamically construct target coordinate lookup tables. |
| FR-07 | Keystroke Action Mapping | Setup GUI must enable users to map key IDs to single keystrokes (e.g., ‘’A’‘, ‘’Space’‘, ‘’Enter’‘). |
| FR-08 | System Command Mapping | Setup GUI must enable users to map key IDs to executable system shell commands or external Python scripts. |
| FR-09 | Action Profile Multiplexing | System must allow users to save, load, and switch between multiple distinct action combination profiles (JSON/XML) for the exact same physical printed layout sheet without reprinting the paper. |
| FR-10 | Multi-Threaded Video Capture | Runtime engine must run OpenCV frame acquisition in a dedicated background thread to prevent frame drops. |
| FR-11 | AprilTag Sub-Pixel Tracking | AprilTag tracker engine must detect tag quads and extract sub-pixel corner coordinates under perspective camera tilt. |
| FR-12 | SVD Homography Solver | Engine must compute the 3×3 Planar Homography matrix H using SVD (Ah=0) from tracked AprilTag correspondences. |
| FR-13 | MediaPipe 21 Joint Pose | Engine must regress 2D pixel coordinates for 21 anatomical hand joint keypoints of the active interacting hand (num_hands=1) per video frame. |
| FR-14 | Unitless Hand Scale Norm | System must translate keypoints to wrist origin P0 and scale coordinates by middle MCP hand length Lhand=∥P9−P0∥2. |
| FR-15 | Direct Raw Propagation | Feature pipeline must propagate scale-normalized coordinates directly to classifier without artificial temporal low-pass smoothing. |
| FR-16 | 5-Frame Window Generation | System must assemble 84-dimensional feature vectors into sliding windows of 5 frames with 2-frame overlap (stride of 3 frames) at 12 FPS representing the active single hand. |
| FR-17 | PyTorch LSTM Touch Inference | Lightweight PyTorch model (best_finger_touch_lstm.pth) must classify active finger paper touch contact probability (P>0.90) across all 5 digits on standard CPU. |
| FR-18 | Planar Coordinate Mapping | Engine must map active fingertip pixel coordinates through H into XML layout space (PXML=H⋅Ppixel) and execute action. |

### 4.5.2	Detailed Functional Requirements Subsection Breakdown

The 18 functional requirements are grouped into four main subsystem areas:

- Layout Designer Domain (FR-01 to FR-04): Implemented in Application 1 (designer/designer_app.py). Ensures that creating buttons, calculating bounding boxes, placing AprilTag anchors, rendering vector PDFs, and writing XML layout files work smoothly for any custom keyboard layout.

- Setup and Command Mapper Domain (FR-05 to FR-09): Implemented in Application 2 Component A. Manages camera selection across standard and low resolutions, XML layout loading, custom keystroke and shell command assignments, and saving or loading profile files, allowing users to switch action combinations on the exact same printed paper sheet without reprinting.

- Vision Perception and Scale Normalization Domain (FR-10 to FR-16): Implemented in Application 2 Component B. Handles multi-threaded camera capture (camera_thread.py), AprilTag tracking (aprilTag/estimater.py), MediaPipe single-hand joint tracking (num_hands=1), hand length scale normalization (Lhand) with direct feature propagation, and 12 FPS 5-frame sliding window creation (create_windows.py).

- Touch Classification and Command Execution Domain (FR-17 to FR-18): Implemented in Application 2 Component B. Manages CPU-based PyTorch LSTM touch inference (best_finger_touch_lstm.pth) for all 5 digits of the active hand, planar homography coordinate mapping (PXML=H⋅Ppixel), XML key detection, and keystroke or shell command triggering.

### 4.5.3	Non-Functional Requirements (NFR)

Non-functional requirements specify quality goals, system constraints, and performance targets, as detailed in Table 4.12.

Detailed Non-Functional Requirements (NFR) specification matrix.

| Req ID | Quality Attribute | Detailed Non-Functional Performance Constraint |
| --- | --- | --- |
| NFR-01 | Execution Latency | End-to-end processing latency per frame must not exceed 83.3 ms (≥12 FPS), operating in near real-time on standard commodity CPUs. |
| NFR-02 | Touch Classification Accuracy | PyTorch LSTM model must achieve a touch classification F1-score ≥0.95 across all five active fingers. |
| NFR-03 | Sub-Pixel Tracking Error | AprilTag corner localization error must not exceed 0.05 pixels under perspective inclination angles up to 75∘. |
| NFR-04 | Scale Distance Invariance | System must maintain consistent touch detection accuracy across camera operating distances ranging from 35 cm to 65 cm. |
| NFR-05 | Memory Footprint | PyTorch LSTM model size must not exceed 1 MB; total runtime RAM footprint must remain <500 MB. |
| NFR-06 | CPU Compute Footprint | Runtime engine must execute near real-time vision loops (≥12 FPS) on standard commodity CPUs without requiring GPU acceleration. |
| NFR-07 | Portability | Software application suite must run across Linux (Ubuntu/Debian) and Windows operating systems. |
| NFR-08 | Data Privacy and GDPR | System must process video frames locally and permanently discard raw camera frames after extracting anonymized landmarks. |
| NFR-09 | Illumination Robustness | Vision pipeline must operate reliably across indoor illuminance levels ranging from 150 lux (dim ambient) to 2,500+ lux (sunlight). |
| NFR-10 | Usability | Interactive layout design in Application 1 must allow non-technical users to create custom keypads in <5 minutes. |

### 4.5.4	Detailed Non-Functional Quality Constraints

The 10 non-functional requirements set key operational targets:

- Performance and Real-Time Execution (NFR-01, NFR-06): The system provides an end-to-end frame processing time of under 30 ms (well within the 12 FPS budget of 83.3 ms) on standard commodity CPUs without requiring a dedicated GPU.

- Touch Classification Accuracy (NFR-02): The PyTorch LSTM model trained on 5-frame scale-normalized feature windows achieves an F1-score of 0.963 across all five fingers.

- Tracking Precision and Camera Tilt Support (NFR-03, NFR-04): AprilTag corner tracking reaches sub-pixel accuracy (0.038 px), maintaining stable homography calculations under camera tilt angles up to 75∘.

- Data Privacy Protection (NFR-08): The system processes video feeds locally in memory and deletes raw camera frames after extracting 2D joint coordinates, keeping only anonymized coordinate arrays (xinorm,yinorm).

## 4.6	Chapter Summary

This chapter presented the System Requirement Specification (SRS) and Unified Modeling Language (UML) analysis for the customizable paper virtual keyboard system.

Section 4.2 identified and classified primary, secondary, and system stakeholders, providing a Power-Interest matrix and design trade-off resolutions. Section 4.3 mapped research findings and data collection to system requirements. Section [sec:req_uml_analysis] provided UML diagrams including Use Case diagrams with specifications for 8 core use cases, Class diagrams with CRC descriptions, Activity diagrams, Sequence diagrams, Statechart diagrams, and Deployment diagrams. Section 4.4 described the System Architecture Framework Diagram and architectural design patterns. Finally, Section 4.5 specified 18 Functional Requirements and 10 Non-Functional Requirements with domain breakdowns.

The requirements and architectural models established in this chapter provide a direct guide for the software implementation, algorithm design, and testing presented in the subsequent chapters.

# 5	Implementation and Software Design

## 5.1	Chapter Overview

Turning theoretical computer vision models, planar homography geometry, and sequential deep learning into a working real-time virtual keyboard on standard plain paper requires a well-structured software architecture. Creating a camera-based human-computer interaction system that works with a single standard webcam, without specialized hardware (such as 3D depth sensors, Time-of-Flight cameras, laser projectors, or electronic gloves), requires clean software patterns, efficient multi-threading, accurate mathematical signal processing, and clear modular separation [1], [19].

Designing a monocular paper virtual keyboard presents several engineering challenges. Unlike physical mechanical keyboards or touchscreen devices, plain paper provides no physical switch movement, key travel, or electrical signals when a user presses a key. Therefore, touch detection must rely entirely on vision, tracking finger movements and speed changes across consecutive webcam frames [6], [12]. In addition, a single regular RGB camera cannot directly measure 3D depth, creating ambiguity when a finger hovers close above the paper [9]. Solving these issues requires combining vision algorithms, scale-invariant feature extraction, deep learning models, and desktop event dispatchers running on a standard CPU without requiring a GPU.

To make development and debugging manageable, the system components were built and tested across isolated partitions before assembling the complete system. The software suite consists of two main applications:

- Application 1 (Layout Designer Suite designer/designer_app.py): An interactive PySide6 desktop GUI tool where users design custom button layouts, position keys, assign commands, and export synchronized printable PDF sheets (with embedded AprilTag markers for standard paper printing) and layout XML files.

- Application 2 (Runtime Virtual Keyboard Engine and Command Mapper): A runtime software application containing a Setup GUI (Component A) for mapping button IDs to keystrokes or shell commands across any standard webcam feed, and a Background Live Watch Engine (Component B) that tracks AprilTag markers on plain paper to compute the 3×3 Homography matrix, tracks MediaPipe hand landmarks, scale-normalizes joint coordinates by hand length (Lhand), evaluates 12 FPS 5-frame sliding windows using a PyTorch LSTM model (best_finger_touch_lstm.pth) on a standard CPU, and triggers the mapped system actions.

This chapter describes the implementation details of the software system. Section 5.2 explains the algorithm designs, workflow block diagrams, pseudocode algorithms, CPU timing budgets, and technology choices. Section 5.3 details the implementation of the five research partitions and the two main application GUIs. Section 5.4 discusses practical engineering challenges faced during development and their software solutions. Finally, Section 5.5 summarizes the chapter.

## 5.2	Algorithmic Design and Software Engineering Steps

Developing the virtual keyboard software followed an organized engineering process covering layout design, video capture, hand joint extraction, scale normalization, deep learning inference, AprilTag marker tracking, and system action execution.

### 5.2.1	Framework Engineering Workflow Breakdown

The complete system is split into two synchronized application tools working together across an end-to-end dataflow pipeline, as shown in the block diagram in Figure 5.1.

End-to-End multi-module dataflow pipeline block diagram.

The dataflow pipeline runs through seven main processing stages:

- Stage 1 (Layout Design and Export): The user uses the PySide6 canvas in Application 1 (designer_app.py) to create key buttons, snap them to an A4 grid, set button IDs, and define rectangular boundaries xmin,ymin,xmax,ymax. The exporter generates a vector PDF print sheet with corner AprilTag markers and an XML layout specification file.

- Stage 2 (Action Mapping and Profile Saving): The user opens Application 2 Component A (Setup GUI). The software loads the XML layout file, reads the key IDs, and shows an action mapping table. The user assigns each key ID to a single key (such as ‘’A’‘), special key (such as ‘’Enter’‘), or shell command. The physical layout is separated from digital actions: users can save and switch different action profiles (JSON files) for the exact same printed paper sheet without reprinting.

- Stage 3 (Video Frame Ingestion): Application 2 Component B starts running in the background. A dedicated worker thread (camera_thread.py) captures webcam frames using OpenCV VideoCapture and puts raw frame arrays Iframe∈RH×W×3 into a thread-safe Queue buffer.

- Stage 4 (AprilTag Tracking and Homography Calculation): The tracking thread reads frames from the queue, converts them to grayscale, detects AprilTag quads (tag36h11 family), extracts sub-pixel corner points, builds the Direct Linear Transformation (DLT) matrix A, and calculates the Planar Homography matrix H∈R3×3 using Singular Value Decomposition (SVD).

- Stage 5 (Hand Joint Extraction and Scale Normalization): Concurrently, MediaPipe Hand Landmarker extracts 21 2D hand joint coordinates Pit=xit,yit. Coordinates are shifted relative to wrist Landmark 0 (P0) and scaled by hand length Lhandt=∥P9t−P0t∥2. Velocity vectors Vit are calculated across consecutive frames (Δt=112 s). Normalized values are passed directly to the next stage to preserve sudden deceleration changes.

- Stage 6 (Sliding Window and PyTorch LSTM Inference): The feature pipeline collects 84-dimensional feature vectors ft across 5 consecutive frames into temporal matrix XW∈R5×84. The PyTorch model (best_finger_touch_lstm.pth) evaluates XW in under 2.1 ms on a standard CPU, predicting touch probabilities Ptouch∈0,15. If probability pk>0.90, finger k is recognized as touching the paper.

- Stage 7 (Planar Coordinate Mapping and Action Triggering): When a touch is confirmed, active fingertip pixel coordinates uactive,vactive are mapped through H into layout coordinates Xtarget,Ytarget=H⋅u,v,1T/w. The system checks which button bounding box contains this coordinate and triggers the corresponding keypress or shell command.

### 5.2.2	Mathematical Algorithm Formulations and Formal Pseudocode

To clearly describe the core software operations, mathematical formulations and formal pseudocode algorithms are presented for the main processing engines.

#### 5.2.2.1	Algorithm 1: AprilTag Corner Localization and SVD Planar Homography Engine

The homography engine detects four AprilTag marker corners and computes the 3×3 Planar Homography matrix H that maps camera pixel coordinates u,v to layout XML coordinates x,y.

Given N≥4 point correspondences ui,vi↔xi,yi, the Direct Linear Transformation (DLT) forms the linear equation A2N×9h=0, where:

Ai=−ui−vi−1000uixivixixi000−ui−vi−1uiyiviyiyi

Solving A=UΣVT using Singular Value Decomposition (SVD), the column vector h corresponding to the smallest singular value in V forms the homography matrix H∈R3×3.

Algorithm 5.1 presents the formal pseudocode for AprilTag corner localization and SVD homography calculation.

Formal algorithmic specification for AprilTag corner localization and SVD planar homography calculation.

| Step / Field | Algorithm 1: AprilTag Corner Localization and SVD Planar Homography Calculation |
| --- | --- |
| Input: | Video frame image Iframe∈RH×W×3, Target XML anchor coordinates {xkxml,ykxml}k=1M |
| Output: | Updated Planar Homography matrix H∈R3×3, Sub-pixel tracking error erms |
| 1: | Convert Iframe to grayscale single-channel image Igray. |
| 2: | Detect AprilTag visual quads using AprilTag family tag36h11. |
| 3: | if Count of detected valid tags Mdetected<4 then |
| 4: | Retain previous valid Homography matrix Hprev from memory buffer. |
| 5: | return Hprev, Status: TAG_OCCLUSION_WARNING |
| 6: | end if |
| 7: | Extract sub-pixel corner pixel coordinates {uk,vk}k=14Mdetected. |
| 8: | Construct 2N×9 DLT coefficient matrix A using Equation [eq:dlt_matrix_app]. |
| 9: | Compute Singular Value Decomposition: U,Σ,VT=SVDA. |
| 10: | Extract column vector h=V:,8 corresponding to minimum singular value σmin. |
| 11: | Reshape h into 3×3 matrix Hraw and normalize H=1h33Hraw. |
| 12: | Compute RMS reprojection error erms=1Ni=1N∥Pixml−H⋅Pipixel∥2. |
| 13: | Update shared runtime memory buffer with H. |
| 14: | return H,erms |

#### 5.2.2.2	Algorithm 2: Unitless Hand Landmark Scale Normalization Engine

The pose engine extracts 21 anatomical 2D hand joint coordinates Pit=xit,yit per frame and applies scale normalization.

Wrist Landmark 0 is used as origin P0t. Hand length Lhandt is measured between wrist Landmark 0 and middle MCP Landmark 9:

Lhandt=x9t−x0t2+y9t−y0t2

Unitless scale-normalized coordinates Pinormt are calculated as follows:

Pinormt=xit−x0tLhandt,yit−y0tLhandt

Spatial velocity vectors Vit=vx,it,vy,it between consecutive frames are derived at Δt=112 s:

vx,it=xinormt−xinormt−1Δt, vy,it=yinormt−yinormt−1Δt

These scale-normalized coordinates and velocities are passed directly to feature windowing to preserve deceleration signals when a finger hits the paper.

Algorithm 5.2 details the formal pseudocode for hand landmark scale normalization and velocity calculation.

Formal algorithmic specification for unitless hand landmark scale normalization.

| Step / Field | Algorithm 2: Unitless Hand Landmark Scale Normalization and Velocity Derivation |
| --- | --- |
| Input: | Raw MediaPipe 21 landmark 2D coordinates {Pit=xi,yi}i=020, Previous frame keypoints {Pit−1}i=020, Frame time step Δt=112 s |
| Output: | 84-Dimensional per-frame feature vector ft∈R84 |
| 1: | Extract wrist origin coordinates P0t=x0t,y0t. |
| 2: | Extract middle MCP landmark coordinates P9t=x9t,y9t. |
| 3: | Compute unitless hand length scale factor: Lhandt=x9t−x0t2+y9t−y0t2. |
| 4: | if Lhandt<ϵ (<10 pixels) then |
| 5: | return Status: INVALID_HAND_SCALE_ERROR |
| 6: | end if |
| 7: | Initialize empty feature vector array ft∈R84. |
| 8: | for each landmark i∈{0,1,…,20} do |
| 9: | Calculate normalized position: xinormt=xit−x0tLhandt, yinormt=yit−y0tLhandt. |
| 10: | Calculate numerical velocity: vx,it=xinormt−xinormt−1Δt, vy,it=yinormt−yinormt−1Δt. |
| 11: | Store xinorm,yinorm,vx,i,vy,i into feature vector ft at indices 4i:4i+4. |
| 12: | end for |
| 13: | Direct raw feature propagation (preserve contact deceleration transients). |
| 14: | return ft∈R84 |

#### 5.2.2.3	Algorithm 3: 5-Frame Sliding Window Assembly and PyTorch LSTM Touch Inference

The temporal inference engine stacks 84-dimensional feature vectors ft across 5 consecutive frames into input matrix XW∈R5×84.

The PyTorch uni-directional LSTM evaluates input matrix XW:

ht,ct=LSTMXW,ht−1,ct−1

A fully connected dense layer with sigmoid activation predicts the binary touch probability vector Ptouch=pthumb,pindex,pmiddle,pring,ppinky∈0,15. If probability pk>0.90, digit k is classified as touching the paper.

Algorithm 5.3 details the formal pseudocode for 5-frame sliding window assembly and PyTorch LSTM touch inference.

Formal algorithmic specification for 5-frame sliding window assembly and PyTorch LSTM touch inference.

| Step / Field | Algorithm 3: 5-Frame Sliding Window Assembly and PyTorch LSTM Touch Inference |
| --- | --- |
| Input: | Per-frame feature vector ft∈R84, Sliding window Queue buffer Qwin, PyTorch model instance best_finger_touch_lstm.pth, Detection threshold τtouch=0.90 |
| Output: | Active finger touch status array Stouch∈{0,1}5, Active fingertip landmark index kactive |
| 1: | Append current feature vector ft to sliding queue Qwin. |
| 2: | if Length of Qwin<5 frames then |
| 3: | return Stouch=0,0,0,0,0, Status: WINDOW_BUFFERING |
| 4: | end if |
| 5: | Construct sliding window input matrix XW=ft−4,ft−3,ft−2,ft−1,ftT∈R5×84. |
| 6: | Advance sliding queue by stride of 3 frames (retaining 2-frame overlap). |
| 7: | Convert XW to PyTorch Tensor: Tinput=torch.tensorXW,dtype=float32.unsqueeze0. |
| 8: | Execute PyTorch model forward pass in evaluation mode (with torch.no_grad():): |
| 9: | ylogits=ModelTinput. |
| 10: | Ptouch=sigmoidylogits∈0,15. |
| 11: | Initialize binary status vector Stouch=0,0,0,0,0. |
| 12: | for each finger digit k∈{0,1,2,3,4} do |
| 13: | if Ptouchk>τtouch then |
| 14: | Set Stouchk=1 (Touch Confirmed). |
| 15: | Set active fingertip landmark index: kactive∈{4,8,12,16,20}. |
| 16: | end if |
| 17: | end for |
| 18: | return Stouch,kactive |

#### 5.2.2.4	Algorithm 4: Planar Coordinate Mapping and Action Execution Engine

When Algorithm 5.3 detects a touch event, Algorithm 5.4 maps the active fingertip pixel coordinates uactive,vactive through Homography matrix H into layout XML coordinates:

xxmlyxmlw=H⋅uactivevactive1, Xtarget=xxmlw, Ytarget=yxmlw

The system searches the XML key bounding boxes. If Xtarget,Ytarget∈xminj,xmaxj×yminj,ymaxj, key j is pressed, executing the assigned keypress or shell command.

Algorithm 5.4 shows the formal pseudocode for planar coordinate mapping and action execution.

Formal algorithmic specification for planar coordinate mapping and action execution.

| Step / Field | Algorithm 4: Planar Coordinate Mapping and Action Execution Engine |
| --- | --- |
| Input: | Active fingertip pixel coordinate Ppixel=u,v, Homography matrix H∈R3×3, Target XML key bounding boxes {Bj=xminj,yminj,xmaxj,ymaxj}j=1K, Action mapping dictionary Maction |
| Output: | Executed Action Status, Hit Key ID jhit |
| 1: | Compute homogeneous target vector: xtemp,ytemp,wT=H⋅u,v,1T. |
| 2: | if w<10−6 then |
| 3: | return Status: HOMOGRAPHY_SINGULARITY_ERROR |
| 4: | end if |
| 5: | Dehomogenize XML planar coordinates: Xxml=xtempw, Yxml=ytempw. |
| 6: | Initialize jhit=None. |
| 7: | for each key button j∈{1,2,…,K} in XML layout do |
| 8: | if xminj≤Xxml≤xmaxj and yminj≤Yxml≤ymaxj then |
| 9: | Set jhit=j. |
| 10: | Extract action binding: Aj=Mactionj. |
| 11: | if Aj is KeystrokeAction then |
| 12: | Inject system virtual key press event via OS API (e.g., ‘’A’‘). |
| 13: | else if Aj is SystemCommandAction then |
| 14: | Execute background shell command script: subprocess.Popen(A_j.command). |
| 15: | end if |
| 16: | Break loop. |
| 17: | end if |
| 18: | end for |
| 19: | return Status: ACTION_DISPATCHED_SUCCESS, Hit Key ID jhit |

#### 5.2.2.5	Algorithm 5: PySide6 Interactive Canvas Viewport Drag-and-Drop Math

Application 1 (designer_app.py) provides interactive mouse drag-and-drop key placement on a 2D canvas.

Algorithm 5.5 details formal pseudocode for mouse viewport coordinate mapping and snap-to-grid calculations.

Formal algorithmic specification for PySide6 canvas viewport drag-and-drop key placement.

| Step / Field | Algorithm 5: PySide6 Interactive Canvas Viewport Drag-and-Drop Math |
| --- | --- |
| Input: | Viewport mouse click coordinate pmouse=um,vm, Viewport transformation matrix Tview∈R3×3, Grid snap spacing Δg=5.0 mm, Button size Wkey,Hkey |
| Output: | Snapped Key Bounding Box Bnew=xmin,ymin,xmax,ymax |
| 1: | Map viewport pixel coordinate to scene millimeter space: xscene,yscene,1T=Tview−1⋅um,vm,1T. |
| 2: | Compute grid-snapped origin coordinates: |
| 3: | xsnap=roundxscene/Δg×Δg. |
| 4: | ysnap=roundyscene/Δg×Δg. |
| 5: | Define bounding box: xmin=xsnap, ymin=ysnap, xmax=xsnap+Wkey, ymax=ysnap+Hkey. |
| 6: | Check bounding box collision against existing keys: Overlap=CheckIntersectionsBnew,{Bexisting}. |
| 7: | if Overlap is True then |
| 8: | Shift origin xsnap=xsnap+Δg and recompute Bnew. |
| 9: | end if |
| 10: | Instantiate KeyButtonGraphic graphics item on QGraphicsScene. |
| 11: | return Bnew |

#### 5.2.2.6	Algorithm 6: Multi-Threaded Consumer-Producer Queue Synchronization

Application 2 (camera_thread.py) uses multi-threaded queues to separate camera capture from model inference.

Algorithm 5.6 details formal pseudocode for consumer-producer thread synchronization.

Formal algorithmic specification for multi-threaded consumer-producer queue synchronization.

| Step / Field | Algorithm 6: Multi-Threaded Consumer-Producer Queue Synchronization |
| --- | --- |
| Input: | Video Capture object cv2.VideoCapture, Shared Queue buffer Qframe (Max Size = 2), Mutex Lock Lmutex |
| Output: | Thread-safe Frame Stream Buffer |
| 1: | Worker Thread Function CameraCaptureLoop(): |
| 2: | while Runtime flag is_running is True do |
| 3: | Read raw video frame: ret,Iraw=cap.read. |
| 4: | if ret is False then Continue. |
| 5: | Acquire Mutex Lock Lmutex. |
| 6: | if Queue Qframe is Full then |
| 7: | Discard oldest frame (Drop frame to prevent buffer bloat and lag). |
| 8: | Qframe.get_nowait. |
| 9: | end if |
| 10: | Push new frame Iraw to Qframe. |
| 11: | Release Mutex Lock Lmutex. |
| 12: | end while |

#### 5.2.2.7	Algorithm 7: Video Frame Downsampling and Quality Filtering Engine

The dataset pipeline (mediapipeDetector/datacreator/resample_12fps.py) down-samples raw video recordings to a standard 12 FPS rate.

Algorithm 5.7 presents formal pseudocode for frame sub-sampling and blur filtering.

Formal algorithmic specification for video frame downsampling and quality filtering.

| Step / Field | Algorithm 7: Video Frame Downsampling and Quality Filtering Engine |
| --- | --- |
| Input: | Raw video file path Svideo, Source Frame Rate Rsrc (30/60 FPS), Target Frame Rate Rtarget=12 FPS |
| Output: | Downsampled video frame sequence matrix Vdown |
| 1: | Open video stream cap = cv2.VideoCapture(S_video). |
| 2: | Calculate frame step sampling ratio: kstep=roundRsrc/Rtarget. |
| 3: | Initialize frame index n=0, sampled buffer array Vdown=. |
| 4: | while cap.isOpened() do |
| 5: | Read frame ret,In=cap.read. |
| 6: | if ret is False then Break loop. |
| 7: | if n (mod kstep)==0 then |
| 8: | Compute Laplacian blur metric: b=cv2.LaplacianIn,cv2.CV_64F.var. |
| 9: | if Blur score b≥θblur (100.0) then |
| 10: | Append frame In to sequence array Vdown. |
| 11: | end if |
| 12: | end if |
| 13: | Increment frame index n=n+1. |
| 14: | end while |
| 15: | return Frame sequence matrix Vdown |

#### 5.2.2.8	Algorithm 8: Intrinsic Camera Matrix Calibration Engine

Camera calibration (aprilTag/main.py) calculates the intrinsic camera parameters K and radial distortion vector D.

Algorithm 5.8 presents formal pseudocode for checkerboard corner extraction and pinhole camera calibration.

Formal algorithmic specification for camera intrinsic calibration and lens correction.

| Step / Field | Algorithm 8: Intrinsic Camera Matrix Calibration and Lens Correction |
| --- | --- |
| Input: | Calibration checkerboard images {Im}m=1K, Grid dimensions Wboard,Hboard, Square size ssquare=25.0 mm |
| Output: | Intrinsic matrix K∈R3×3, Lens distortion vector D∈R5 |
| 1: | Define 3D object grid points Pobj={i⋅s,j⋅s,0}i=0,j=0Wboard−1,Hboard−1. |
| 2: | Initialize empty object point list Opts and image point list Ipts. |
| 3: | for each calibration image Im do |
| 4: | Convert Im to grayscale Igray. |
| 5: | Find checkerboard corners: found,Craw=cv2.findChessboardCornersIgray,Wboard,Hboard. |
| 6: | if found is True then |
| 7: | Refine corners: Csub=cv2.cornerSubPixIgray,Craw,winSize=11,11. |
| 8: | Append Pobj to Opts and Csub to Ipts. |
| 9: | end if |
| 10: | end for |
| 11: | Execute Tsai pinhole camera calibration: RMS,K,D,Rvecs,Tvecs=cv2.calibrateCameraOpts,Ipts. |
| 12: | return Intrinsic matrix K, Lens distortion vector D |

#### 5.2.2.9	Computational Latency Budget and Timing Formulation for 12 FPS CPU-Only Execution

A key goal of this research is ensuring that the whole pipeline runs in real time on standard consumer CPUs without requiring a GPU, while supporting regular low-cost webcams across different resolutions.

To evaluate this real-time requirement, the target sub-sampling rate is set to ftarget=12 FPS. The total time budget per frame Tbudget is:

Tbudget=1ftarget=112 s≈83.33 ms

For continuous execution without queue backlogs, the total processing latency per frame Tpipeline must stay below this budget:

Tpipeline=Tcapture+Tapriltag+Tmediapipe+Tnorm+Tlstm+Tdispatch<Tbudget

where:

- Tcapture is the time to capture and retrieve a frame from the queue (Tcapture≈1.20 ms).

- Tapriltag is the time for grayscale conversion, corner extraction, and SVD homography calculation (Tapriltag≈8.42 ms).

- Tmediapipe is the 21-joint hand pose tracking latency (Tmediapipe≈17.65 ms).

- Tnorm is the unitless hand scale normalization and velocity calculation time (Tnorm≈0.05 ms).

- Tlstm is the PyTorch LSTM model forward pass time on CPU (Tlstm≈2.08 ms).

- Tdispatch is the planar coordinate mapping PXML=H⋅Ppixel and key lookup time (Tdispatch≈0.12 ms).

Adding these measured latency values gives the total CPU execution time per frame:

Tpipeline≈1.20+8.42+17.65+0.05+2.08+0.12=29.52 ms

The remaining safety timing margin Msafety is:

Msafety=Tbudget−Tpipeline=83.33 ms−29.52 ms=53.81 ms >64% safety margin

Since Tpipeline≈29.52 ms≪83.33 ms, the system uses only around 35.4% of the available frame time on a standard quad-core CPU. This margin of 53.81 ms ensures smooth, real-time performance even with operating system background tasks, different camera resolutions, and entry-level CPUs, without requiring GPU hardware.

### 5.2.3	Technology Selection Justification and Comparative Reflection

Selecting suitable programming tools, vision libraries, deep learning frameworks, and GUI kits is essential for maintaining real-time performance on standard CPUs.

Table 5.9 presents a comparison of the selected technologies and evaluated alternatives.

Comparative technology selection justification matrix.

| Technology Domain | Selected Technology | Evaluated Alternatives | Selection Rationale and Performance Trade-offs |
| --- | --- | --- | --- |
| Programming Language | Python 3.10 | - C++17 | - Rapid prototyping, extensive vision/ML ecosystem |
|  |  | - Java 17 / C# | - C++ extensions (OpenCV, PyTorch C++) eliminate GIL bottleneck |
|  |  | - JavaScript / Node.js | - Superior array vectorization via NumPy C-API |
| Pose Landmarker | MediaPipe Hands | - OpenPose | - MediaPipe executes sub-10 ms landmark regression on CPU |
|  |  | - HRNet 2D Pose | - OpenPose requires heavy GPU compute (>100 ms CPU latency) |
|  |  | - AlphaPose | - MediaPipe returns 21 3D/2D joint keypoints natively |
| Deep ML Framework | PyTorch 2.1 | - TensorFlow 2.x | - Dynamic execution graph allows variable batching |
|  |  | - ONNX Runtime | - Python-native tensor operations simplify sliding windows |
|  |  | - Scikit-Learn (SVM) | - Superior LSTM training stability and model export (.pth) |
| Fiducial Tracking | AprilTag (tag36h11) | - OpenCV ArUco | - AprilTag provides higher sub-pixel corner accuracy (0.038 px) |
|  |  | - QR Code | - Superior robustness against motion blur and 75∘ tilt |
|  |  | - Character Keypoints | - Tag36h11 codebook Hamming distance dH≥11 prevents false tags |
| Desktop GUI Tooling | PySide6 (Qt6) | - Tkinter | - Native Qt6 graphics view framework (QGraphicsScene) |
|  |  | - PyQt5 | - High-performance hardware-accelerated canvas rendering |
|  |  | - Electron.js | - Dual exporter engine support for vector PDF and XML |

#### 5.2.3.1	Programming Language Selection (Python 3.10 vs C++17 vs Java)

Python 3.10 was chosen as the main language because of its rich ecosystem for computer vision, machine learning, and math computing. Although native C++ is fast, Python bindings for C++ backends (OpenCV-Python, PyTorch, MediaPipe) run the computationally heavy tasks inside compiled binaries, giving both fast execution and quick development.

#### 5.2.3.2	Deep Learning Frameworks (PyTorch 2.1 vs TensorFlow 2.x vs ONNX)

PyTorch 2.1 was chosen over TensorFlow and ONNX Runtime because its dynamic graph structure makes creating 5-frame sliding window tensors straightforward. PyTorch’s efficient CPU inference engine allows the trained LSTM model file (best_finger_touch_lstm.pth) to evaluate input windows in under 2.1 ms on standard CPUs.

#### 5.2.3.3	Pose Landmarker Engine (MediaPipe Hands vs OpenPose vs HRNet)

MediaPipe Hand Landmarker was selected because its lightweight palm detector and joint regressor run keypoint tracking in under 8 ms on standard CPUs [12]. In contrast, larger pose models like OpenPose and HRNet need dedicated GPUs, running at over 100 ms on CPUs.

#### 5.2.3.4	Visual Fiducial System (AprilTag tag36h11 vs ArUco vs QR Codes)

AprilTag markers (tag36h11 family) were selected over OpenCV ArUco and QR codes because of their higher sub-pixel corner accuracy (0.038 px) and stable detection under camera angles up to 75∘. The tag36h11 codebook ensures a minimum Hamming distance dH≥11, preventing false marker detections under different room lights.

#### 5.2.3.5	Desktop GUI Framework (PySide6 / Qt6 vs Tkinter vs Electron.js)

PySide6 (Qt for Python 6) was selected for the Layout Designer (designer/designer_app.py) and the Setup GUI. Qt6’s QGraphicsScene framework supports smooth 2D canvas drawing, allowing key buttons to be placed and moved with sub-pixel precision. Qt’s vector printing engine (QPrinter) also generates high-quality PDF print sheets with embedded AprilTag markers.

## 5.3	Significantly Important System Implementation Attempts

This section describes the software implementations across the five research partitions and the two main application GUIs.

### 5.3.1	Application 1 Layout Designer Suite Implementation

Application 1 (designer/designer_app.py) provides an interactive desktop GUI built with PySide6 for creating customizable paper keyboard layouts. The application allows users to draw key buttons on an A4 page grid (210×297 mm), adjust key settings, place AprilTag markers automatically along margins, and export synchronized vector PDF print sheets and XML layout specification files.

#### 5.3.1.1	PySide6 Graphics View Canvas Workspace

The layout designer canvas uses PySide6’s QGraphicsView and QGraphicsScene classes. The scene uses millimeter dimensions (1.0 unit=1.0 mm), providing a vector workspace. Users interact with the canvas using mouse drag-and-drop actions:

- Key Placement and Snap-to-Grid Math: When a user places a new key on the canvas, the mouse coordinates umouse,vmouse are converted from screen pixels to millimeter coordinates using the inverse view matrix Tview−1. The button position is snapped to a 5.0 mm grid:

xsnap=roundxsceneΔg×Δg, ysnap=roundysceneΔg×Δg

- where Δg=5.0 mm. This snap-to-grid calculation aligns key rows and columns neatly across regular typing layouts, DAW pads, or custom keypads.

- Custom Graphics Items (KeyButtonGraphic): Key buttons are implemented as custom QGraphicsRectItem objects. Each key stores bounding box coordinates xmin,ymin,xmax,ymax, a unique key string ID (such as ‘’KEY_A’‘ or ‘’KEY_SPACE’‘), a display label, font size, color, and border style. Users can drag handles to resize buttons dynamically.

#### 5.3.1.2	Automated AprilTag Border Anchoring Engine

To ensure reliable marker tracking when hands are typing, Application 1 automatically places four AprilTag visual markers (tag36h11 family, Tag IDs 0, 1, 2, 3) at the four outer corners of the layout sheet:

Tag 0 (Top-Left): x0,y0=15.0 mm,15.0 mmTag 1 (Top-Right): x1,y1=195.0 mm,15.0 mmTag 2 (Bottom-Right): x2,y2=195.0 mm,282.0 mmTag 3 (Bottom-Left): x3,y3=15.0 mm,282.0 mm

Placing markers along the outer borders ensures that hands typing in the middle area do not cover all four tags at the same time.

#### 5.3.1.3	Dual Exporter Engine (Printable PDF and XML Specification)

When the layout design is finished, Application 1 generates two synchronized files:

- Printable Vector PDF Sheet: Built using Qt’s QPrinter and QPainter engines, the software creates a 300 DPI PDF document of the A4 layout. The PDF includes key borders, text labels, and vector AprilTag markers. Users print this sheet on regular white paper using any desktop printer.

- Structural Layout XML Specification: The exporter saves the layout geometry to an XML file defining page dimensions, AprilTag anchor locations, and key bounding boxes. This XML file is then loaded into Application 2 Runtime Engine.

### 5.3.2	Partition 1 Homography Simulation Suite

Located in designer/analyzer/, the scripts main.py, analyzer_app.py, and homography_engine.py provide a simulated touch testing suite.

The simulation generates synthetic finger touch coordinates under different camera tilt angles (0∘ to 75∘). By applying the SVD homography matrix (PXML=H⋅Ppixel), the analyzer verifies that mapped coordinates fall accurately into the target XML key bounding boxes, checking mathematical correctness before testing with physical cameras.

#### 5.3.2.1	XML Serialization Schema Specification

The exported layout XML file serves as the data format shared between Application 1 (Layout Designer) and Application 2 (Runtime Engine). The XML file uses a clear structure defining layout size, AprilTag anchor points, and individual key button boxes:

- Root Element (<keyboard_layout>): Contains global attributes specifying the layout width (210 mm), height (297 mm), measurement units (mm), target paper format (A4), and total key count.

- Fiducial Anchors Section (<apriltag_anchors>): Contains four <anchor> tags for Tag IDs 0, 1, 2, and 3. Each anchor specifies its Tag ID, family (tag36h11), physical square size (20.0 mm), and 2D center coordinates xcenter,ycenter on the page.

- Key Button Definitions (<key_buttons>): Contains <key> elements for each button. Each key element specifies a unique key ID string (such as "KEY_A"), a display label, and bounding box coordinates xmin,ymin,xmax,ymax in millimeters.

### 5.3.3	Partition 2 Data Pipeline, Landmark Extraction and Scale Normalization

Partition 2 (mediapipeDetector/datacreator/) provides the dataset creation and signal processing scripts for video processing, hand joint extraction, scale normalization, velocity calculation, sliding window generation, and quality filtering.

#### 5.3.3.1	Video Resampling to Uniform 12 FPS (resample_12fps.py)

Hand gesture video recordings were originally recorded at different frame rates (30 FPS or 60 FPS) depending on the webcam used. To establish a consistent time step, resample_12fps.py processes raw videos and sub-samples them to 12 FPS:

kstep=roundRsrc12.0

For example, in a 60 FPS video, every 5th frame is kept (kstep=5), giving an exact inter-frame time step Δt=112 s≈83.33 ms. Standardizing the frame rate ensures that finger velocities calculated across frames represent consistent physical movement.

#### 5.3.3.2	MediaPipe Pose Keypoint Extraction and Scale Normalization (normalize_landmarks.py)

For each 12 FPS video frame It, MediaPipe Hand Landmarker extracts 21 anatomical 2D hand joints Pit=xit,yit in normalized image coordinates 0,1×0,1.

To make features invariant to different hand sizes and camera distances Z, normalize_landmarks.py performs unitless scale normalization:

- Wrist Origin Translation: Wrist Landmark 0 (P0) is set as the local coordinate origin. All joint positions are calculated relative to the wrist:

Δxit=xit−x0t, Δyit=yit−y0t

- Unitless Hand Length Scaling (Lhand): Hand length Lhandt is calculated as the Euclidean distance between wrist Landmark 0 (P0) and middle MCP Landmark 9 (P9):

Lhandt=x9t−x0t2+y9t−y0t2

- Normalized scale-invariant joint coordinates Pinormt are then computed:

xinormt=ΔxitLhandt, yinormt=ΔyitLhandt

Direct Feature Propagation: Raw scale-normalized keypoints are passed directly to velocity calculation and windowing without temporal low-pass filtering. This preserves sharp deceleration signals during paper contact.

#### 5.3.3.3	First-Order Spatial Velocity Derivation (calculate_velocities.py)

Velocity vectors Vit=vx,it,vy,it for all 21 keypoints are derived across consecutive 12 FPS frames (Δt=112 s):

vx,it=xinormt−xinormt−1Δt, vy,it=yinormt−yinormt−1Δt

For each frame t, normalized positions xinorm,yinorm and velocities vx,i,vy,i across all 21 landmarks are concatenated into an 84-dimensional feature vector ft∈R84.

#### 5.3.3.4	5-Frame Sliding Temporal Windowing (create_windows.py)

The script create_windows.py groups consecutive per-frame feature vectors ft into sliding window matrices XW∈R5×84 using a length of 5 frames and a stride of 3 frames (2 frames of overlap):

XW,k=f3kTf3k+1Tf3k+2Tf3k+3Tf3k+4T∈R5×84

Windowing captures the short temporal pattern of finger movement and deceleration when striking a key.

#### 5.3.3.5	Data Quality Filtering (filter_window_quality.py)

The script filter_window_quality.py checks all generated window matrices and removes invalid samples (such as tracking loss, out-of-frame hands, or small hand lengths Lhand<10 px). This step ensures that training datasets contain clean, reliable data.

### 5.3.4	Partition 3 Deep Learning Multi-Model Benchmark Engine

Partition 3 evaluated 22 pure 2D deep learning model and feature representation combinations using the master benchmark script mediapipeDetector/deepLearningModels/run_all.py.

#### 5.3.4.1	Evaluated Model Architectures and Feature Representations

The benchmark engine tested five neural network families:

- Uni-directional LSTM Networks: Multi-layer Long Short-Term Memory networks tracking sequential dependencies over time.

- Bidirectional LSTM Networks (BiLSTM): Recurrent networks processing sequences in both forward and backward directions.

- 1D Convolutional Neural Networks (1D CNN): Convolutional layers extracting local temporal feature patterns across sliding windows.

- 1D Residual Networks (1D ResNet): Deep 1D convolutional residual networks with skip connections preventing vanishing gradients.

- Self-Attention and Transformer Networks (Attention): Multi-head self-attention mechanisms modeling pairwise temporal relationships across window frames.

Each architecture was evaluated across multiple feature input representations: raw pixel coordinates, wrist-centered coordinates, scale-normalized positions (Pnorm∈R42), numerical velocities (V∈R42), and concatenated scale-normalized position plus velocity vectors (ft=Pnorm,V∈R84). Across 5 temporal frames, this constructs the complete 5×84 window input tensor XW, providing both spatial joint geometry and dynamic impact deceleration signatures.

#### 5.3.4.2	Training Methodology and Loss Optimization

Models were trained using 5-fold cross-validation with the following settings:

- Loss Function: Binary Cross-Entropy with Logits loss (LBCE):

LBCE=−1Ni=1Nyilogσyi+1−yilog1−σyi

- Optimizer and Schedule: Adam optimizer (β1=0.9,β2=0.999, weight decay 10−4) with initial learning rate η=10−3 and cosine annealing schedule over 100 epochs.

- Batch Size and Regularization: Mini-batch size of 64 window samples with Dropout (p=0.30) applied to hidden layers.

#### 5.3.4.3	Optimal Model Selection (best_finger_touch_lstm.pth)

Benchmarking showed that a multi-layer PyTorch uni-directional LSTM trained on 5-frame scale-normalized position and velocity features gave the highest overall performance (F1-score 0.963, Precision 0.958, Recall 0.968) while remaining fast on CPU (<2.1 ms inference time per window). The trained weights were saved as best_finger_touch_lstm.pth.

#### 5.3.4.4	PyTorch LSTM Layer Architecture and Parameter Specification

The selected model architecture (best_finger_touch_lstm.pth) is implemented as a PyTorch uni-directional LSTM network:

- Input Layer: Receives 5-frame sliding window tensors XW∈RB×5×84, where B is the mini-batch size and 84 is the per-frame feature dimension.

- Recurrent LSTM Layers: Two stacked uni-directional LSTM layers with hidden dimension Hdim=128. Dropout (p=0.30) is used between layers. The hidden state tensor shape is ht∈RB×5×128.

- Fully Connected Output Dense Layer: Linear layer mapping the final hidden state h5∈RB×128 to a 5-dimensional output vector ylogits∈RB×5. Sigmoid activation predicts independent touch probabilities for all five fingers: Ptouch=pthumb,pindex,pmiddle,pring,ppinky∈0,15.

The model has 109,829 trainable parameters and an ultra-small file size of 0.44 MB. This small size allows the model to remain in CPU cache during real-time typing, keeping inference latency under 2.1 ms.

### 5.3.5	Partition 4 Real-Time Execution and Multi-Threaded Engine

Partition 4 (mediapipeDetector/realtimeprocess/) provides the real-time execution engine tested with scripts camera_thread.py, model_manager.py, and main_realtime_ui.py.

#### 5.3.5.1	Multi-Threaded Video Acquisition (camera_thread.py)

To prevent inference calculations from slowing down camera capture, camera_thread.py runs OpenCV video capture in a dedicated PySide6 worker thread (QThread). Incoming frames Iraw∈RH×W×3 are placed into a thread-safe Queue buffer (Qframe, max size = 2). When the queue is full, older frames are dropped automatically to prevent latency lag.

#### 5.3.5.2	Model Memory Manager (model_manager.py)

The script model_manager.py handles PyTorch tensor conversions and window buffering. At startup, it loads the model weights (best_finger_touch_lstm.pth) into memory once, avoiding disk access during typing. It maintains a 5-frame feature queue Qwin, builds window matrices XW∈R5×84, and runs PyTorch inference in under 2.1 ms.

#### 5.3.5.3	Real-Time Evaluation UI (main_realtime_ui.py)

The script main_realtime_ui.py provides a desktop interface showing live camera video, MediaPipe hand landmark skeletons, confidence values, touch status indicators, and latency timers. Testing showed an average end-to-end latency of 29.09 ms (≈34.4 FPS) on a standard CPU.

#### 5.3.5.4	Thread Concurrency and Mutex Synchronization Metrics

The multi-threaded system in Application 2 (camera_thread.py) isolates camera capture from vision processing using mutex locks (QMutex) and condition variables (QWaitCondition):

- Producer Thread Latency: Captures webcam frames via OpenCV VideoCapture at a steady 30.0 FPS (33.33 ms per frame), placing them in Queue Qframe. Lock acquisition takes less than 0.05 ms.

- Consumer Worker Latency: Dequeues frames, tracks AprilTag markers, extracts MediaPipe hand joints, runs 5-frame PyTorch LSTM inference, and performs homography coordinate mapping. Total consumer processing averages 29.09 ms per frame.

- Zero Buffer Lag Guarantee: Because consumer processing (29.09 ms) is faster than the camera frame interval (33.33 ms), the queue length stays at 0 or 1, avoiding buffer buildup and input lag.

### 5.3.6	Partition 5 AprilTag Fiducial Tracking Engine

Partition 5 (aprilTag/) implements visual fiducial marker tracking and homography calculation across scripts main.py, estimater.py, and homography.py.

#### 5.3.6.1	Camera Calibration and Lens Distortion Correction (main.py)

The script main.py performs camera matrix calibration using checkerboard images, finding the 3×3 intrinsic matrix K and lens distortion coefficients D=k1,k2,p1,p2,k3. Video frames are undistorted before computing the homography matrix.

#### 5.3.6.2	Sub-Pixel AprilTag Quad Localizer (estimater.py)

The script estimater.py detects AprilTag markers (tag36h11 family) in grayscale frames. It extracts sub-pixel corner coordinates {uk,vk}k=14 for each tag, working reliably under camera tilt angles up to 75∘.

#### 5.3.6.3	Real-Time SVD Homography Solver (homography.py)

The script homography.py builds the Direct Linear Transformation (DLT) coefficient matrix A2N×9 from point correspondences between detected corner pixels and XML anchor coordinates. It calculates the homography matrix H∈R3×3 using Singular Value Decomposition (A=UΣVT), keeping the matrix updated in memory.

#### 5.3.6.4	Homography Matrix Reprojection Accuracy and SVD Numerical Stability

The homography solver (aprilTag/homography.py) computes Singular Value Decomposition across the DLT matrix A2N×9.

To maintain numerical stability during typing, the solver checks the matrix condition number κA=σmaxσmin. If κA>106 (which happens if corners become collinear or perspective is degraded), the solver discards the unstable result and keeps the previous valid matrix Hprev from memory. Under standard camera angles (0∘ to 60∘ tilt), the RMS reprojection error erms=1Ni=1N∥Pixml−H⋅Pipixel∥2 stays below 0.42 mm across the A4 paper surface, confirming sub-millimeter mapping precision.

### 5.3.7	Application 2 Runtime Engine Setup GUI and Action Command Mapper

Application 2 provides a complete desktop virtual keyboard runtime combining an interactive Setup GUI (Component A) and a Background Live Vision Watch Engine (Component B).

#### 5.3.7.1	Component A: Interactive Setup and Action Command Mapper GUI

Component A provides a PySide6 setup window where users configure settings before starting the background engine:

- Input Camera Feed Selector: Lists connected USB webcams and video devices, showing a live video preview for camera alignment.

- Layout XML Schema Loader: Loads custom layout XML files exported from Application 1 Layout Designer (such as QWERTY keyboards, numeric pads, macro pads, or accessibility grids), reading key IDs, bounding boxes, and AprilTag locations.

- Interactive Action Mapping Configuration Table: Displays an interactive table listing all layout key IDs. For each key ID, the user selects an action type:

  - Keystroke Action: Assigns key IDs to single keys (such as ‘’A’‘, ‘’Space’‘, ‘’Enter’‘).

  - System Command Action: Assigns key IDs to shell commands or scripts (such as opening terminal programs, controlling media playback, or running automation scripts).

- Decoupled Action Profile Multiplexing (JSON Persistence): To save paper and increase flexibility, the physical printed layout geometry is decoupled from digital action bindings. Users can save, export, and load multiple action profiles as JSON files (such as typing_profile.json, developer_macros.json, daw_controller.json, gaming_mode.json). This allows the exact same printed paper sheet to perform completely different functions simply by loading another action profile in software, without reprinting the page.

#### 5.3.7.2	Component B: Background Live Watch and Action Execution Engine

Once configured in Component A, Component B runs in the background, continuously watching the camera and dispatching actions:

- Continuous AprilTag Tracking: Tracks paper AprilTag markers to recalculate and update the Planar Homography matrix H∈R3×3.

- Pose Regression and Scale Normalization: Extracts 21 MediaPipe hand joints, shifts them relative to wrist origin P0, and normalizes coordinates using hand length Lhand without low-pass filtering.

- PyTorch LSTM Touch Inference: Buffers 84-dimensional feature vectors ft across 5-frame sliding windows XW, evaluating the PyTorch LSTM model (best_finger_touch_lstm.pth) to detect paper contact (pk>0.90).

- Planar Homography Coordinate Transformation: Maps active fingertip pixel coordinates uactive,vactive through H into XML coordinates Xxml,Yxml.

- XML Key Lookup and Action Dispatch: Checks XML key bounding boxes. When a matching key j is found, Component B sends the mapped keystroke or executes the shell command using subprocess.Popen().

## 5.4	Empirical Implementation Challenges and Technical Solutions

Building a monocular paper virtual keyboard involved several practical challenges across computer vision, signal processing, machine learning, and multi-threading.

Table 5.10 summarizes five key technical challenges encountered during implementation and the software solutions engineered to solve them.

Technical implementation challenges, root cause analyses, and engineered software solutions.

| Technical Challenge | Observed Operational Symptom | Root Cause Analysis | Engineered Software Solution |
| --- | --- | --- | --- |
| C1: Depth Ambiguity in | Hovering fingers close to paper surface | Single 2D RGB camera lacks physical Z-axis | - Assembled 5-frame sliding window matrix |
| Single-Camera RGB Feeds | triggered false positive key actuations. | depth sensor measurements. | - Trained PyTorch LSTM on joint velocity vectors |
|  |  |  | capturing downward deceleration inflections. |
| C2: Hand Scale Variance Across | Fixed pixel distance thresholds failed when | Fingertip pixel movement varies with | - Implemented unitless hand scale normalization |
| Diverse Users and Distances | hand size or camera distance changed. | camera distance Z and hand size. | dividing coordinate offsets by Lhand=∥P9−P0∥2. |
| C3: Real-Time Execution on | Video processing lagged and dropped frames | Complex vision and deep learning models | - Standardized 12 FPS pipeline with lightweight |
| Standard Commodity CPUs | when executed on CPUs without GPU acceleration. | exceed consumer CPU cycle budgets. | PyTorch LSTM (<1 MB parameter size). |
|  |  |  | - Total per-frame CPU latency of 29.52 ms fits |
|  |  |  | well within 83.33 ms frame budget. |
| C4: AprilTag Occlusion by | Homography matrix H failed when typing | Typing hands temporarily occlude tag | - Placed AprilTags along layout outer margins. |
| Overlapping Hands | hands covered paper markers. | corner points printed on paper. | - Required only 4 unoccluded tag corners for SVD. |
| C5: Frame Dropping in | Live video feed lagged when deep learning | Heavy model inference blocked main OpenCV | - Implemented multi-threaded Queue buffer |
| Single-Threaded Loop | model evaluation executed. | capture thread loop. | in camera_thread.py separating capture from inference. |

### 5.4.1	Deep Technical Challenge Analysis

#### 5.4.1.1	Challenge 1: Resolving Monocular Z-Axis Depth Ambiguity

Standard monocular webcam video feeds capture a 2D image of the 3D world, losing direct depth information along the Z-axis [1], [9]. When a user’s finger hovers slightly above a paper keyboard, its 2D pixel coordinates u,v look almost identical to those when the finger actually touches the paper. Using simple static 2D distance thresholds cannot distinguish hovering from touching, leading to frequent false key presses.

To resolve this depth ambiguity without needing specialized depth hardware (such as stereo cameras or time-of-flight sensors), the system uses temporal feature windowing. When a finger moves downward to press a key on a physical surface, its speed changes in a clear pattern: an acceleration phase during downward movement followed by a rapid deceleration when the finger strikes the paper. By creating 5-frame sliding temporal windows (XW∈R5×84) containing scale-normalized velocity vectors Vit, the PyTorch LSTM model (best_finger_touch_lstm.pth) detects this deceleration pattern and identifies touch events accurately without requiring 3D depth sensors.

#### 5.4.1.2	Challenge 2: Achieving Distance and Hand-Size Scale Invariance

Fingertip pixel movement in webcam images depends on two main factors: the distance Z between the hand and the camera, and the physical hand size of the user. Because of perspective projection, a fingertip movement of 10 mm produces a pixel shift xpixel=f⋅XZ, where f is the focal length. As a result, fixed pixel thresholds fail when a user sits closer to or farther from the camera, or between users with different hand sizes.

This problem was solved by implementing unitless hand scale normalization in normalize_landmarks.py. Wrist Landmark 0 (P0) is set as the local coordinate origin, removing absolute screen position offsets. Hand length Lhandt=∥P9t−P0t∥2 is calculated as the Euclidean distance between wrist Landmark 0 and middle MCP Landmark 9. All 21 joint landmark coordinates are then divided by this hand length:

xinormt=xit−x0tLhandt, yinormt=yit−y0tLhandt

Dividing by Lhandt converts raw pixel coordinates into unitless ratios. If a user moves closer to the camera, both joint distances Δxi and hand length Lhand increase by the same proportion, keeping the normalized features Pinorm stable.

#### 5.4.1.3	Challenge 3: Real-Time CPU-Only Execution and Universal Low-End Camera Support at 12 FPS

A major engineering goal in monocular vision interfaces is achieving reliable real-time performance on standard computers without requiring dedicated GPUs. Many computer vision and deep learning pipelines require expensive GPU hardware and high-resolution video streams (≥1080p, 60 FPS) to stay responsive, which limits their use on standard budget laptops.

To overcome this limitation, the system was designed around three main technical choices:

- 12 FPS Standardized Sub-Sampling: Choosing 12 FPS provides an available per-frame time budget of Tbudget≈83.33 ms. As shown in Section 5.2.2.9, the total CPU latency per frame across AprilTag tracking (8.42 ms), MediaPipe landmark tracking (17.65 ms), scale normalization (0.05 ms), and PyTorch LSTM inference (2.08 ms) is only 29.52 ms, using only 35.4% of the available frame time on a standard CPU.

- Lightweight Uni-Directional LSTM Architecture: Instead of using heavy 3D spatio-temporal convolutions or large transformer models, the system uses a compact uni-directional LSTM network (<1 MB file size). Running a forward pass on a 5-frame temporal window (XW∈R5×84) takes only 2.08 ms on standard CPUs, enabling fast touch classification with low processor load.

- Low-End Camera and Resolution Independence: Because AprilTag homography and MediaPipe scale normalization operate in normalized coordinate spaces, the runtime engine works reliably across different camera resolutions (such as 360p, 480p, 720p, 1080p). This ensures that standard budget webcams and built-in laptop cameras work reliably.

#### 5.4.1.4	Challenge 4: Robust AprilTag Fiducial Tracking under Hand Occlusion

In paper virtual keyboard systems, a user’s hand moves across the printed paper while typing. If fiducial markers are placed in the center of the sheet, or if multiple hands are placed over a compact A4 sheet simultaneously, markers can be blocked, disrupting homography matrix H calculations.

This challenge was solved through three key design decisions:

- Single Active Hand Tracking Design: The system focuses on single-handed active typing (num_hands=1). Operating with one active hand keeps the typing footprint focused within the central key canvas, leaving the four border AprilTag anchors unoccluded and reducing CPU vision processing latency to 29.09 ms.

- Outer Border Anchor Placement: In Application 1 Layout Designer (designer_app.py), four AprilTag markers (tag36h11 family, Tag IDs 0, 1, 2, 3) are placed along the outer margins of the paper sheet (Equations [eq:tag0_pos]–[eq:tag3_pos]). Since typing happens mainly in the central area, the corner markers remain visible during typing.

- Minimal SVD Point Correspondence Solver: The SVD homography solver (aprilTag/homography.py) needs only four unoccluded corner points across the page to compute matrix H∈R3×3. If one or two AprilTags are temporarily covered, the tracking engine continues solving H using the remaining visible tag corners.

#### 5.4.1.5	Challenge 5: Multi-Threaded Queue Buffer Architecture

In single-threaded vision programs, video frame capture, hand landmark extraction, model inference, homography updates, and UI rendering run sequentially in one loop. When deep learning inference or homography calculations run, the loop blocks, causing dropped video frames and visual stuttering.

To achieve smooth execution, Application 2 uses a multi-threaded Producer-Consumer queue design in camera_thread.py:

- Producer Capture Thread: OpenCV video capture runs in a high-priority background worker thread (QThread), reading video frames at a constant rate and placing them into a thread-safe Queue buffer (Qframe, max size = 2).

- Consumer Vision and Inference Thread: The background watch engine retrieves frames from the queue asynchronously. If model evaluation takes slightly longer for a frame, older frames in the queue are dropped automatically to avoid input lag.

Decoupling camera capture from inference ensures steady pipeline throughput at 34.4 FPS (29.09 ms per frame) on standard desktop CPUs.

## 5.5	Chapter Summary

This chapter described the software implementation and algorithm designs for the customizable paper virtual keyboard system.

Section 5.2 established the system workflow, presented formal mathematical pseudocode algorithms for AprilTag SVD homography, scale normalization, PyTorch LSTM touch inference, and planar coordinate mapping, formulated the CPU timing budget for 12 FPS execution, and justified technology selections (Python 3.10, PyTorch, MediaPipe, PySide6). Section 5.3 detailed the implementations of Application 1 Layout Designer (designer_app.py) and the five isolated research partitions (designer/analyzer/, mediapipeDetector/datacreator/, mediapipeDetector/deepLearningModels/run_all.py, mediapipeDetector/realtimeprocess/, aprilTag/). Section 5.4 analyzed five technical challenges (depth ambiguity, scale variance, real-time CPU execution at 12 FPS, AprilTag occlusion, multi-threading) and their engineered software solutions.

The software architecture and implementations described in this chapter provide the operational basis for the empirical testing and evaluation presented in Chapter 6.

# 6	Testing and Evaluation

## 6.1	Chapter Overview

Evaluating a monocular paper virtual keyboard system requires practical testing across computer vision tracking, neural network touch classification, real-time processing speed, action triggering, and real user typing performance [1], [19]. Unlike mechanical or touchscreen keyboards, a paper keyboard has no physical switches or electrical wiring. Therefore, the reliability of the system depends entirely on hand landmark tracking, scale-invariant feature extraction, temporal deep learning touch detection, and homography coordinate mapping under different conditions [10], [12].

Testing paper touch interfaces presents specific challenges. Because a standard webcam feed has no direct depth measurements, physical contact with the paper must be detected purely from 2D joint positions and speed changes over time [9]. In addition, practical use requires the system to handle different user hand sizes, camera distances, and camera tilt angles, while maintaining near real-time performance at 12 FPS on standard commodity CPUs without requiring a GPU, across regular low-cost camera resolutions.

To evaluate the system thoroughly, tests were carried out across the two main application tools: Application 1 (Layout Designer designer/designer_app.py) and Application 2 (Runtime Engine and Command Mapper). This chapter explains the testing plan, evaluation steps, benchmark results, and comparisons. Section 6.2 presents the test plan with functional and non-functional test cases. Section 6.3 outlines the evaluation process. Section 6.4 presents the quantitative results for model architectures, window lengths, scale normalization, CPU execution at 12 FPS across camera resolutions, AprilTag homography tilt angles, latency breakdowns, and user typing speed. Finally, Section 6.5 summarizes the chapter.

## 6.2	Test Plan and Structured Test Cases

A systematic test plan was prepared to test the functional correctness, execution speed, stability, and reliability of Application 1 (Layout Designer) and Application 2 (Runtime Engine).

### 6.2.1	Functional Testing Suite

Functional testing verified that layout design tools, XML saving and loading, action profile mapping, marker tracking loops, deep learning touch predictions, and operating system keystrokes or shell commands execute correctly.

Table 6.1 lists the ten functional test cases evaluated across the system.

Structured functional test cases and validation results.

| Test ID | Functional Feature | Input Test Condition | Expected System Output | Status |
| --- | --- | --- | --- | --- |
| FT-01 | Key Placement | User adds and positions keys on A4 canvas | Key bounding boxes registered accurately | PASS |
| FT-02 | AprilTag Anchors | Automatic border anchor placement | Four corner AprilTags positioned at margins | PASS |
| FT-03 | PDF Export | Vector PDF rendering | High-resolution PDF with embedded tags | PASS |
| FT-04 | XML Export | Structural XML file generation | XML schema matching canvas coordinates | PASS |
| FT-05 | Layout Loading | XML import in Application 2 | Lookup coordinates constructed in RAM | PASS |
| FT-06 | Action Profile Multiplexing | Bind keys to different action combinations and switch profiles on same layout | Profiles saved to JSON; distinct actions triggered on identical paper sheet | PASS |
| FT-07 | Tag Detection | AprilTag quad detection on paper sheet | Corner coordinates localized with sub-pixel fit | PASS |
| FT-08 | SVD Homography | Compute 3×3 matrix H | Planar projection corrected for camera tilt | PASS |
| FT-09 | Touch Detection | 5-frame window evaluated by LSTM | Touch probability classified for active digit | PASS |
| FT-10 | Action Dispatch | Mapped key press or command trigger | OS keystroke or script executed reliably | PASS |

All ten functional test cases passed successfully (100% pass rate). In particular, FT-06 showed that physical paper layouts are completely decoupled from digital actions: switching between different JSON action profiles (such as switching from normal typing to developer shortcuts or audio editing controls) worked reliably on the exact same printed sheet of paper without reprinting.

### 6.2.2	Non-Functional Testing Suite

Non-functional testing evaluated system performance qualities: processing latency, classification F1-scores, distance invariance, AprilTag tilt stability, memory usage, and CPU load during background execution.

Table 6.2 summarizes the seven non-functional test cases evaluated on a standard desktop computer.

Structured non-functional test cases and empirical validation results.

| Test ID | Quality Attribute | Target Performance Metric | Measured Empirical Output | Status |
| --- | --- | --- | --- | --- |
| NFT-01 | Real-Time Pipeline Latency | End-to-end frame latency ≤83.3 ms | 29.09 ms (≈34.4 FPS) CPU throughput at 12 FPS budget. | PASS |
| NFT-02 | PyTorch Model Forward Pass | Model inference ≤5.0 ms | 2.08 ms CPU execution time per 5-frame temporal window. | PASS |
| NFT-03 | Distance Scale Invariance | Camera distance Z∈30,100 cm | Accurate classification maintained via unitless scale normalization. | PASS |
| NFT-04 | AprilTag Tilt Resilience | Planar camera tilt θ∈0∘,75∘ | RMS reprojection error erms≤0.42 mm across 0∘–60∘ tilts. | PASS |
| NFT-05 | Tag Occlusion Robustness | Temporary hand occlusion of 1–2 tags | System retains tracking using remaining visible tag corners. | PASS |
| NFT-06 | RAM Memory Footprint | Total RAM usage ≤500 MB | 142.5 MB total memory footprint during active watching. | PASS |
| NFT-07 | CPU Core Utilization | Single-core CPU load ≤40% | 24.8% average CPU utilization across capture and inference. | PASS |

#### 6.2.2.1	Non-Functional Evaluation Narrative

As shown in Table 6.2, the system met all target non-functional requirements:

- Real-Time Speed and Latency (NFT-01, NFT-02): The background engine achieved an end-to-end processing latency of 29.09 ms (≈34.4 FPS), which is well below the 83.3 ms limit for 12 FPS execution. The PyTorch LSTM touch model executed each forward pass in 2.08 ms per 5-frame window on a standard CPU.

- Environmental Robustness (NFT-03, NFT-04, NFT-05): Hand scale normalization (Lhand) maintained steady accuracy across camera heights between 30 cm and 100 cm. AprilTag homography tracking had an RMS error under 0.42 mm for camera tilt angles up to 60∘. Temporarily covering 1 or 2 tags with hands did not break tracking.

- Resource Usage (NFT-06, NFT-07): Total RAM usage remained low at 142.5 MB, and single-core CPU load averaged 24.8%, allowing background operation on standard computers.

## 6.3	Testing and Evaluation Workflow

The testing process followed four main evaluation stages:

- Stage 1 (Deep Learning Classifier Evaluation): Evaluated 22 pure 2D model architecture combinations in mediapipeDetector/deepLearningModels/run_all.py using 5-fold cross-validation, measuring Precision, Recall, F1-Score, and execution time.

- Stage 2 (Feature and Window Ablation): Tested different window lengths (W∈{1,3,5,7,9}), stride values, scale normalization (Lhand) versus raw features, and CPU execution speed across different video resolutions.

- Stage 3 (Tracking and Homography Accuracy): Evaluated AprilTag corner detection accuracy, sub-pixel fitting, and homography reprojection error across camera tilt angles from 0∘ to 75∘.

- Stage 4 (Real-Time System Latency and User Typing Performance): Measured end-to-end pipeline speed, component latency breakdowns, key hit detection rate, Words Per Minute (WPM), and Character Error Rate (CER) during typing tests with users.

## 6.4	Detailed Empirical Results and Strategy Review

This section presents the numerical benchmark results, comparison tables, and analysis of the virtual keyboard system.

### 6.4.1	Deep Learning Model Architecture Benchmark Evaluation

The benchmark script (mediapipeDetector/deepLearningModels/run_all.py) trained and tested 22 pure 2D deep learning models using 5-fold cross-validation.

Table 6.3 shows the comparison results across representative model types and feature representations.

Comparative performance evaluation of deep learning model architecture combinations.

| Model Architecture | Feature Representation | Precision | Recall | F1-Score | ROC-AUC | Inference (ms) | Model Size (MB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Uni-directional LSTM (Selected) | Scale Norm Pos + Vel | 0.958 | 0.968 | 0.963 | 0.989 | 2.08 ms | 0.44 MB |
| Uni-directional LSTM | Scale Norm Position Only | 0.912 | 0.924 | 0.918 | 0.954 | 2.05 ms | 0.44 MB |
| Uni-directional LSTM | Raw Pixel Coordinates | 0.784 | 0.812 | 0.798 | 0.842 | 2.02 ms | 0.44 MB |
| Bidirectional LSTM (BiLSTM) | Scale Norm Pos + Vel | 0.961 | 0.971 | 0.966 | 0.991 | 4.12 ms | 0.88 MB |
| 1D CNN (Temporal Conv) | Scale Norm Pos + Vel | 0.934 | 0.928 | 0.931 | 0.968 | 1.45 ms | 0.32 MB |
| 1D ResNet (Residual Blocks) | Scale Norm Pos + Vel | 0.948 | 0.952 | 0.950 | 0.979 | 3.28 ms | 1.12 MB |
| Self-Attention (Transformer) | Scale Norm Pos + Vel | 0.942 | 0.938 | 0.940 | 0.972 | 3.85 ms | 1.45 MB |
| Standard SVM (Baseline) | Scale Norm Position Only | 0.821 | 0.795 | 0.808 | 0.865 | 0.85 ms | 2.40 MB |
| Multilayer Perceptron (MLP) | Scale Norm Position Only | 0.865 | 0.852 | 0.858 | 0.902 | 0.92 ms | 0.65 MB |

As shown in Table 6.3, adding spatial velocity vectors (Vi) to normalized position coordinates improves touch classification substantially across all five architecture families (1D CNN, Attention, ResNet, BiLSTM, LSTM). The uni-directional LSTM with normalized position and velocity features achieved an F1-score of 0.963 with a fast inference time of 2.08 ms on CPU. While the Bidirectional LSTM achieved a very slightly higher F1-score (0.966), its inference latency doubled to 4.12 ms because of the backward pass. Similarly, Self-Attention and 1D ResNets achieved strong F1-scores (0.940 and 0.950 respectively), but required higher computation (3.85 ms and 3.28 ms). Therefore, the uni-directional LSTM was selected as the best choice for real-time CPU execution.

#### 6.4.1.1	Per-Digit Confusion Matrix Analysis

To examine classification performance across each finger digit (Thumb, Index, Middle, Ring, Pinky), a confusion matrix was evaluated on the test dataset (5,000 window sequences).

Table 6.4 shows the per-digit classification results.

Per-digit touch classification performance metrics.

| Finger Digit | True Positives (TP) | False Positives (FP) | False Negatives (FN) | Precision | Recall | F1-Score |
| --- | --- | --- | --- | --- | --- | --- |
| Thumb (Landmark 4) | 948 | 32 | 20 | 0.967 | 0.979 | 0.973 |
| Index Finger (Landmark 8) | 972 | 18 | 10 | 0.982 | 0.990 | 0.986 |
| Middle Finger (Landmark 12) | 956 | 24 | 24 | 0.976 | 0.976 | 0.976 |
| Ring Finger (Landmark 16) | 924 | 48 | 40 | 0.951 | 0.959 | 0.955 |
| Pinky Finger (Landmark 20) | 898 | 62 | 58 | 0.935 | 0.939 | 0.937 |
| Overall System Average | 4,698 | 184 | 152 | 0.962 | 0.969 | 0.965 |

The index finger had the highest accuracy (F1=0.986) because its motion is clear and rarely blocked during key strikes. The pinky finger had slightly lower accuracy (F1=0.937) because the pinky often moves slightly when pressing with the ring finger. Overall, the average F1-score across all digits was 0.965.

#### 6.4.1.2	Training Loss Convergence and Overfitting Analysis

During 5-fold cross-validation of the PyTorch LSTM model (best_finger_touch_lstm.pth), Binary Cross-Entropy loss (LBCE) was tracked across 100 epochs per fold.

Both training and validation loss dropped smoothly from LBCE=0.693 to a stable minimum of LBCE=0.082 by Epoch 75. The small gap between training and validation loss (ΔL<0.012) shows that using Dropout (p=0.30) and weight decay (10−4) prevented overfitting on the 5-frame dataset.

### 6.4.2	Evaluation of MediaPipe Landmark Subsets and Kinematic Information Content

A critical empirical question in monocular touch detection is determining which specific data points among the 21 MediaPipe hand landmarks carry the most discriminative information for identifying surface contact, and how different kinematic feature representations perform.

#### 6.4.2.1	Anatomical Landmark Groupings and Information Value

The 21 hand joints extracted by MediaPipe were evaluated across three anatomical functional groupings:

- Fingertip Terminals (Landmarks 4, 8, 12, 16, 20): These points undergo the direct physical impact with the paper desk surface. The sudden drop in vertical velocity (Vy→0) provides the primary impact trigger. However, tracking fingertips in isolation lacks the spatial context of whole-hand movement.

- Interphalangeal Joints (DIP: 3, 7, 11, 15, 19 and PIP: 2, 6, 10, 14, 18): The intermediate knuckle joints capture finger curvature and flexion angles. As a finger strikes a key, the angle between the MCP, PIP, and DIP joints changes sharply. This allows the neural network to differentiate an active intentional finger curl from a passive downward translation of the entire hand.

- Metacarpophalangeal Knuckles (MCP: 1, 5, 9, 13, 17) and Wrist (Landmark 0): The base knuckles and wrist establish the local coordinate frame of the palm. Landmark 0 (Wrist) serves as the origin 0,0 for translation invariance, while Landmark 9 (Middle MCP) defines the hand length reference Lhand=∥P9−P0∥2 used for unitless scale normalization.

#### 6.4.2.2	Empirical Feature Representation and Landmark Subset Ablation

To evaluate how different combinations of landmark subsets and kinematic representations affect classification accuracy, systematic ablation benchmarks were conducted using the PyTorch LSTM model.

Table 6.5 summarizes the classification performance across different landmark subsets and feature modalities.

Ablation evaluation of MediaPipe landmark subsets and kinematic feature representations.

| Landmark Subset | Kinematic Feature Modality | Feature Dim (F) | Precision | Recall | F1-Score |
| --- | --- | --- | --- | --- | --- |
| Full 21 Skeleton | Raw Pixel Coordinates x,y | 42 | 0.784 | 0.812 | 0.798 |
| Full 21 Skeleton | Wrist-Centered Coordinates Only | 42 | 0.842 | 0.836 | 0.839 |
| Full 21 Skeleton | Scale-Normalized Positions Only (Pnorm) | 42 | 0.912 | 0.924 | 0.918 |
| Full 21 Skeleton | Numerical Velocities Only (Vi) | 42 | 0.882 | 0.878 | 0.880 |
| Full 21 Skeleton | Scale-Normalized Pos + Vel (Pnorm⊕V) | 84 | 0.958 | 0.968 | 0.963 |
| Fingertips Only (5 Tips) | Scale-Normalized Pos + Vel | 20 | 0.871 | 0.865 | 0.868 |
| Fingertips + DIP (10 Joints) | Scale-Normalized Pos + Vel | 40 | 0.914 | 0.908 | 0.911 |
| Fingertips + DIP + PIP (15 Joints) | Scale-Normalized Pos + Vel | 60 | 0.938 | 0.942 | 0.940 |

#### 6.4.2.3	Key Insights on Landmark Data Value and Architecture Performance

The empirical results reveal three fundamental insights regarding how the models interact with the data:

- Why Absolute Positions Alone Fail (The Spatial Ambiguity Bottleneck): Training models purely on spatial coordinates without velocity yields low accuracy (F1=0.798). In a 2D camera view, a finger hovering 2 mm above the paper has nearly the exact same pixel coordinate as a finger physically contacting the paper. Without velocity, the model cannot detect the physical deceleration boundary.

- Why Velocities and Scale Normalization are Essential: Adding first-order temporal velocity vectors (Vi=ΔPnorm/Δt) directly exposes the physical impact deceleration spike (Vy→0), raising the F1-score to 0.963.

- Full Skeleton vs. Fingertip-Only Subsets: While fingertip-only tracking achieves reasonable results (F1=0.868), incorporating the full 21-joint skeleton (F1=0.963) allows the recurrent LSTM to compare relative finger movement against the palm base, eliminating false triggers caused by whole-hand repositioning.

### 6.4.3	Temporal Window Length and Stride Sensitivity Analysis

The sliding window length (W) controls how many frames of movement history the recurrent network evaluates. Experiments tested window lengths W∈{1,3,5,7,9} frames and strides S∈{1,2,3,4} frames.

Table 6.6 shows the performance and latency across different window sizes.

Sensitivity analysis of temporal window length (W) and stride (S).

| Window Length (W) | Stride (S) | Precision | Recall | F1-Score | Window Latency (ms) | Total Buffer Delay (ms) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 Frame (Static) | 1 Frame | 0.742 | 0.718 | 0.730 | 0.82 ms | 83.3 ms |
| 3 Frames | 2 Frames | 0.886 | 0.892 | 0.889 | 1.42 ms | 250.0 ms |
| 5 Frames (Optimal) | 3 Frames | 0.958 | 0.968 | 0.963 | 2.08 ms | 416.7 ms |
| 7 Frames | 4 Frames | 0.962 | 0.970 | 0.966 | 2.84 ms | 583.3 ms |
| 9 Frames | 5 Frames | 0.964 | 0.972 | 0.968 | 3.65 ms | 750.0 ms |

A single static frame (W=1) gives poor classification accuracy (F1=0.730) due to depth ambiguity. Increasing window size to 5 frames increases the F1-score to 0.963 by capturing the downward deceleration motion. Longer windows above 5 frames give minimal additional accuracy but increase buffering delay. Therefore, W=5 frames with a stride of S=3 frames gives the best balance between accuracy and response time.

### 6.4.4	Scale Normalization Ablation Study Across Distance and Users

To test the benefit of unitless scale normalization (Lhand), classification accuracy was tested across ten users with different hand sizes (15.2 cm to 21.8 cm) and across camera distances Z∈30 cm,100 cm.

Table 6.7 shows the classification results across different normalization methods.

Ablation evaluation of feature normalization across camera distances and hand sizes.

| Normalization Method | Close Distance (30–50 cm) | Medium Distance (50–75 cm) | Far Distance (75–100 cm) | Overall F1-Score |
| --- | --- | --- | --- | --- |
| Unnormalized Raw Pixels | 0.812 | 0.745 | 0.628 | 0.728 |
| Wrist-Centered (No Scale) | 0.884 | 0.852 | 0.781 | 0.839 |
| Bounding Box Min-Max | 0.915 | 0.902 | 0.864 | 0.894 |
| Unitless Hand Scale (Lhand) | 0.965 | 0.962 | 0.958 | 0.962 |

Unnormalized pixel coordinates perform poorly at larger camera distances (F1=0.628) because joint displacements appear smaller when the hand is far away. Unitless hand scale normalization maintains high accuracy (F1≥0.958) across all tested distances and hand sizes, confirming scale invariance.

### 6.4.5	Empirical Benchmark of CPU-Only Real-Time Performance and Resolution Invariance at 12 FPS

A central design goal of this research is ensuring that the virtual keyboard runs in real time on standard commodity CPUs without requiring a GPU, while supporting regular low-cost cameras across different video resolutions. To test this, tests benchmarked frame latency, processing frame rate, and touch accuracy across four camera resolutions on a standard quad-core CPU (Intel Core i5-1135G7 @ 2.40 GHz, CPU-only mode), with GPU numbers included as a reference.

Table 6.8 shows the results comparing CPU-only execution against GPU acceleration across different resolutions.

Empirical benchmark of CPU-only execution and resolution invariance at 12 FPS standard.

| Camera Resolution Tier | Hardware Execution Mode | Frame Latency (ms) | Throughput (FPS) | Touch F1-Score | 12 FPS Budget Margin |
| --- | --- | --- | --- | --- | --- |
| 360p (640×360, Low-End) | Standard CPU (No GPU) | 22.4 ms | 44.6 FPS | 0.961 | +60.9 ms (73.1% Margin) |
| 360p (640×360, Low-End) | Accelerated GPU Reference | 6.8 ms | 147.0 FPS | 0.961 | +76.5 ms (91.8% Margin) |
| 480p (640×480, Standard VGA) | Standard CPU (No GPU) | 25.1 ms | 39.8 FPS | 0.963 | +58.2 ms (69.8% Margin) |
| 480p (640×480, Standard VGA) | Accelerated GPU Reference | 7.4 ms | 135.1 FPS | 0.963 | +75.9 ms (91.1% Margin) |
| 720p (1280×720, HD Webcam) | Standard CPU (No GPU) | 29.1 ms | 34.4 FPS | 0.963 | +54.2 ms (65.1% Margin) |
| 720p (1280×720, HD Webcam) | Accelerated GPU Reference | 8.5 ms | 117.6 FPS | 0.964 | +74.8 ms (89.8% Margin) |
| 1080p (1920×1080, Full HD) | Standard CPU (No GPU) | 31.8 ms | 31.4 FPS | 0.962 | +51.5 ms (61.8% Margin) |
| 1080p (1920×1080, Full HD) | Accelerated GPU Reference | 9.4 ms | 106.4 FPS | 0.963 | +73.9 ms (88.7% Margin) |

The results in Table 6.8 demonstrate three key points:

- Guaranteed CPU Real-Time Execution: Across all resolution tiers, CPU latency stays between 22.4 ms and 31.8 ms (31.4 to 44.6 FPS), using less than 39% of the available 83.33 ms time budget at 12 FPS. Even at 1080p, the pipeline maintains a 51.5 ms safety margin, proving that a dedicated GPU is not necessary.

- Low-End Camera Viability: Lower resolutions (360p and 480p) reduce pixel processing time for AprilTag tracking and MediaPipe keypoint detection, lowering CPU latency to 22.4 ms (44.6 FPS) and running very efficiently on budget hardware.

- Resolution Invariance: Touch classification accuracy remains stable (F1∈0.961,0.964) across 360p, 480p, 720p, and 1080p camera feeds. Because hand landmarks are scale-normalized relative to hand length Lhand and mapped through homography, image resolution changes do not affect the feature inputs sent to the LSTM model.

### 6.4.6	AprilTag Homography Tracking Accuracy and Tilt Sensitivity

Tracking accuracy was tested on printed AprilTag layout sheets (tag36h11 family) under camera tilt angles from 0∘ to 75∘ relative to the surface normal.

Table 6.9 presents the corner detection rates, sub-pixel corner errors, and RMS reprojection errors.

AprilTag homography tracking accuracy under perspective camera tilt angles.

| Camera Tilt Angle (θ) | Tag Corner Detection Rate | Sub-Pixel Corner Error | RMS Reprojection Error (erms) | Planar Target Accuracy |
| --- | --- | --- | --- | --- |
| 0∘ (Overhead Flat) | 100.0% | 0.038 pixels | 0.18 mm | 99.8% |
| 15∘ (Slight Tilt) | 100.0% | 0.042 pixels | 0.22 mm | 99.6% |
| 30∘ (Nominal Desk Angle) | 100.0% | 0.058 pixels | 0.28 mm | 99.4% |
| 45∘ (Moderate Inclination) | 99.4% | 0.084 pixels | 0.35 mm | 98.8% |
| 60∘ (Steep Angle) | 97.8% | 0.124 pixels | 0.42 mm | 97.6% |
| 75∘ (Extreme Perspective) | 84.2% | 0.286 pixels | 1.15 mm | 88.2% |

The AprilTag tracking engine maintains high detection rates (≥97.8%) and sub-millimeter reprojection accuracy (erms≤0.42 mm) for standard desk tilt angles up to 60∘.

#### 6.4.6.1	Reprojection Error Heatmap and Sub-Pixel Interpolation Analysis

To evaluate homography distortion across different areas of the paper keyboard, reprojection error ereprojx,y=∥Pxml−H⋅Ppixel∥2 was measured across a 10×10 grid on the A4 page.

Under normal desk camera placement (30∘ tilt), errors were lowest in the central typing area (ereproj≤0.22 mm), where all four AprilTag anchors provide strong alignment. Errors increased slightly near the paper edges (ereproj≤0.42 mm), remaining well within the key padding margin (2.5 mm), confirming uniform mapping accuracy across the layout.

### 6.4.7	Real-Time Pipeline Latency and Resource Profiling

The multi-threaded background engine (Application 2 Component B) was profiled on an Intel Core i7 desktop processor to measure component latencies and system resource usage.

Table 6.10 details the latency breakdown per frame cycle.

Detailed execution latency breakdown per frame processing cycle.

| Pipeline Processing Stage | Execution Time (ms) | Percentage of Total Cycle | Execution Context Thread |
| --- | --- | --- | --- |
| OpenCV Video Frame Ingestion | 3.25 ms | 11.2% | Background Capture Thread |
| AprilTag Detection and SVD Homography | 6.84 ms | 23.5% | Background Vision Thread |
| MediaPipe 21 Hand Pose Landmarking | 7.92 ms | 27.2% | Background Vision Thread |
| Scale Normalization and Feature Extraction | 0.45 ms | 1.5% | Background Vision Thread |
| PyTorch LSTM Touch Model Forward Pass | 2.08 ms | 7.1% | Background Vision Thread |
| Planar Coordinate Mapping (H⋅Ppixel) | 0.12 ms | 0.4% | Background Vision Thread |
| XML Bounding Box Lookup | 0.18 ms | 0.6% | Main GUI Event Loop |
| OS Virtual Keystroke / Shell Command Dispatch | 8.25 ms | 28.4% | Main GUI Event Loop |
| Total End-to-End Latency | 29.09 ms | 100.0% | Multi-Threaded Pipeline |

The total end-to-end latency is 29.09 ms, corresponding to a frame rate of 34.4 FPS. MediaPipe landmark tracking (7.92 ms) and AprilTag detection (6.84 ms) represent the main vision steps, while PyTorch LSTM inference takes only 2.08 ms. RAM usage is 142.5 MB and CPU load is 24.8%.

#### 6.4.7.1	CPU Thread Memory Contention and Mutex Overhead Analysis

Profiling Application 2 Component B showed that the multi-threaded queue design effectively isolates camera capture from deep learning inference. Mutex lock acquisition (QMutex.lock()) averaged 0.042 ms per cycle, accounting for less than 0.15% of the total pipeline latency.

Because PyTorch LSTM forward passes run on pre-allocated tensor memory, there was minimal memory contention between OpenCV frame decoding and PyTorch inference. No thread deadlocks or race conditions occurred during long testing runs.

### 6.4.8	User Typing Performance and Character Error Rate Benchmark

User typing performance was tested with ten participants executing single-handed text transcription phrases on a printed A4 paper virtual keyboard. Testing single-hand typing matches the system scope, keeping camera marker visibility unobstructed and CPU latency low. Metrics included Words Per Minute (WPM) and Character Error Rate (CER):

WPM=C5/Tmin, CER=S+D+IR×100%

where C is total characters typed, Tmin is time in minutes, S is substitutions, D is deletions, I is insertions, and R is reference text length.

Table 6.11 shows the typing test results comparing single-handed paper virtual keyboard typing with physical mechanical keyboards and touchscreen tablets.

User typing performance benchmark comparing input device modalities.

| Input Device Modality | Mean WPM | Character Error Rate (CER) | Key Hit Accuracy | User Fatigue Score (1–5) |
| --- | --- | --- | --- | --- |
| Physical Mechanical Keyboard | 58.4 WPM | 1.8% | 98.8% | 1.2 |
| Capacitive Touchscreen Tablet | 34.2 WPM | 4.5% | 95.8% | 2.8 |
| Paper Virtual Keyboard (Proposed) | 24.6 WPM | 5.2% | 95.2% | 2.4 |
| Single-Camera Baseline (Literature [6]) | 12.8 WPM | 11.4% | 88.6% | 3.6 |

The paper virtual keyboard achieved an average typing speed of 24.6 WPM with a Character Error Rate of 5.2%, which is a clear improvement over previous single-camera virtual keyboards in literature (12.8 WPM, 11.4% CER [6]).

## 6.5	Chapter Summary

This chapter presented the empirical testing and evaluation results for the paper virtual keyboard system.

Section 6.2 defined ten functional and seven non-functional test cases. Section 6.3 described the evaluation workflow. Section 6.4 presented quantitative benchmarks confirming that:

- The uni-directional PyTorch LSTM model trained on normalized position and velocity features achieved an F1-score of 0.963 with 2.08 ms inference latency on CPU.

- Unitless scale normalization (Lhand) maintained distance invariance (F1≥0.958) across camera distances from 30 cm to 100 cm.

- The 12 FPS standard architecture ran smoothly in real time on standard commodity CPUs without requiring a GPU, maintaining consistent touch classification accuracy (F1≈0.963) across low-end and high-end video resolutions (360p to 1080p).

- AprilTag marker tracking maintained sub-millimeter reprojection accuracy (erms≤0.42 mm) for camera tilt angles up to 60∘.

- Multi-threaded pipeline execution achieved a frame rate of 34.4 FPS (29.09 ms latency) with low resource usage (142.5 MB RAM, 24.8% CPU).

- User typing tests showed an average typing speed of 24.6 WPM with a Character Error Rate of 5.2%.

These evaluation results confirm that the virtual keyboard system provides reliable, low-latency performance on standard consumer hardware.

# 7	Concluding Remarks

## 7.1	Chapter Overview

This chapter summarizes the main findings, technical achievements, practical lessons, and future possibilities of the customizable paper virtual keyboard system. Using only a single standard monocular webcam and plain printed paper (without specialized hardware such as depth cameras, Time-of-Flight sensors, laser projectors, or wearable gloves) and running in real time on standard commodity CPUs, this project shows that everyday paper sheets can be turned into customizable virtual input devices across standard and low-cost camera resolutions.

Reviewing the research journey involves checking how the original goals were met, analyzing technical problems encountered during development, reflecting on skills and insights gained, discussing real-world applications, and suggesting directions for future work. Section 7.2 evaluates the achievement of each research objective by comparing literature review, software engineering, and experimental testing results. Section 7.3 discusses major technical problems faced and their solutions. Section 7.4 presents personal self-reflections on technical learning and project insights. Section 7.5 discusses practical deployment opportunities and business use cases. Finally, Section 7.6 outlines recommendations for future research.

## 7.2	Accomplishment of Research Objectives

To verify the results of this research, a triangulation strategy was used. Triangulation links the empirical testing findings (Chapter 6) back through the implementation modules (Chapter 5) and requirements specifications (Chapter 4) to the original research objectives stated in Chapter 1.

### 7.2.1	Triangulation Evaluation of Objective 1: Problem Identification and Literature Review

- Objective Statement: To identify existing limitations in monocular vision-based virtual keyboards and synthesize a comprehensive literature taxonomy.

- Triangulation Evidence: Chapter 2 reviewed 54 peer-reviewed publications, creating a structured taxonomy and comparison matrix. The review identified four key research gaps in previous literature [1], [6], [10]: (1) dependence on expensive specialized hardware (depth sensors, ToF cameras, active laser projectors) or high-end GPUs, (2) monocular depth ambiguity when fingers hover above flat surfaces, (3) sensitivity to hand size and camera distance variations, and (4) lack of dynamic layout customization on plain printed paper.

- Validation Outcome: Objective Fully Accomplished. The literature taxonomy directly guided the system architecture and requirements in Chapter 4.

### 7.2.2	Triangulation Evaluation of Objective 2: Scale Normalization and Deep Learning Pipeline

- Objective Statement: To design and implement a scale-invariant hand feature extraction pipeline and a temporal PyTorch deep learning classifier for touch contact detection.

- Triangulation Evidence: Chapter 5 implemented the Partition 2 Data Pipeline and Partition 3 Deep Learning Benchmark Engine. Hand joint coordinates were scale-normalized using unitless hand length (Lhand=∥P9−P0∥2), and sliding windows of 5 frames with 2-frame overlap (XW∈R5×84) were constructed at a 12 FPS standard rate. Scale-normalized features were passed directly to preserve clear deceleration signals during contact. Benchmark tests in Chapter 6 showed that the uni-directional PyTorch LSTM model (best_finger_touch_lstm.pth) achieved an F1-score of 0.963 with a fast CPU inference time of 2.08 ms.

- Validation Outcome: Objective Fully Accomplished. Testing confirmed distance invariance (F1≥0.958 across Z∈30 cm,100 cm) and camera resolution invariance (F1∈0.961,0.964 across 360p to 1080p) running in real time on standard commodity CPUs without a GPU.

### 7.2.3	Triangulation Evaluation of Objective 3: System Suite Design and Engineering

- Objective Statement: To design and implement an integrated two-application software suite enabling arbitrary custom layout design and decoupled action combination multiplexing on the same printed paper layout.

- Triangulation Evidence: Chapter 5 built Application 1 Layout Designer (designer/designer_app.py) and Application 2 Runtime Engine. Application 1 provides a PySide6 drag-and-drop canvas, automatic AprilTag border placement, XML layout export, and 300 DPI vector PDF generation for custom layouts. Application 2 provides a Setup GUI (Component A) for assigning key IDs to keystrokes or shell commands with full support for saving and switching multiple JSON action profiles on the exact same printed paper sheet without reprinting, and a background engine (Component B) running video capture, AprilTag homography tracking, PyTorch LSTM inference, and coordinate mapping (PXML=H⋅Ppixel).

- Validation Outcome: Objective Fully Accomplished. Functional testing in Chapter 6 demonstrated a 100% pass rate across all ten test cases (FT-01 through FT-10), confirming that the same printed layout can trigger different action profiles through software configuration.

### 7.2.4	Triangulation Evaluation of Objective 4: Empirical System Evaluation

- Objective Statement: To conduct rigorous empirical benchmarking evaluating real-time pipeline latency, optical homography tracking, and human typing performance.

- Triangulation Evidence: Chapter 6 evaluated the integrated system on standard CPU hardware. The multi-threaded engine achieved an end-to-end processing latency of 29.09 ms (34.4 FPS) with low resource usage (142.5 MB RAM, 24.8% CPU load), easily fitting within the 12 FPS budget (83.33 ms). AprilTag homography tracking maintained an RMS error under 0.42 mm for camera tilt angles up to 60∘. User typing tests showed an average typing speed of 24.6 WPM with a Character Error Rate of 5.2%.

- Validation Outcome: Objective Fully Accomplished. The system outperformed previous single-camera virtual keyboards in published literature (12.8 WPM, 11.4 CER [6]).

Table 7.1 presents the objective accomplishment and triangulation matrix.

Research objective accomplishment and triangulation matrix.

| Research Objective | Design / Chapter Origin | Implementation Artifact | Empirical Validation Evidence | Status |
| --- | --- | --- | --- | --- |
| O1: Literature Review and Gap Identification | Chapter 2 Literature Taxonomy | Research DB (references.bib, summary-matrix.md) | Identified 4 key gaps; established evaluation baseline. | ACCOMPLISHED |
| O2: Scale Normalization and Neural Touch Classifier | Chapter 3 Methodology and Chapter 5 Pipeline | Partition 2 (datacreator/) and Partition 3 (run_all.py) | F1-score 0.963; CPU inference 2.08 ms; CPU-only real-time at 12 FPS. | ACCOMPLISHED |
| O3: Two-Application System Suite Creation | Chapter 4 Requirements and Chapter 5 Architecture | Application 1 (designer_app.py) and Application 2 Suite | 100% pass rate across FT-01–FT-10; same-layout multi-profile switching verified. | ACCOMPLISHED |
| O4: Empirical Latency and Typing Evaluation | Chapter 6 Evaluation Framework | Multi-threaded Engine and Evaluation Scripts | Latency 29.09 ms (34.4 FPS); 24.6 WPM; 5.2 CER. | ACCOMPLISHED |

## 7.3	Problems Encountered and Technical Mitigations

During the design, implementation, and evaluation of the system, several practical problems arose from optical camera limitations and user interaction dynamics.

### 7.3.1	Hardware Constraints of Monocular RGB Capture

Standard webcams operate at 30 FPS with rolling shutter sensors, which can introduce motion blur during fast finger taps [9]. When a user types quickly, fast downward finger motion can cause landmark tracking confidence to drop momentarily.

- Technical Mitigation: A 12 FPS resampling script (resample_12fps.py) was used during dataset creation, training the PyTorch LSTM model on lower-frame-rate inputs. As a result, the model handles frame jitter and slight motion blur effectively during live camera execution.

### 7.3.2	Ambient Environmental Lighting and Shadow Variations

Monocular landmark tracking and AprilTag detection are sensitive to sudden lighting changes, strong directional shadows, and uneven glare [1]. Harsh sunlight casting dark finger shadows on the paper can distort corner detection.

- Technical Mitigation: The AprilTag detector (aprilTag/estimater.py) applies adaptive thresholding to grayscale frames before quad extraction. In addition, the SVD homography solver needs only four unoccluded tag corners across the page margins to maintain accurate mapping.

### 7.3.3	Anatomical Finger Co-articulation

Human fingers naturally share tendons, especially between the ring finger and pinky finger. When pressing a key with the ring finger, the pinky finger often moves downward slightly as well, which could trigger unwanted touches.

- Technical Mitigation: The PyTorch LSTM model predicts probabilities for all five fingers simultaneously using multi-label Binary Cross-Entropy loss (LBCE). This allows the network to distinguish primary intentional key strikes from minor secondary finger movements.

### 7.3.4	Absence of Tactile Haptic Surface Feedback

Tapping on flat paper provides no mechanical key travel or switch click sound. During initial tests, users were sometimes unsure if a key press had registered, causing them to press harder.

- Technical Mitigation: Immediate visual and audio feedback were added to Application 2 Component B. When a touch is confirmed (pk>0.90), the software plays a short click sound (<5 ms latency) and briefly flashes a green box over the pressed key on the live screen overlay.

## 7.4	Self-Reflection

Working on this research provided valuable academic and engineering experience in computer vision, deep learning, and software architecture.

### 7.4.1	Ideology Regarding Paper-Based Touch Interaction

Before starting this research, I assumed virtual input devices required expensive hardware, such as laser projectors, Time-of-Flight sensors, or electronic gloves. However, studying monocular computer vision and homography geometry showed that smart software design can replace complex hardware. By combining a regular sheet of printed paper with a standard webcam and a lightweight PyTorch LSTM model running on a regular CPU without requiring a GPU, a working virtual input interface can be built at near-zero hardware cost. This experience highlighted how software-driven physical interfaces can help make computing input more accessible in low-resource environments.

### 7.4.2	Technical Benefits Gained Across Engineering Domains

This project provided hands-on experience across four main computer science areas:

- Applied Computer Vision and Geometry: Gained practical experience in camera calibration (K), lens distortion correction (D), AprilTag fiducial tracking, Direct Linear Transformation (DLT), and Singular Value Decomposition (SVD) for Planar Homography (H∈R3×3).

- Deep Learning for Temporal Signals: Learned how to design and train PyTorch recurrent models, build multi-model benchmark pipelines (run_all.py), apply unitless scale normalization (Lhand), and format sliding temporal windows (XW∈R5×84).

- Concurrent Multi-Threaded Programming: Designed a producer-consumer thread system in PySide6 (QThread, QMutex, QWaitCondition), separating video capture from neural network inference to prevent frame drops and keep real-time throughput (34.4 FPS).

- Academic Research and Documentation: Developed skills in systematic testing, measuring typing metrics (WPM, CER), structuring research chapters, and writing formal LaTeX documentation with IEEE citations.

### 7.4.3	Critical Learning Curves and Methodological Reflections

A key learning curve in this project was balancing computing load with responsive touch detection on standard consumer computers. Early thoughts were to use high frame rates (30–60 FPS) and heavy neural networks, which would have required expensive GPUs and restricted the system from running on everyday laptops.

Through testing and profiling, it became clear that standardizing the pipeline at 12 FPS provides the right balance: it gives enough temporal detail to detect touch deceleration while leaving an ample 83.33 ms frame budget. This lightweight design allows the entire vision and deep learning pipeline to run smoothly on a standard CPU without a GPU, taking only 29.52 ms per frame. Combining this with unitless hand scale normalization and SVD homography mapping also ensures that low-resolution cameras (such as 360p or 480p) achieve the same touch accuracy (F1≈0.963) as higher-end cameras.

## 7.5	Business Insights and Real-World Application Possibilities

The paper virtual keyboard system offers practical advantages over traditional hardware keyboards, including low cost, custom layouts, easy cleaning, and quick deployment across different settings.

### 7.5.1	Sterile Medical Operating Rooms and Clinical Environments

In hospital operating rooms and clinics, physical keyboards and touchscreens can harbor bacteria because physical keys are difficult to disinfect between patients. A paper virtual keyboard printed on laminated, waterproof paper can be wiped clean with disinfectant or replaced for pennies per sheet, providing a clean input surface for medical staff.

### 7.5.2	Industrial Cleanrooms and Hazardous Chemical Laboratories

Semiconductor cleanrooms and chemical laboratories require spark-free, non-hazardous equipment. Electronic devices can accumulate dust or produce electrical sparks. A printed paper layout with AprilTag markers is completely passive and requires no electrical power on the table, making it safe to use in sensitive environments.

### 7.5.3	Low-Cost Educational Classrooms in Developing Regions

Schools in low-resource communities often lack funds for computer accessories and replacement keyboards. Using Application 1 Layout Designer (designer_app.py), teachers can print custom keyboard layouts, piano pads, or learning sheets on standard A4 paper. Connected to a single computer and basic webcam, students can interact with educational programs through paper interfaces.

### 7.5.4	Customized Digital Audio Workstations and Accessibility Interfaces

Hardware macro pads and MIDI controllers for software applications (such as digital audio workstations like Ableton Live, video editors like Adobe Premiere, or CAD software) are expensive and have fixed physical keys. Application 2 allows users to use a single printed sheet of paper and switch between multiple action profiles (such as a recording profile, an editing profile, and a mixing profile) in software without reprinting the sheet. In addition, users with motor impairments can design custom large-button accessibility layouts suited to their physical mobility, switching between speech tools, navigation shortcuts, and text input on the same printed layout.

## 7.6	Future Recommendations

While the virtual keyboard system performs reliably, there are several promising directions for future research:

### 7.6.1	Integration of Monocular Neural Depth Models

Future work could explore adding lightweight monocular depth estimation models (such as Depth Anything or MiDaS-nano) directly to the pipeline. Combining 2D hand joint coordinates with estimated depth maps could provide millimeter-level Z-axis distance data, further improving touch contact classification.

### 7.6.2	Multi-Camera Stereo Vision Fusion

Extending the single-camera setup to a dual-camera stereo configuration would help resolve hand occlusion. Tracking hand joints from two different angles would provide 3D triangulation and keep tracking active even when one camera view is partially blocked.

### 7.6.3	Two-Handed (Bimanual) Typing with Dual Window Buffering

While the current system implementation is optimized for single-handed active typing to ensure low CPU computation time and avoid fiducial marker occlusion on compact A4 paper sheets, the underlying modular architecture can be expanded to support two hands. Future research can configure MediaPipe for two hands (num_hands=2) and run separate 5-frame sliding window buffers for both the left and right hands. With the lightweight PyTorch model taking only 2.08 ms per forward pass, dual-hand inference would take approximately 33 ms total on CPU, remaining well within the 83.33 ms 12 FPS frame budget.

### 7.6.4	Spatial Acoustic and Ultrasonic Mid-Air Haptic Feedback

To provide physical feeling without mechanical keys, future versions could incorporate ultrasonic mid-air haptic arrays. By focusing ultrasonic pressure waves onto the active fingertip when a key press is confirmed, users could feel physical tactile feedback in the air.

### 7.6.5	Deep Learning Based Fiducial Marker Tracking

While classical AprilTag 2/3 detection operates efficiently on CPU, future research could explore integrating lightweight convolutional marker detectors like DeepTag [54]. By estimating dense keypoints directly through neural networks, such approaches could provide even greater homography stability under severe motion blur or heavy finger occlusions.

### 7.6.6	Transformer-Based Temporal Self-Attention Architectures

Although the PyTorch uni-directional LSTM model achieved strong results (F1=0.963), testing lightweight temporal Transformer models could help capture longer sequential patterns, potentially reducing Character Error Rates during fast, continuous typing.

## 7.7	Chapter Summary

This chapter concluded the thesis by reviewing the achievement of research objectives, discussing problems and mitigations, reflecting on technical lessons, presenting business use cases, and outlining future research directions.

The research demonstrated that a customizable paper virtual keyboard powered by monocular computer vision, hand scale normalization (Lhand), AprilTag planar homography (H∈R3×3), and PyTorch temporal deep learning (best_finger_touch_lstm.pth) achieves reliable, real-time typing performance (29.09 ms latency, 34.4 FPS, 24.6 WPM) on standard commodity CPU hardware. By replacing specialized hardware with smart software engineering, this work provides a flexible, hygienic, and accessible input interface.

# References

[1] 	Virtual Keyboard Survey Group, “Review of virtual keyboard,” International Journal of Engineering Research and Technology, vol. 9, no. 6, pp. 112–117, 2020.

[2] 	T.-H. Lee, S. Kim, T. Kim, J.-S. Kim, and H.-J. Lee, “Virtual keyboards with real-time and robust deep learning-based gesture recognition,” IEEE Transactions on Human-Machine Systems, vol. 52, no. 4, pp. 725–736, 2022, doi: 10.1109/THMS.2022.3165165.

[3] 	Z. Zhang, Y. Wu, Y. Shan, and S. Shafer, “Visual panel: Virtual mouse, keyboard and 3D controller with an ordinary piece of paper,” in Proceedings of the 2001 workshop on perceptive user interfaces (PUI), 2001, pp. 1–8.

[4] 	S. Srivastava and R. C. Tripathi, “Real time mono-vision based customizable virtual keyboard using finger tip speed analysis,” in Proceedings of the international conference on CIIA, 2012, pp. 143–148. doi: 10.1145/2160125.2160154.

[5] 	P. Khare, “QWERTY keyboard in virtual domain using image processing,” in 2019 international conference on intelligent computing and control systems (ICICCS), 2019, pp. 1364–1368. doi: 10.1109/ICICCS46578.2019.9037042.

[6] 	H. Ji, J. Chen, Q. Lin, and A. Li, “A local fingertips movement and fingertips clustering based virtual keyboard adopting a camera,” in Proceedings of the 2nd international conference on computer science and artificial intelligence (CSAI), 2018, pp. 253–257. doi: 10.1145/3232829.3232840.

[7] 	D. Lee and S. Kwon, “Virtual touch sensor using a depth camera,” Sensors, vol. 19, no. 4, p. 885, 2019, doi: 10.3390/s19040885.

[8] 	A. Kudale and K. Wanjale, “Real time webcam based infrared tracking for projection display system,” International Journal of Mathematical Sciences and Computing (IJMSC), vol. 2, no. 4, pp. 41–53, 2016, doi: 10.5815/ijmsc.2016.04.05.

[9] 	J. Thomas, “A camera based virtual keyboard with touch detection by shadow analysis,” Department of Computer Science, 2013.

[10] 	E. Posner, N. Starzicki, and E. Katz, “A single camera based floating virtual keyboard with improved touch detection,” in 2012 IEEE 27th convention of electrical and electronics engineers in israel (IEEEI), 2012, pp. 1–5. doi: 10.1109/EEEI.2012.6377033.

[11] 	Q. Yue, Z. Ling, B. Liu, X. Fu, and W. Zhao, “Blind recognition of touched keys: Attack and countermeasures,” IEEE Transactions on Information Forensics and Security, vol. 10, no. 7, pp. 1490–1503, 2015, doi: 10.1109/TIFS.2015.2428612.

[12] 	F. Zhang et al., “MediaPipe hands: On-device real-time hand tracking,” arXiv preprint arXiv:2006.10214, 2020.

[13] 	S. Sreenath, D. I. Daniels, A. S. D. Ganesh, Y. S. Kuruganti, and R. G. Chittawadigi, “Monocular tracking of human hand on a smart phone camera using MediaPipe and its application in robotics,” in 2021 IEEE 9th region 10 humanitarian technology conference (R10-HTC), 2021, pp. 1–6. doi: 10.1109/R10-HTC53172.2021.9641542.

[14] 	M. Gil-Martín, M. R. Marini, I. Martín-Fernández, S. Esteban-Romero, and L. Cinque, “Hand gesture recognition using MediaPipe landmarks and deep learning networks,” in Proceedings of the 18th international joint conference on computer vision, imaging and computer graphics theory and applications (VISIGRAPP), 2023, pp. 617–624. doi: 10.5220/0011689200003417.

[15] 	N. Ivačko, I. Ćirić, and M. Simonović, “Edge computing approach to AI-based gesture for human–robot interaction and control,” Computers, vol. 13, no. 4, pp. 1–18, 2024, doi: 10.3390/computers15040241.

[16] 	N. Andriyanov and S. Mikhailova, “Improving gesture recognition efficiency with MediaPipe and YOLO-pose,” The International Archives of the Photogrammetry, Remote Sensing and Spatial Information Sciences, vol. XLVIII–2/W9–2025, pp. 13–18, 2025, doi: 10.5194/isprs-archives-XLVIII-2-W9-2025-13-2025.

[17] 	G. Sánchez-Brizuela, A. Cisnal, E. de la Fuente-López, J.-C. Fraile, and J. Pérez-Turiel, “Lightweight real-time hand segmentation leveraging MediaPipe landmark detection,” Virtual Reality, vol. 27, no. 4, pp. 3125–3132, 2023, doi: 10.1007/s10055-023-00858-0.

[18] 	Y.-M. Li, T.-H. Lee, J.-S. Kim, and H.-J. Lee, “CNN-based real-time hand and fingertip recognition for the design of a virtual keyboard,” in 2021 36th international technical conference on circuits/systems, computers and communications (ITC-CSCC), 2021, pp. 1–4. doi: 10.1109/ITC-CSCC52171.2021.9501471.

[19] 	B. Maman and A. Bermano, “TypeNet: Towards camera enabled touch typing on flat surfaces through self-refinement,” in IEEE/CVF conference on computer vision and pattern recognition (CVPR), 2023, pp. 1–10.

[20] 	S.-S. Yoo and H.-S. Lee, “Word-level motion learning for contactless QWERTY typing with a single camera,” Sensors, vol. 26, no. 4, p. 1087, 2026, doi: 10.3390/s26041087.

[21] 	J. Wang and E. Olson, “AprilTag 2: Efficient and robust fiducial detection,” in 2016 IEEE/RSJ international conference on intelligent robots and systems (IROS), 2016, pp. 4193–4198. doi: 10.1109/IROS.2016.7759617.

[22] 	J. Kallwies, B. Forkel, and H.-J. Wuensche, “Determining and improving the localization accuracy of AprilTag detection,” in 2020 IEEE international conference on robotics and automation (ICRA), 2020, pp. 8188–8194. doi: 10.1109/ICRA40945.2020.9197314.

[23] 	K. Shabalina, A. Sagitov, L. Sabirova, H. Li, and E. Magid, “ARTag, AprilTag and CALTag fiducial systems comparison in a presence of partial rotation: Manual and automated approaches,” in Swarm intelligence and evolutionary computation, Springer, 2018, pp. 191–203. doi: 10.1007/978-3-319-93818-9_16.

[24] 	M. Kalaitzakis, B. Cain, S. Carroll, A. Ambrosi, C. Whitehead, and N. Vitzilaios, “Fiducial markers for pose estimation: Overview, applications and experimental comparison of the ARTag, AprilTag, ArUco and STag markers,” Journal of Intelligent & Robotic Systems, vol. 101, no. 4, p. 71, 2021, doi: 10.1007/s10846-020-01307-9.

[25] 	C. Pirchheim and G. Reitmayr, “Homography-based planar mapping and tracking for mobile phones,” in 2011 10th IEEE international symposium on mixed and augmented reality (ISMAR), 2011, pp. 193–198. doi: 10.1109/ISMAR.2011.6092388.

[26] 	H. Zhang, Y. Yin, L. Xie, T. Gu, M. You, and S. Lu, “DynaKey: Dynamic keystroke tracking using a head-mounted camera device,” IEEE Internet of Things Journal, vol. 9, no. 9, pp. 6563–6577, 2022, doi: 10.1109/JIOT.2021.3114224.

[27] 	I. Pilkov, “Markerless motion capture for pianists’ hand movements analysis,” Master’s thesis, Johannes Kepler University Linz, 2024.

[28] 	Y. Chen, “Gaze-based text entry with common RGB cameras,” Master’s thesis, Simon Fraser University, 2024.

[29] 	A. Enkhbat, T. K. Shih, N. L. Hakim, W. Aditya, and T. Thaipisutikul, “HandKey: An efficient hand typing recognition using CNN for virtual keyboard,” in 2020 IEEE international conference on systems, man, and cybernetics (SMC), 2020, pp. 3852–3857. doi: 10.1109/SMC42975.2020.9283182.

[30] 	P. Ošcádal et al., “Improved pose estimation of aruco tags using a novel 3D placement strategy,” Sensors, vol. 20, no. 17, p. 4825, 2020, doi: 10.3390/s20174825.

[31] 	S. Hochreiter and J. Schmidhuber, “Long short-term memory,” Neural Computation, vol. 9, no. 8, pp. 1735–1780, 1997, doi: 10.1162/neco.1997.9.8.1735.

[32] 	H. Gammulle, S. Denman, S. Sridharan, and C. Fookes, “TMMF: Temporal multi-modal fusion for single-stage continuous gesture recognition,” IEEE Transactions on Image Processing, vol. 30, pp. 7689–7701, 2021, doi: 10.1109/TIP.2021.3108420.

[33] 	M. A. Simão, P. Neto, and O. Gibaru, “Unsupervised gesture segmentation by motion detection of a real-time data stream,” IEEE Transactions on Industrial Informatics, vol. 13, no. 2, pp. 473–481, 2017, doi: 10.1109/TII.2016.2613683.

[34] 	J. Herdiansyah, F. A. S. Putra, and D. Septiyanto, “Implementation of zhang’s camera calibration algorithm on a single camera for accurate pose estimation using ArUco markers,” Journal of Fuzzy Systems and Control, vol. 2, no. 3, pp. 176–181, 2024, doi: 10.59247/jfsc.v2i3.256.

[35] 	S. Garrido-Jurado, F. J. Romero-Ramirez, and R. Muñoz-Salinas, “ArUco nano: A simpler, faster, and more reliable fiducial marker detector,” SoftwareX, vol. 34, p. 102690, 2026, doi: 10.1016/j.softx.2026.102690.

[36] 	T.-N. Doan, “An efficient patient activity recognition using LSTM network and high-fidelity body pose tracking,” International Journal of Advanced Computer Science and Applications (IJACSA), vol. 13, no. 8, pp. 1–9, 2022, doi: 10.14569/IJACSA.2022.0130800.

[37] 	J. C. Núñez, R. Cabido, J. J. Pantrigo, A. S. Montemayor, and J. F. Vélez, “Convolutional neural networks and long short-term memory for skeleton-based human activity and hand gesture recognition,” Pattern Recognition, vol. 76, pp. 80–94, 2018, doi: 10.1016/j.patcog.2017.10.033.

[38] 	K. Lai and S. N. Yanushkevich, “CNN+RNN depth and skeleton based dynamic hand gesture recognition,” in 2018 24th international conference on pattern recognition (ICPR), 2018, pp. 3649–3654. doi: 10.1109/ICPR.2018.8545718.

[39] 	M. Toshpulatov, W. Lee, S. Lee, and G. Lee, “Real-time multimodal fingertip contact detection via depth and motion fusion for vision-based human-computer interaction,” in IEEE/CVF conference on computer vision and pattern recognition (CVPR) workshops, 2024, pp. 1–10.

[40] 	S. Kumar, “HYBRID TEMPORAL FILTERING FOR STABLE TOUCHLESS DRAWING: NOISE REDUCTION IN MEDIAPIPE-BASED AIR CANVAS SYSTEMS,” TIJER - International Research Journal, vol. 13, no. 3, pp. 1–8, 2026.

[41] 	J. Cheng, Q. Wang, R. Song, and X. Wu, “Fingertip-based interactive projector–camera system,” Signal Processing, vol. 110, pp. 54–66, 2015, doi: 10.1016/j.sigpro.2014.08.043.

[42] 	J. MacRitchie and A. P. McPherson, “Integrating optical finger motion tracking with surface touch events,” Frontiers in Psychology, vol. 6, p. 702, 2015, doi: 10.3389/fpsyg.2015.00702.

[43] 	C. Boletsis and S. Kongsvik, “Text input in virtual reality: A preliminary evaluation of the drum-like VR keyboard,” Technologies, vol. 7, no. 2, p. 31, 2019, doi: 10.3390/technologies7020031.

[44] 	Z. Yang, Y. Chen, and B. Y. Zhao, “Towards a general video-based keystroke inference attack,” in Proceedings of the USENIX security symposium, 2022, pp. 1–18.

[45] 	J. Webber, A. Mehbodniya, R. Teng, A. Arafa, and A. Alwakeel, “Finger-gesture recognition for visible light communication systems using machine learning,” Applied Sciences, vol. 11, no. 24, p. 11582, 2021, doi: 10.3390/app112411582.

[46] 	Y. Sun, J. M. Hollerbach, and S. A. Mascaro, “Estimation of fingertip force direction with computer vision,” IEEE Transactions on Robotics, vol. 25, no. 6, pp. 1356–1369, 2009, doi: 10.1109/TRO.2009.2032954.

[47] 	C. Schenck et al., “Utilizing inpainting for keypoint detection for vision-based control of robotic manipulators,” in IEEE international conference on robotics and automation (ICRA), 2024, pp. 1–8.

[48] 	J. Isaksson and L. Magnusson, “Camera pose estimation with moving aruco-board,” Jönköping University, School of Engineering, 2020.

[49] 	A. Ikram and Y. Liu, “Skeleton based dynamic hand gesture recognition using LSTM and CNN,” in Proceedings of the 2020 international conference on artificial intelligence and computer engineering, 2020, pp. 248–253. doi: 10.1145/3421558.3421568.

[50] 	C. Cui, M. S. Sunar, and G. E. Su, “Deep vision-based real-time hand gesture recognition: A review,” PeerJ Computer Science, vol. 6, p. e2921, 2020, doi: 10.7717/peerj-cs.2921.

[51] 	X. Wang, M. Christie, and E. Marchand, “Relative pose estimation and planar reconstruction via superpixel-driven multiple homographies,” in 2020 IEEE/RSJ international conference on intelligent robots and systems (IROS), 2020, pp. 10705–10712. doi: 10.1109/IROS45743.2020.9341707.

[52] 	H. C. Longuet-Higgins, “Machine perception of visual motion,” GEC Journal of Research, vol. 3, pp. 145–161, 1985.

[53] 	M. Fiala, “Designing highly reliable fiducial markers,” IEEE Transactions on Pattern Analysis and Machine Intelligence, vol. 32, no. 7, pp. 1317–1324, 2010, doi: 10.1109/TPAMI.2009.146.

[54] 	Z. Zhang, Y. Hu, G. Yu, and J. Dai, “DeepTag: A general framework for fiducial marker design and detection,” IEEE Transactions on Pattern Analysis and Machine Intelligence, vol. 45, no. 2, pp. 2307–2320, 2023, doi: 10.1109/TPAMI.2022.3174603.

[55] 	Z. Jiang, L. Zhao, S. Li, and Y. Jia, “Real-time object detection method for embedded devices,” IEEE Access, vol. 9, pp. 114940–114951, 2021, doi: 10.1109/ICCV.2019.00140.

[56] 	M. F. Tariq and M. A. Javed, “Small object detection with YOLO: A performance analysis across model versions and hardware,” arXiv preprint arXiv:2504.09900, 2025.